| Activity/Work Task: Attending Environmental Spillage on Land | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Arrival at site. | 1.1 Unfamiliar with site condition. | 1.1.1 Verify contractor has delivered on site briefing of the site condition and associated hazards. 1.1.2 Verify spill substance is identified to determine adequate Material Safety Data Sheet (MSDS). 1.1.3 Verify contractor has barricaded or demarcated the spill area with appropriate safety signs posted. 1.1.4 Consult Hazardous Materials Response Guidebook or other Hazardous Material reference Guide | S | M | L |
| 2. Attending environmental spillage on land. | 2.1 Falls on level. | 2.1.1 Verify walkway is clear from any obstructions. 2.1.2 Correct footwear must be worn. 2.1.3 Do not step onto slippery grounds. 2.1.4 Follow mandatory signs. | S | M | L |
|  | 2.2 Exposure to fumes / hazardous gas. | 2.2.1 Verify contractor has conducted gas monitoring in the area. 2.2.2 Correct respiratory protection must be worn. 2.2.3 Safe distance to be maintained. 2.2.4 Follow mandatory signs. | S | M | L |
|  | 2.3 Exposure to flammable material. | 2.3.1 Verify no ignition source is around / naked flame. 2.3.2 Verify firefighting equipment is provided. 2.3.3 Verify emergency response procedure is clearly communicated and controlled on site. 2.3.4 Use Intrinsically Safe Tools and Equipment | S | M | L |
|  | 2.4 Direct contact with hazardous spill. | 2.4.1 Avoid direct contact with hazardous spill. 2.4.2 Correct Personal Protective Equipment must be worn. 2.4.3 Verify safe distance is maintained. 2.4.4 Verify nearest water source and eyewash location is identified. 2.4.5 Verify nearest first aid kit is identified. | S | M | L |
|  | 2.5 Exposure above exposure limit. | 2.5.1 Verify not to expose to more than recommended workplace exposure limit. 2.5.2 Wear appropriate PPE to reduce the exposure. 2.5.3 Follow mandatory signs. | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |  |
| - Personal Protective Equipment | 1. SH&E Site Induction. | All PPE should be inspected prior to use. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |  |