| Activity/Work Task: Attending Spillage on Marine Environment | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Setting up the machine | 1.1 Refer to the hazards of AHA 04 for Manual Handling. | 1.1.1 Refer to the controls of AHA 04 for Manual Handling. 1.1.2 Verify familiarization with the equipment prior to handling. | S | M | L |
|  | 1.2 Falls on same level. | 1.2.1 Always look and scan ahead of your way. 1.2.2 Verify adequate illumination / proper lighting for work location is provided. 1.2.3 Avoid muddy/greasy/wet floors where possible. 1.2.4 Avoid stepping over cables, tools, materials, etc. Verify walkway is clear. 1.2.5 Wear correct safety footwear. | S | M | L |
|  | 1.3 Falls from height | 1.3.1 Avoid getting close to open / unprotected manholes, openings, etc. 1.3.2 Do not step on / over covered voids where possible. 1.3.3 Follow mandatory warning signage and out of bounds areas. | S | M | L |
|  | 1.4 Adverse weather conditions. | 1.4.1 Reduce exposure by correct work planning. 1.4.2 Monitor temperature and humidity regularly. 1.4.3 Stay hydrated during hot weather condition 1.4.4 Use skin protection like sun block, etc.. during hot weather conditions. 1.4.5 Wear proper PPE. | S | M | L |
| 2. Observing the monitor. | 2.1 Uncomfortable workstation. | 2.1.1 Reduce exposure time and take regular breaks. 2.1.2 Maintain correct posture while seating. 2.1.3 Do light body stretching once in a while. 2.1.4 Verify no materials / objects placed under the workstation that restrict leg movement. | U | N | L |
|  | 2.2 Glare exposure. | 2.2.1 Reduce exposure time and take regular breaks. 2.2.2 Verify correct positioning of light / lamp / window blinds. 2.2.3 Wear correct eyeglass. | U | N | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |  |
| - Personal Protective Equipment | 1. SH&E Site Induction. | All PPE should be inspected prior to use. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |  |