| Activity/Work Task: Confined Space Entry | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Arrival near the confined space entrance. | 1.1 Unsafe Access towards the entrance. | 1.1.1 Verify edge protection and warning signs are in place. 1.1.2 Verify access is unobstructed and non-slippery. 1.1.3 Verify access is fit for the area. 1.1.4 Verify access is inspected and tagged. | S | M | L |
|  | 1.2 Area not classified as confined space. | 1.2.1 Do not enter the confined space and stop the work immediately. | U | Cr | L |
|  | 1.3 Falling materials. | 1.3.1 Verify no one is working above. 1.3.2 Verify no materials are hanging above / unsecured. | U | M | L |
| 2. Entering the confined space. 2. Entering the confined space. (Cont...) | 2.1 No permit to work secured for the confined space. | 2.1.1 Do not enter the confined space and stop the work immediately. 2.1.2 Contractor and employees to be oriented on the permit to work system. | U | Cr | L |
|  | 2.2 Presence of asphyxiating fumes/gases or toxic substances. 2.2 Presence of asphyxiating fumes/gases or toxic substances. (Cont…) | 2.2.1 Attend a pre-task briefing before entering a confined space. 2.2.2 Verify continuous gas monitoring is being conducted in the area with standby system. 2.2.3 Verify continuous supply of fresh air/operation of ventilating fans. 2.2.4 No hot work shall be performed without the appropriate permit to work. 2.2.5 Verify machine exhausts are diverted away from the confined space. 2.2.6 Verify adequate PPE is worn (filter mask). | S | Cr | M |
|  | 2.3 Unsafe access (ladder). | 2.3.1 See AHA 06 Use of Ladders | S | M | L |
|  | 2.4 Poor Illumination. | 2.4.1 Verify contractor has completed illumination level monitoring of the area. 2.4.2 Verify provision of lighting system in the absence of natural light. | U | M | L |
| 3. Inspection inside the confined space. 3. Inspection inside the confined space. (Cont…) 3. Inspection inside the confined space. (Cont…) 3. Inspection inside the confined space. (Cont…) | 3.1 Exposure to dust. | 3.1.1 Verify area had been pre-ventilated and is continuously ventilated. 3.1.2 Verify adequate PPE is worn (filter mask) 3.1.3 Limit grinding / de-shuttering operations during the inspection. 3.1.4 Verify ventilation system do not re-introduce dust in the area. | S | M | L |
|  | 3.2 Extreme Temperature. | 3.2.1 Verify continuous operation of the ventilation system in the area with standby system. 3.2.2 Rehydrate before entering a confined space. 3.2.3 Attend a pre-task briefing before entering a confined space. 3.2.4 Verify contractor has carried out temperature monitoring. 3.2.5 Limit time of exposure at the confined space area. | S | M | L |
|  | 3.3 Water intrusion. | 3.3.1 Verify dewatering system are on standby. 3.3.2 Verify contractor enforces lock-out / tag-out of existing water and sewer line system. 3.3.3 Verify opening of the confined space is adequately bunded to prevent intrusion from the surface. 3.3.4 Cancel inspection in the event of rain. | S | Cr | M |
|  | 3.4 Release of or presence of asphyxiating gases/fumes or toxic substances. 3.4 Release of or presence of asphyxiating gases/fumes or toxic substances. (Cont…) | 3.4.1 Attend a pre-task briefing before entering a confined space. 3.4.2 Verify continuous gas monitoring is being conducted in the area. 3.4.3 Verify continuous supply of fresh air/operation of ventilating system with standby system. 3.4.4 No hot work shall be performed without the appropriate permit to work. 3.4.5 Verify machine exhausts are diverted away from the confined space. 3.4.6 Verify adequate PPE is worn (filter mask). 3.4.7 Verify contractor enforces lock-out / tag-out of existing gas/oil/fuel lines. | S | Cr | M |
|  | 3.5 Poor air circulation – Oxygen deficiency. | 3.5.1 Verify contractor is doing continuous monitoring of the atmosphere. 3.5.2 Verify continuous operation of the ventilation system with standby system. 3.5.3 Attend a pre-task briefing before entering a confined space. 3.5.4 No hot work shall be performed without the appropriate permit to work. 3.5.5 Verify machine exhausts are diverted away from the confined space. 3.5.6 Verify adequate PPE is worn (filter mask). 3.5.7 Verify only limited workers are allowed in the area. | S | Cr | M |
|  | 3.6 Non-insulated live utilities (electrical / steam lines). | 3.6.1 Verify all live utilities are properly marked / highlighted/ identified in the area. 3.6.2 Follow all warning signs. 3.6.3 Stay away from live electrical utilities or steam lines. | S | Cr | M |
|  | 3.7 Slippery surface. | 3.7.1 Verify all walking surfaces are free from mud/oil/grease/algae. 3.7.2 Verify proper footwear is worn and is free from mud/oil/grease. | S | M | L |
|  | 3.8 Obstructed access. | 3.8.1 Verify walkways are free from excess materials/debris. 3.8.2 Verify that adequately illumination is provided on site. | S | M | L |
|  | 3.9 Absence of emergency response team / emergency rescue equipment. 3.9 Absence of emergency response team / emergency rescue equipment. (Cont…) | 3.9.1 Verify rescue equipment is available and is fit for the operation. 3.9.2 Attend a pre-task briefing before entering a confined space and be familiar with the confined space emergency procedure. 3.9.3 Verify all emergency response team members are trained and competent. | U | Cr | L |
|  | 3.10 High noise levels. | 3.10.1 Verify contractor had conducted a noise level monitoring in the area. 3.10.2 Verify adequate PPE is worn (ear muffs/ear defenders/ear plugs). | U | M | L |
|  | 3.11 Impalement hazards. | 3.11.1 Always look and scan ahead of your way. 3.11.2 Follow all warning signs in the area. 3.11.3 Verify that adequately illumination is provided on site. 3.11.4 All protruding materials shall be provided with end caps. 3.11.5 Verify all scrap wood is free from nails | S | M | L |
|  | 3.12 Lack of communication. | 3.12.1 Verify constant communication with confined space supervisor/ watchman / emergency response team. 3.12.2 Verify confined space supervisor / watchman / emergency response team area aware of the inspection/activity inside the confined space. 3.12.3 Verify a permit to work had been secured and approved prior to entry. 3.12.4 Lone entry / lone working shall not be permitted | U | Cr | L |
|  | 3.13 Falling materials. | 3.13.1 Verify adequate PPE is worn (hard hat). 3.13.2 Verify no unsecured materials are placed overhead | S | M | L |
|  | 3.14 Engulfment hazard. | 3.14.1 Verify lock-out / tag-out or isolation of live services had been implemented 3.14.1 Verify adequate shoring of deep trenches | S | Cr | M |
|  | 3.15 Failure of ventilation system. | 3.15.1 Verify availability of back-up ventilation system 3.15.2 Verify ventilation system have updated maintenance record | S | Cr | M |
|  | 3.16 Fire / Explosion. 3.16 Fire / Explosion. (Cont…) | 3.16.1 Verify continuous ventilation of the area to prevent build-up of flammable or explosive gases 3.16.2 No flammable materials are to be stored in the area 3.16.3 No hot work shall be performed without the appropriate permit to work. 3.16.4 Verify adequate fire fighting equipment is available in the area. 3.16.5 Verify oxygen level in the area does not exceed normal ranges. 3.16.6 Use only intrinsically safe tools inside the confined space. | S | Cr | M |
|  | 3.17 Getting trapped on smaller sections of the confined space. | 3.17.1 Be familiar with the configuration of the area. 3.17.2 Verify barriers or guards are installed to prevent being trapped. | S | M | L |
|  | 3.18 Presence of poisonous animals (snakes) / harmful insects/ pests. | 3.18.1 Verify contractor had done a preliminary sweep of the area for poisonous animals / harmful insects. 3.18.1 Have a basic knowledge on handling poisonous animals / harmful insects. | U | Cr | L |
| 4. Climbing out of the confined space | 4.1 Unsafe ladder access. | 4.1.1See AHA 06 Use of Ladders | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Personal Protective Equipment. | SH&E Site Induction. Permit to Work System (Confined Space and Hotwork) Lock-out / Tag-out System Emergency Rescue on Confined Spaces Working Safely on Confined Spaces Fire Fighting Training Use of gas monitoring detectors | PPE should be inspected prior to use. Atmospheric gas monitoring Noise monitoring Illumination level monitoring Temperature level monitoring Ventilation System inspection Emergency Rescue Equipment Inspection Ladder Inspection All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |