| Activity/Work Task: Decanting Fuel And Fueling Small Plant | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Transporting Fuel | - Fire, Explosion and Spillage | - All fuel shall only be carried in suitable containers of the appropriate color and marked with contents - All fuel shall be secured within the vehicle - Only carry sufficient fuel for the task - Spill kits and a Dry Powder fire extinguisher shall be available in all vehicles transporting fuels check | S | M | L |
| - Decanting Fuel from One Container To Another / Filling Small Plant with Fuel. | - Fire, Explosion - Fumes | - Where possible fill all plant at the depot, rather than on site - Personal protective equipment, as detailed in the Control of substance hazardous to health risk assessments for fuel, shall be worn. - Undertake a pre-use check of plant to check that there are no leakages and that the fuel cap fits correctly and that the seal is fitted - During hours of darkness check that suitable lighting is available – Do not use naked flames - All plant shall be re-fuelled away from any worksite, other machines or naked flames – Take the plant to the fuelling point and not the fuel to the plant - Smoking is not permitted within plant or fuelling points - A fire extinguisher shall be available at all fuelling points - Before fuelling small plant check that it has cooled down significantly - Slowly release the pressure with the fuel cap on the plant as hot fuel may be released under pressure - Decant / Re-fuel machines from the container using a flexible spout or funnel – Do not fill / decant direct from the container - Avoid spilling any fuel over the machine / container, but if spilling occurs then wipe off the excess fuel (dispose of any rags as hazardous waste and check that they are kept apart from any ignition sources) and leave the machine for approximately two minutes before starting it up - Check that the fuel cap has been replaced and tightened correctly before starting the plant, including the seal is in place - Check that the fuel container cap has been replaced and tightened | S | M | L |
|  | - Contact With Fuel | - Decanting and fuelling of plant shall only be performed by competent persons - Adhere to additional requirements for use of personal / respiratory protective equipment contained in the COSHH risk assessment - Eye protection shall be worn when fuelling / decanting fuel - Good fitting impervious (PVC, Nitrite, or similar) gloves shall be worn – Remove gloves at the end of fuelling / decanting and dispose of in approved waste containers - Wash hands before smoking, eating or drinking – If hand washing facilities are not immediately available use antibacterial hand rubs or wipes | S | M | L |
|  | - Environmental Damage | - See AHA for Refueling | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Hand gloves - Respiratory protection (As Needed) - Safety Glasses | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures - Training In The Use Of Spill Kit | - Daily - Weekly - When Required |  |  |  |