| Activity/Work Task: Directional boring | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Directional boring | - Access and egress | - Access and egress will be provided separate for man and for machinery - Daily inspection of access will be done and maintained accordingly - Without a safe access, the employees will not be allowed to enter the area | S | M | L |
|  | - Contact with underground existing utilities | - Permit to work must be obtained prior to starting work - All Required NOC;s will be obtained and copies will be available at site - Area will be scanned for existing underground utilities | S | M | L |
|  | - Confined space | - Gas monitoring will be done by a competent person frequently as required - Confined space procedure will be followed - Confined space entry watch will be in place to record the number of employees in the confined space - No naked flame will be allowed on site - Smoking will not be allowed in the area - Artificial ventilation will be provided - Frequent rotation of employees will be followed - Only trained employees will be allowed in the confined space - Emergency rescue plan will be in place - Adequate supervision will be available on site at all times | S | M | L |
|  | - Falls | - All employees will be trained in working at height - Safe work platform will be provided for all employees - Full body safety harness will be worn and anchored - No employee will be allowed to work out of the work platform - Adequate supervision will be available on site | S | M | L |
|  | - Excavation | - A permit must be obtained for excavation - Only trained employees will be allowed in excavations - Shoring or benching will be provided - Excavation will be protected by a hard barrier - All spoil material will be kept minimum of 2 meters away from the excavation - Caution signage’s will be placed around the excavation - Adequate lighting will be around the excavation area - Area will be checked for any underground utilities - NOC;s will be obtained from all concerned parties | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Hard Hat, Safety Glasses, Class 2 Reflective Vest & Safety footwear, gloves, safety Harness. Respiratory protection | - AHA training - Safety Orientation - Working at height - Confined space |  |  |  |  |