| Activity/Work Task: Diving Operations | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1 Diving | 1.1 Un-experienced personnel | 1.1.1 Qualified divers (good health condition / training) 1.1.2 Each diver must possess a valid medical certificate that clearly specifies the conditions for which the certificate is valid. 1.1.3 Each diver must be free from illness and 100% sober 1.1.4 First aid - arrangements for first aid and medical treatment. 1.1.5 Proper briefing prior to start work 1.1.6 ALWAYS a stand-by diver available 1.1.7 Verify that diving area is free from obstacles 1.1.8 Never dive head first in the water 1.1.9 Continuous monitoring and coaching 1.1.10 Pre dive test to be conducted for divers. | S | M | L |
|  | 1.2 Bad Maintained equipment - super cooling and drowning | 1.2.1 Properly maintained diving equipment/tools - certified correctly 1.2.2 Equipment shall be inspected and determined to be in good working order prior to start of each diving operation. This inspection will form part of the Pre-Dive Checks and should be rechecked frequently during continuous Diving Operations 1.2.3 Diving logbook must be maintained 1.2.4 Proper communication 1.2.5 Nearest re-compression chamber to be identified | S | M | L |
|  | 1.3 No proper communication | 1.3.1 Location of diving operations will be marked and Port Authorities will be notified 1.3.2 Posting of diving flag 1.3.3 No vessel activity in the environment of diving activities. Skipper of assisting boat shall keep watch 1.3.4 Proper supervision and communication | S | M | L |
|  | 1.4 Communications are lost and cannot be restored quickly | - Abort dive - Emergency rescue boat must be on standby at all times - Check weather report from meteorological service to be updated prior to diving & at 12 hour intervals thereafter. - Emergency standby diver must be available for rescue. | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Diving equipment | Site Induction Training in AHA | PPE before use All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |