| Activity/Work Task: Electric Arc Welding | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Electric Arc Welding | - Electrocution | - See AHA for Working Adjacent to DC Electrified Rails - See AHA for Working Near Electrical Overhead Line Equipment - When required, continuity bonds are fitted when working on the traction return rail in AC areas | S | M | L |
|  | - Inhalation of Fumes from Oil, Grease, Electrodes / flux cored wires and skin contact from NDT spray | - Site specific briefing, including hazardous substances hazards and controls shall be given - Use Personal Protective Equipment e.g. eye / face protection and respiratory protective equipment - Sheer speed tents shall not be used for arc welding - Local exhaust ventilation shall be used where necessary - Non-Destructive Testing shall not be undertaken until rail temperature is below 50°c - Provision of information on hazardous substances and control measures shall be available - Only trained and competent welders shall be used | S | M | L |
|  | - Electric Shock From Equipment | - Only trained and competent welders shall be used - Scheduled planned maintenance shall be in place for all equipment - Pre-work checks shall be completed on equipment - Site specific briefing shall be given - Work piece shall be earthed | S | M | L |
|  | - Fire/ Explosion / Splatter | - Personal protective equipment shall be used including visor, gauntlets, flame retardant clothing, knee length welding boots - Clean area of oil and grease before welding - Cylinders and petroleum spirit shall be excluded from ignition sources - Site specific briefing shall be given - Waste hot material shall be placed in an area free of flammable material whilst cooling and prior to disposal shall be placed in a metal receptacle (electrode stub ends) - Dry powder fire extinguisher shall be taken to site - Propane shall not be used in confined spaces or tunnels | S | M | L |
|  | - Radiation Exposure | - Hand or head shield shall be used at all times | S | M | L |
|  | - Adverse Weather Conditions | - See AHA Work On or Near the Line - A welding umbrella can be used for this activity but in extreme weather conditions or very high winds an assessment shall be made for its continued use. | S | M | L |
|  | - Grinding dust/ sparks | - Provision of hazardous substances information and controls shall be available - Personal Protective Equipment shall be issued and used, such as eye and respiratory protective equipment - Site specific briefing shall be given | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Safety Glasses - Hand Gloves - Hearing Protection - Welding Mask - Flame Resistant Over All - Dust Mask | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly - When Required |  |  |  |