| Activity/Work Task: Electric High Power – Existing Buried and Overhead | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Electric High Power – Existing Buried and Overhead | - Contact with underground and over head services | - All permits must be obtained prior to starting work on any existing services - A lock out tag out procedure must be in place and followed. - All drawings must be checked for existing services in the work area - A CAT must be used to identify if there are any existing services in the work area - All existing services must be supported to avoid collapse of service - A verification must be made that the services are de activated prior to start of work - Only manual excavation will be followed till services are located - Trial pits will be dug to locate existing services - A area audit will be established | S | M | L |
|  | - Property Damage | - All permits must be obtained prior to start of work - All NOC;s must be readily available - Only trained persons must be allowed to work in the area - All documents must be maintained - Inspection of the site must be done daily to check for damage to equipment - A competent person must supervise the job | S | M | L |
|  | - Traffic | - All traffic must be diverted away from the work area - All permits for traffic diversions must be made available prior to start of work - Traffic diversion signs must be posted prior to the actual diversion - Speed limit signs must be posted to reduce the speed of the traffic - A traffic control plan must be in place - Hard barriers must be placed to avoid heavy traffic from entering the work area - Security will be available to ensure that visitors and other road users are controlled from entering the area | S | M | L |
|  | - Working in hot weather | - All employees will be provided with cool drinking water - Shaded areas will be provided on site - Air conditioned areas will be provided - Mid day break will be followed according to the UAE law | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Hard Hat, Safety Glasses, Class 2 Reflective Vest & Safety footwear, gloves, safety Harness, | - AHA training - Safety Orientation - Lock out tag out Procedure training | - Daily - Weekly |  |  |  |