| Activity/Work Task: Entering Excavation | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared : |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Arrival / Passing near/around the excavation. | 1.1 Absence of edge protection and warning signs. | 1.1.1 Maintain a safe distance away from the edge of the excavation. 1.1.2 Maintain approved edge protection and warning will be immediately provided. 1.1.3 Install gangways across trenches to eliminate jumping over the trench. | S | Cr | M |
|  | 1.2 Presence of tension cracks near the edge of the excavation / Evidence of soil collapse. | 1.2.1 Maintain a safe distance away from the edge of the excavation. 1.2.2 Prevent materials from being placed on excavation edge. 1.2.3 Follow warning signs on site. 1.2.4 Check for the excavation protection, shoring/benching/sloping in place. 1.2.5 Maintain daily pre-shift inspection made for the excavation. 1.2.6 Do not enter the excavation. | S | Cr | M |
| 2. Entering the excavation. | 2.1 Access and Egress – Unsafe Ramp. | 2.1.1 Look ahead/be aware of moving plant and vehicles. 2.1.2 Keep the hands free (not in the pocket) while walking 2.1.3 Avoid slippery surface (oil, water mud, stones, etc…) 2.1.4 Maintain adequate illumination on ramp/walkway is adequately illuminated. 2.1.5 Keep scanning the floor avoid obstacles, such as building material, cables and tools. | S | M | L |
|  | 2.2 Access and egress – Unsafe Ladder. | 2.2.1 install ladder so that the top and bottom ends of the ladder are secure. 2.2.2 Make a visual inspection to ensure that the ladder is safe and sound. 2.2.3 install ladder so that it will extend one meter clearance overtop. 2.2.4 Maintain clean ladders free from oil, grease or mud. 2.2.5 Maintain three-point contact. 2.2.6 Check for proper angle of the ladder (4:1). 2.2.7 Do not use job made ladder unless certified. 2.2.8 Do not carry a load on a ladder. 2.2.9 Only one person at a time will use a ladder. 2.2.10 Maintain proper illumination on site. | S | Cr | M |
|  | 2.3 Access and egress – Unsafe Stairs. | 2.3.1 Check for the proper angle of the stairs. 2.3.2 Check if the tread is anti slip. 2.3.3 make sure that railing is in good condition. 2.3.4 Maintain 3-point contact. 2.3.5 maintain stair treads make sure they are free from oil, grease or mud. 2.3.6 Maintain adequate illumination on site. 2.3.7 Install handrail on stairs with 4 or more treads | S | Cr | M |
|  | 2.4 Access and egress - Unsafe manbasket. | 2.4.1 Maintain proper third party certification of the manbasket and crane. 2.4.2 Perform a pre-use inspection on the manbasket to ensure that it is in good condition 2.4.3 Check for the SWL of the manbasket 2.4.4 Check for the full body harness and adequate anchor point 2.4.5 Ensure that the crane operator and rigger are all certified. | S | Cr | M |
|  | 2.5 Access and Egress - Steps | 2.5.1 Verify the correct height of the riser (approx 30 cm) 2.5.2 Keep shoe laces tied 2.5.3 Ensure that the adequately illumination is provided 2.5.4 install proper handrail when anymore than four (4) stairs 2.5.5 Maintain cleanliness so that each step is free from oil, grease or mud 2.5.6 Maintain good housekeeping so that the steps are free from any obstruction. | S | M | L |
|  | 2.6 Soil adversely affected by heavy rain | 2.6.1 Access should not be allowed in any excavation after a rain event and should be properly assessed and inspected by a competent person. | S | Cr | M |
|  | 2.7 Fumes/Gases | 2.7.1 If excavation is considered as a confined space refer to confined space AHA controls 2.7.2 Check if there is any activity taking place where to produce fumes/gases. If so, input appropriate control measures are provided. 2.7.3 Maintain an adequate level of ventilation to prevent the build up of fumes/gases. | S | Cr | M |
| 3. Walking inside the excavation | 3.1 Falling Materials | 3.1.1 prevent materials from being placed on the edge 3.1.2 Follow all mandatory signs / out of bound areas 3.1.3 Require that basic PPE is worn (hard hat, safety glass, safety shoes). 3.1.4 Inspect for no overhanging/undermined sides 3.1.5 confirm that utilities passing over the excavation are properly supported 3.1.6 No resting should be allowed inside excavations or trenches. | S | M | L |
|  | 3.2 Falls on same level | 3.2.1 Use designated route/walkway 3.2.2 Look ahead/be aware 3.2.3 Keep hands free (not in pocket) while walking on site 3.2.4 Follow mandatory signs on site 3.2.5 Avoid grease mud water on walking surfaces where possible. 3.2.6 Avoid obstructing on walkways such as tools materials and cables. | S | M | L |
|  | 3.3 Signs of cracks or collapse on the sides of the excavation | 3.3.1 Work should be stopped and adequate support system shall be installed to prevent cave-ins | S | Cr | M |
| 4. Walking on elevated areas of the excavation | 4.1 Falls from Height | 4.1.1 install approved edge protection 4.1.2 Follow mandatory warning signs on site 4.1.3 Do not approach unprotected edges 4.1.4 Use designated routes and walkways 4.1.5 Do not stop on/over covered voids where possible. | S | Cr | M |
| 5. Passing a noisy area in the excavation | 5.1 Noise | 5.1.1 Check if the contractor has conducted noise survey 5.1.2 Follow mandatory use of PPE. | S | M | L |
| 6. Passing near Moving Plants and Vehicles on or near the excavation | 6.1 Moving Plant/Vehicles | 6.1.1 Wear high-visibility vest 6.1.2 Use designated walkways 6.1.3 Do not pass behind plant/vehicles 6.1.4 Look out for moving plants and vehicles. 6.1.5 Make eye contact with operators / banksman. | S | Cr | M |
| 7. Passing live utilities | 7.1 Live Utilities | 7.1.1 Coordinate with the contractor regarding presence of any live utilities. If so, make sure that control measures are provided. 7.1.2 Follow mandatory signs / out of bound area. | S | Cr | M |
| 8. Passing flooded areas | 8.1 Flooding / Presence of water in the excavation/trench | 8.1.1 Check for the weather condition before entering the excavation .Exit if heavy rain starts 8.1.2 maintain water intrusion control by dewatering | S | M | L |
| 9. Walking during extreme temperatures/weather conditions | 9.1 Exposure to extreme temperature/adverse weather condition | 9.1.1 Check for humidity/temperature outside working site 9.1.2 Wear approved clothing and PPE’s 9.1.3 Reduce exposure 9.1.4 Implement work rotations 9.1.5 Stay hydrated during hot weather condition 9.1.6 Use skin protection like sunblock, etc... during hot weather condition. | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Personal Protective Equipment (PPE). | SH&E Site Induction. Crane and man-basket third party test certificate Operator, rigger and banksman competency | PPE should be inspected prior to use. Daily Inspection of Excavation (Competent Person). Ladder / stairs / access pre-use inspection All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |