| Activity/Work Task: Entering or Working Near an Excavation or Trench | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1 Entering or Working Near an Excavation or Trench | 1.1 Slips Trips and Falls | 1.1.1 Walk with hands free (not in pockets) 1.1.2 Wear Proper footwear 1.1.3 Be aware of your surroundings 1.1.4 Pay attention to signs 1.1.5 Do Not cross barriers | S | M | L |
|  | 1.2 Overcome from fumes or lack of oxygen – illness, injury, death | 1.2.1 Make sure proper air monitoring is being utilized 1.2.2 If hazardous atmosphere is present make sure proper PPE is being worn 1.2.3 Proper Ventilation | S | Cr | M |
|  | 1.3 Crushing hazard from collapse or equip | 1.3.1 Do not cross cordoned off areas 1.3.2 Stay away from edges of excavations 1.3.3 Wear high-visibility vest 1.3.4 Use designated walkways. 1.3.5 Do not stay behind plant/vehicles. 1.3.6 Look out for moving plants and vehicles. 1.3.7 Make sure banks men are provided in the area. | S | M | L |
|  | 1.4 Working in the Heat | 1.4.1 Check for humidity/temperature outside before moving across site. 1.4.2 Wear correct clothing and PPE. 1.4.3 Reduce exposure. 1.4.4 Stay hydrated during hot weather condition 1.4.5 Use skin protection like sun block, etc. during hot weather conditions. | S | M | L |
|  | 1.5 Ground conditions: obstructions (tools, materials, cables, etc…) | 1.5.1 Keep hands free (not in pocket) while walking on site. 1.5.2 Be aware/look ahead. 1.5.3 Verify suitable lighting is provided on site. 1.5.4 Keep shoe laces tied. 1.5.5 Don’t step over obstructions walk around. | S | M | L |
|  | 1.6 Access and egress – Unsafe Ladder. | 1.6.1 Verify that the top and bottom ends of the ladder are secure. 1.6.2 Make a visual inspection to ensure that the ladder is safe and sound. 1.6.3 Verify that the ladder will extend one meter over the landing platform. 1.6.4 Ensure that ladder is free from oil, grease or mud. 1.6.5 Maintain three-point contact. 1.6.6 Check for proper angle of the ladder (4:1). 1.6.7 Do not use job made ladder unless certified. 1.6.8 Do not carry a load on a ladder. 1.6.9 Only one person at a time will use a ladder. 1.6.10 Ensure that adequately illumination is provided on site. 1.6.11 Ensure that there are two means of access and egress. | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Hard Hat, Safety Glasses, Reflective Vests, Safety Footwear. | Eliminate by Design - SH&E Plan Requirement - Contractor HSE Plan Requirement - Contractor HSE Plan Requirement including - PTW System and Daily Checklist (or refuse - inspection/entrance to excavation) - Confined Space Entry Plan Requirement - PTW System Requirement (Contractor) - Daily Excavation Checklist Requirement - (Contractor) - Emergency Response Plan - Audit Protocol Development - Safety Orientation - AHA Training | Excavation inspection checklist and Inspect for violations SHM Procedures Excavation inspection checklist and Inspect for violations of CSHM Procedures All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Excavation inspection checklist and Inspect for violations of HSE Procedures Effort should be made to communicate findings to contractor. |  |  |  |