| Activity/Work Task: Entry Into Confined Spaces | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Entry Into Confined Spaces | - All Tasks | - All entry to confined spaces shall be performed in accordance with Abu Dhabi Construction Code Of - Confined Spaces Entry Procedure with a Permit to Work - A site specific risk assessment and Safe system of work shall be in place before any work is performed - An Appointed Person shall be appointed to oversee and implement a safe system of work prior to entry to confined spaces - Only staff specifically trained to enter confined spaces shall do so - Emergency rescue arrangements (including fire and first aid arrangements, rescue equipment) shall be in place prior to and whilst all work is performed within the confined space - All confined spaces access points shall have suitable barriers in place to prevent access by the public, etc, these points shall be monitored to prevent unauthorized access during use - All confined spaces access points shall be suitably guarded to prevent unauthorized access when not in use - Staff shall be Medically Suitable to carry out task | S | M | L |
|  | - Presence of Gas/ Fumes , Oxygen Deficiency and Gas Vapors entering from other activities. | - A test of the confined space shall be performed, prior to entry, using a calibrated gas monitor meter - Where gas or fumes are present then: Ventilation (such as force ventilation with fans or compressed air) of the confined space shall be performed, with purging of the space with an inert gas where necessary. Entry to the confined space shall be with breathing apparatus only by staff who have specifically been trained in the use of breathing apparatus - Where other process may produce gases or fumes then: Isolate other potential processes that may produce gas or vapors – Lock off at the control valves and physically isolate any pipe work. The permit to work system shall check that all potential processes are recorded as being locked off - After ventilation, purging, isolations, etc a further test shall be performed of the confined space using a calibrated gas monitor meter | S | M | L |
|  | - Fumes generated from sludge deposits, Presence of toxic substances / vapors, Gas or methane present or seeps into confined space, Fumes produce from processes, Use of generators within confined space, Use of oxy-propane | - Where sludge is present: Check that a sample of the sludge is tested prior to entry – Contact the Route or Delivery Unit Infrastructure maintenance Workforce Safety Advisor - Where toxic substances or vapors are present: Force ventilate the confined space using fans or compressed air - Where gas or methane are present: Conduct research of adjacent land for possible sources – Isolate where possible Force ventilate the confined space using fans or compressed air - Where fumes are produced during work activities: Ventilate the area prior to entry and continue to provide ventilation / local exhaust ventilation during work Implement a permit to work in the event of any hot work being performed (in addition to a permit to enter confined spaces) - Where generators or oxy-propane are used during work activities: Petrol or diesel driven generators shall not be used within confined spaces Provide additional ventilation when generators are used so that an adequate supply of fresh air is available at all times - For all the above categories entry to the confined space shall be with breathing apparatus only, by staff who have specifically been trained in the use of breathing apparatus and a further test shall be performed of the confined space using a calibrated gas monitor meter after control measures are put in place | S | M | L |
|  | - Unexpected egress of water, Contact with electricity / machinery, Exposure to excessive heat, Overcome by dust / fumes, Contact with free flowing solids / biological substances | - Where unexpected water egress may occur then: Conduct research of adjacent land for possible water sources and isolate / divert water courses where possible, Check weather forecasts prior to entering confined spaces – If heavy rain is forecast do not commence work before or during the heavy rain or a period afterwards, Do not enter confined spaces where there is a fast flowing water course - Where contact with electricity or machinery is possible then: All electricity and machinery within confined spaces shall be isolated and locked off before entering confined spaces The permit to work system shall check that all potential processes are recorded as being locked of - Where staff exposure to excessive heat from plant or work activities is possible then: Check that an adequate supply of fresh air is available at all times Monitor the temperature within the confined space at all times and remove staff if the working temperature rises too high, Check that staff take regular breaks – Rotate staff where necessary Provide hot or cold liquid refreshments - Where staff may be overcome by dust and fumes then: Dust shall be extracted or damped down prior to entry Dust sampling shall be performed by a competent person prior to entry – Contact the Area Maintenance Workforce Safety Advisor Provide local exhaust ventilation and respiratory protective equipment (RPE shall only be provided as a last resort only) | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Hand gloves - Respiratory protection (As Needed) - Safety Glasses | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly - When Required |  |  |  |