| Activity/Work Task: Erection Dismantling And Use Of Welding Tents / Umbrella And Support Clamps | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Erection Dismantling and Use of Welding Tents / Umbrella and Support System | - Equipment Damage | - Check Welding Tents/Umbrellas have been inspected 6-monthly to check that they are still fit for purpose – records shall be maintained of these inspections | S | M | L |
|  | - Electrocution | - See AHA Working Adjacent to DC Electrified Rails - See AHA Working Near Electrical Overhead Line Equipment - DC electrified lines shall be isolated before erection or use | S | M | L |
|  | - Transporting Welding Umbrellas and Tents | - Manual handling training and carrying handles and bags for carrying welding umbrellas shall be provided - Consideration shall be given to using welder’s trolley to transport equipment or carrying bags shall be used whilst transporting tents to site - Umbrellas shall be carried in carrying bags so that the umbrella is not unfurled by gusts of wind - Provide manual handling training and carrying handles and bags for carrying welding tents | S | M | L |
|  | - Adjacent Open Lines/ Contact With OHLE, Adverse Weather e.g. High Winds | - Formal instructions shall be given by the Area Welding Engineer or Welding Trainer prior to authorizing staff to erect tents / umbrellas - Welding umbrellas shall only be erected / dismantled by 2 trained and competent persons working in accordance with erection / dismantling sequence - The person in charge of the work shall perform a site assessment to assess the weather conditions before the tent/umbrella is pitched - Welding umbrellas shall be positioned with the external surface facing the direction of the wind - Welding tents/umbrellas shall not be erected in severe weather conditions (i.e. high winds) - Pictorial instruction cards on erecting tents shall be provided with the tent by the manufacturer - Danger over head electrical line warning signs shall be attached to the side of the tent - Tent shall only be erected / dismantled by 2 trained and competent persons working in accordance with erection / dismantling sequence - Welding tents shall be secured by the under rail anchors or the guide ropes in accordance with the Work Instructions as an initial step - Tents shall be positioned with adequate clearance between the tent sidewalls and the crucible - Tents shall be positioned with the closed end / rear of tent facing the direction of wind | S | M | L |
|  | - Tripping Hazards, Heat, Flames, Hazardous Fumes, Flammable Atmosphere | - Staff shall be provided with COSHH information on the by-products of welding and the associated respiratory protective equipment required - All welding equipment shall be maintained in accordance with the manufacturer’s instructions - Only trained and competent welders shall perform welding activities - Welding tents/umbrellas shall be positioned with adequate clearance between the umbrella canopy and preheating flame / crucible - Only welding staff shall be permitted in the vicinity of the welding umbrella during the welding process (from the alignment of the rails to the completion of trimming) - Welding tents shall be vacated during the welding operation and an exclusion zone established and enforced - Vents shall be built into the tents to enable circulation and rapid removal of fumes – Vents shall not be blocked at any point and a check shall be made prior to welding to check that all events are in the open position - Re-entry to the tent shall only occur when the fumes have dissipated - Welders shall perform a pre-start check of all welding equipment to check that it is fit for purpose and will not cause a flammable atmosphere in the welding tent due to leaking gases | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Safety Glasses - Hand Gloves - Hearing Protection - Respiratory Protection | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly - When Required |  |  |  |