| Activity/Work Task: Generic Track Risks | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Working On Railway | - Electrocution | - Only Insulated tools must be used - Shields shall be used where ever required - All tools and yourself must maintain minimum safe distance from electrical lines - See AHA for Working Near Electrical Overhead Line Equipment, | S | M | L |
|  | - Fueling | - Equipment shall be switched off prior to refueling. - Hand and eye protection shall be worn - No naked flame/ hot work will be allowed near the refueling area - Fire extinguishers must be available on site. - See AHA for Decanting Fuel and fueling small plant. | S | M | L |
|  | - Hand Tools | - All Tools Shall be checked and inspected so they are in good condition and safe to use - The tools being used shall be correct for the task. | S | M | L |
|  | - Lack of welfare facilities | - Welfare Facilities must be considered in the work planning stage and arrangements must be made when and as required. | S | M | L |
|  | - Dust, Silicates, Oil, Biological Hazards | - See AHA for Biological and Chemical Hazards - Wash hands before eating, drinking or smoking. - Cover all cuts with water proof plasters - Use dust masks - Dust suppression must be done frequently to avoid dust from lifting. | S | M | L |
|  | - Use of OTP | - Competent operator and machine controller shall be appointed. - The pre use checks shall have been carried out. - Prior to movement of any OTP the horn shall be sounded - All lifts shall be planned controlled and the crane controller shall be appointed. | S | M | L |
|  | - Use of hand trolley | - See AHA for manual trolley / rail skate/ scooter. - The trolley shall be within its service data. - The necessary pre use checks and brakes test shall be carried out prior to each use. - Trolley shall not be used on gradients greater than 1.50 - Trolleys shall be operated by competent persons and shall have at least two people with it when being moved with one person in control of the break. - Hand trolleys shall not be overloaded, load shall be secured and not foul of open lines. | S | M | L |
|  | - Hand arm vibration from powered plant. | - Plant used shall be within its service date - Competent operators only shall operate powered plant. - Rotation of staff, regular breaks from all vibratory equipment shall be implemented - Time worked on the machine shall be controlled - Hands shall be kept warm to increase blood flow to extremities - Planned preventive maintenance shall be instigated | S | M | L |
|  | - Manual Handling | - See AHA for Manual Handling - All staff shall be trained in manual handling - Lifting operation shall be mechanized wherever possible - A person shall be appointed to control the movement and shall brief all staff prior to the movement, on how the load is to be moved and where it is to be moved - Gloves shall be used during manual handling | S | M | L |
|  | - Slips Trips and falls | - See AHA for Under Foot Conditions - Site shall regularly be checked for slips trios and falls hazards - Site shall be kept tidy - Protective footwear shall be worn at all times and it shall be in good condition. | S | M | L |
|  | - Loading / Unloading road vehicles | - See AHA loading and unloading wagons and vehicles - Sufficient supervision shall be available on site to carry out the lift safely - Load shall be evenly distributed and secured - The load shall not exceed the safe working load of the vehicle - Only a competent person shall operate the lifting device - No one will be allowed under the suspended load. - The vehicle shall be parked in a suitable position for the lifting operation and the hand break shall be applied for the vehicle. | S | M | L |
|  | - Buried Services | - All ground disturbances shall b e planned - See AHA for ground penetration and excavation - See AHA for use of Cable Avoidance Tool (CAT) - Area shall be CAT Scanned Prior to work commencing by a competent person only and all hazard areas will be marked and identified. - Permit to dig shall be issued and available on site. - Safe digging practices shall be adopted - No Objection Certificates from all concerned parties shall be obtained and available on site prior to commencing work | S | M | L |
|  | - Noise | - Planned preventive maintenance program for all machinery shall be implemented - Before using the machine the employee shall check the machine for defects and that it is within the scheduled maintenance date. - Ear defenders shall be worn within the hearing protection zone - Wherever possible the site noisy equipment shall be away from the worksite. - Warning Protection (Touch Lookout) shall be provided if required | S | M | L |
|  | - Environmental | - All waste shall be removed from site in a controlled manner - All spillages/ leaks of fuels/ contaminants shall be cleaned up using spill kits. - General public shall be advised of noise and light nuisance in advance of work being undertaken | S | M | L |
|  | - Lone Working | - See AHA for Lone Working - Means of communication shall be available at all times - Carry water to prevent dehydration - Protect skin from sun rays, use light weight clothing and sun block | S | M | L |
|  | - Adverse Weather (Sun, Visibility, Cold, Wet) | - Carry water to prevent dehydration - Protect skin from sun rays, use light weight clothing and sun block - Do not start work if the visibility on site does not provide appropriate warning time for safe system of work - Fog / Mist , Leave the work site in a safe condition and go to a place of safety. - Warm clothing shall be worn underneath high visibility vest during the cold season - Wet weather, High visibility clothing shall be worn - Suitable preventive foot wear shall be worn - See AHA Under Foot Conditions.. | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Hand gloves - Eye protection - Hearing protection - Dust mask - Safety harness | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly |  |  |  |