| Activity/Work Task: Inspecting Refueling of Plant and Equipment | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Inspecting refueling of plant & equipment. 1. Inspecting refueling of plant & equipment. (Cont…) | 1.1 Exposure to Collisions with vehicles. | 1.1.1 Verify that refueling is carried out in designated areas. | S | M | L |
|  | 1.2 Exposure to fire/ explosion. | 1.2.1 Verify the employee is trained and aware of the refueling procedure. 1.2.2 Verify that the fuel tanks are grounded. 1.2.3 Verify that flammable liquid signage is inplace. 1.2.4 Verify that no smoking signage is in place & no one is smoking within the vicinity of the refueling area. 1.2.5 Verify adequate fire protection is available at the refueling area. 1.2.6 Verify that the machinery is switched off while refueling. 1.2.7 Verify that emergency procedures are in place. | S | M | L |
|  | 1.3 Contact with oil/ fuel. | 1.3.1 Verify that you remain at a safe distance away from the refueling. | U | M | L |
|  |  | 1.3.2 Verify the use of PPE (Gloves, safety glasses) |  |  |  |
|  | 1.4 Oil spillage. | 1.4.1 Verify that the refueling is done at a designated area. | U | M | L |
|  |  | 1.4.2 Verify that there is a spill kit in place. |  |  |  |
|  |  | 1.4.3 Verify that there are no leaks in the refueling hose & bund. |  |  |  |
|  | 1.5 Exposure to fuel vapor. | 1.5.1 Verify refueling is carried out in a well ventilated location (Outside) | S | M | L |
|  |  | 1.5.2 Stand in the direction of the wind so that the vapor blows away from you. |  |  |  |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Personal Protective Equipment. | SH&E Site Induction. | All PPE should be inspected prior to use. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |