| Activity/Work Task: Inspection & Supervision Of Concrete Placement | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Mobilizing to site for supervision of concrete placement. 1. Mobilizing to site for supervision of concrete placement. (Cont…) | 1.1 Exposure to public road hazards. | 1.1.1 Driver of vehicle to posses a valid driving license for the type of vehicle being driven. 1.1.2 Drive defensively. (Obey all traffic regulations). | S | M | L |
|  | 1.2 Over speeding due to time constraint. | 1.2.1 Sufficient time should be calculated to arrive on time to site. | S | M | L |
|  | 1.3 Exposure to collision with site plant & equipment. | 1.3.1 Driver of vehicle to posses a valid driving license for the type of vehicle being driven. 1.3.2 Employee to be inducted & made familiar with site routes & site conditions. | S | M | L |
|  | 1.4 Exposure to collision with pedestrians/ workers. | 1.4.1 Site speed limit to be strictly complied to. 1.4.2 Only designated access roads must be used. 1.4.3 Pedestrians to be given right of way. | S | M | L |
|  | 1.5 Exposure to collision with over head obstructions. | 1.5.1 Only designated access roads must be used. | S | M | L |
|  | 1.6 Entering with the proximity (energized zone) of power lines. | 1.6.1 Only designated access roads must be used. 1.6.2 Drive with caution & look out for over head, adjacent or ground level power lines. | S | M | L |
|  | 1.7 Entering close to underground facilities (Excavations, sewers, etc). | 1.7.1 Maintain a safe distance from underground facilities. 1.7.2 Only designated access routes to be used. | S | M | L |
| 2. Inspection Of Concrete Pump Set Up. | 2.1 Exposure to high noise levels. | 2.1.1 Ear protection to be worn where noise levels are high. | S | M | L |
|  | 2.2 Exposure to slips, trips & falls. | 2.2.1 Operator and contractor to be involved during the inspection of concrete pump set up. 2.2.2 Caution to be taken while moving around the concrete pump for inspection. 2.2.3 that all material that is not required at the concrete pump set up location is removed from the area. 2.2.4 that general housekeeping is done prior to set up inspection. | S | M | L |
|  | 2.3 Exposure to being crushed/ pinched between moving machinery and or static objects. | 2.3.1 Maintain a safe distance from Concrete pump moving parts while inspecting. 2.3.2 all moving parts are correctly guarded. 2.3.3 Wear hand gloves, safety glasses, safety shoes & hard hat while inspecting the set up of the concrete pump. 2.3.4 Operator to assist during the set up inspection. | S | M | L |
| 3. Inspection Of Horizontal & Vertical Delivery Lines. 3. Inspection Of Horizontal & Vertical Delivery Lines. (Cont…) | 3.1 Exposure to slips, trips & falls. | 3.1.1 Caution to be taken while inspecting delivery lines. 3.1.2 that the delivery lines are kept free from obstructions during inspection. | S | M | L |
|  | 3.2 Exposure to fall from height. | 3.2.1 that scaffolding is in good condition & tagged safe to use before climbing. 3.2.2 Use of full body safety harness with 100% tie off system to be followed. 3.2.3 Always use three points of contact while climbing ladders. 3.2.4 that stairs & ladders are clear from debris & slippery material. 3.2.5 Ladder should extend one meter above step off point/ landing platform. 3.2.6 Stairs with more than four rises shall have hand rails on both sides. | S | Cr | M |
|  | 3.3 Exposure to extreme weather conditions. | 3.3.1 Consumption of sufficient water to be maintained. 3.3.2 Use shaded areas for resting/ cooling off. 3.3.3 Use appropriate skin protection. 3.3.4 Wear light colored clothing, covering all parts of body. | S | M | L |
|  | 3.4 Exposure to falling objects. | 3.4.1 that over head work is not in progress while under scaffoldings. 3.4.2 Hard hat must be used during inspection. | O | M | M |
| 4. Supervision Of Concrete Placement. 4. Supervision Of Concrete Placement. (Cont…) 4. Supervision Of Concrete Placement. (Cont…) | 4.1 Whipping of concrete pump end hose. | 4.1.1 Keep a safe distance from concrete pump flexible end hose (at a minimum distance of twice the length of the end hose). 4.1.2 Hard hat must be worn at all times while on site. | O | M | M |
|  | 4.2 Entry of concrete/ foreign body in eye. | 4.2.1 Maintain a safe distance from concrete placement. 4.2.2 Safety goggles for eye protection to be worn at all times while supervising concrete placement. 4.2.3 Eye wash station to be kept readily available during the concrete placement operation. | S | M | L |
|  | 4.3 Contact of concrete with skin. 4.3 Contact of concrete with skin. (CONT…) | 4.3.1 Maintain a safe distance from concrete placement. 4.3.2 Wear full sleeved clothing. 4.3.3 Long pants must be worn at all times during supervision of concrete placement. 4.3.4 Safety foot wear must be worn at all times while onsite. 4.3.5 Verify fully equipped first aid box to be kept available during concrete placement operation. 4.3.6 Verify First Aider to be available during the concrete placement operation. 4.3.7 Verify that there is an emergency shower or availability of fresh water for washing of body in case of concrete contact with skin/ body. | U | M | L |
|  | 4.4 Dislodge/ collapse of end hose onto employee. | 4.4.1 Verify that boom tip safety device is fitted to the concrete pump end hose to prevent falling of hose incase of dislodge of end hose. 4.4.2 Keep a safe distance from concrete pump flexible end hose (at a minimum distance of twice the length of the end hose. 4.4.3 Verify that contractor has carried out a full inspection of the hose lines before placement starts. 4.4.4 Hard hat must be worn at all times while on site. 4.4.5 Verify fully equipped first aid box to be kept available during concrete placement operation. 4.4.6 Verify first aider to be available during the concrete placement operation. | O | M | M |
|  | 4.5 Exposure to falling of section’s of delivery hose. 4.5 Exposure to falling of section’s of delivery hose. (CONT…) | 4.5.1 Keep a safe distance from concrete pump flexible end hose (at a minimum distance of twice the length of the end hose. 4.5.2 Verify that all safety pins are fitted for all sections of concrete pump delivery hose. 4.5.3 Verify that no employee is under the concrete pump boom/ delivery hose (Rigid & Flexible). 4.5.4 Hard hat must be worn at all times while on site. 4.5.5 Verify fully equipped first aid box to be kept available during concrete placement operation. 4.5.6 Verify first aider to be available during the concrete placement operation. | S | Cr | M |
| 5. Inspection Of Site After Concrete Placement Is Completed. | 5.1 Exposure to Waste/ Excessive concrete on site. | 5.1.1 Wear safety footwear at all times while on site. 5.1.2 Do not walk or step in concrete wash out waste. | S | M | L |
|  | 5.2 Exposure to moving equipment. | 5.2.1 Stay within a safe zone away from moving equipment. 5.2.2 Stay clear from vehicle access roads. 5.2.3 Take caution by checking area around you before commencing actual inspection. | S | M | L |
|  | 5.3 Exposure to extreme weather conditions. | 5.3.1 Carry adequate water for drinking. 5.3.2 Use appropriate skin protection. 5.3.3 Wear light colored clothing, covering all parts of body. 5.3.4 Use shaded areas for resting/ cooling off. | U | M | L |
| 6. Demobilizing From Site. 6. Demobilizing From Site. (Cont…) | 6.1 Exposure to public road hazards. | 6.1.1 Driver of vehicle to posses a valid driving license for the type of vehicle being driven. 6.1.1 Drive defensively. (Obey all traffic regulations). | S | M | L |
|  | 6.2 Exposure to collision with site plant & Equipment. | 6.2.1 Driver of vehicle to posses a valid driving license for the type of vehicle being driven. 6.2.2 Employee to be inducted & made familiar with site routes & site conditions. | S | M | L |
|  | 6.3 Exposure to collision with pedestrians/ Workers. | 6.3.1 Site speed limit to be strictly complied to. 6.3.2 Only designated access roads must be used. | S | M | L |
|  | 6.4 Exposure to collision with over head obstructions. | 6.4.1 Only designated access roads must be used. | U | M | L |
|  | 6.5 Entering with the proximity (energized zone) of power lines. | 6.5.1 Only designated access roads must be used. 6.5.2 Drive with caution & look out for over head, adjacent or ground level power lines. | S | M | L |
|  | 6.6 Entering close to underground facilities (Excavations, sewers, etc). | 6.6.1 Maintain a safe distance from underground facilities. 6.6.2 Only designated access routes to be used. | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Personal Protective Equipment. | SH&E Site Induction. | PPE to be inspected monthly. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |