| Activity/Work Task: Inspection Near Concrete Chipping/Breaking | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Inspection near Concrete Chipping/Breaking - 1. Inspection near Concrete Chipping/Breaking - (Cont…) | 1.1 Falls on level. | 1.1.1 Verify designated walkway is clear. Avoid protruding parts of the equipment. Avoid stepping over obstacles such as tools, materials and cables. 1.1.2 Correct safety footwear should be worn. 1.1.3 No oil, grease, and other slippery substance on floors and shoes. Avoid slippery walkway surface. 1.1.4 Verify adequate illumination/lighting provided. 1.1.5 Follow mandatory signs. | S | Cr | M |
|  | 1.2 Exposure to flying concrete particles/foreign objects | 1.2.1 Maintain safe distance from the concrete chipping/breaking activity. 1.2.2 Wear approved eye protection. 1.2.3 Verify proper protection/cover is provided by the contractor to isolate concrete chipping/breaking activity. 1.2.4 Follow mandatory safety signs. | S | Cr | M |
|  | 1.3 Exposure to dust | 1.3.1 Refer to AHA 19 Exposures to Dust. | S | M | L |
|  | 1.4 Exposure to Noise | 1.4.1 Refer to AHA 20 Exposures to Noise. | S | M | L |
|  | - Exposure to moving equipment/vehicle | 1.5.1 Verify work area is barricaded and provided with safety signs. 1.5.2 Verify competent banksman is designated. 1.5.3 Verify traffic management plan is observed and implemented. 1.5.4 Maintain safe distance from moving equipment and vehicle. - 1.5.5 Follow mandatory safety signs. | S | Cr | M |
|  | 1.6 Exposure to pressurized hose line. | 1.6.1 Verify pressurized hose lines are in good condition and regular maintenance is being conducted by contractor. 1.6.2 Verify hose connections have whip checks and couplings are in good condition 1.6.3 Maintain safe distance from concrete chipping/breaking operation. | S | Cr | M |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Personal Protective Equipment. | SH&E Site Induction. | All PPE should be inspected prior to use. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |