| Activity/Work Task: Inspection Of Form Work Or False Work | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Arrival at location of form work/ False work. | 1.1. Falls on same level. | 1.1.1. Wear appropriate foot wear. 1.1.2. Do not step over tools or material. 1.1.3. Use designated walk ways/ cross over’s. 1.1.4. Obey all mandatory signs. 1.1.5. Sty out of cordoned areas. 1.1.6. Keep hands out of pockets & free of objects. 1.1.7. Do not walk on slippery surfaces. 1.1.8. Verify adequate lighting is available for work in dark areas or at night. | U | M | L |
|  | 1.2. Collision With Plant. | 1.2.1. Park vehicle in a safe position (Use hand break). 1.2.2. Wear high visibility vest. 1.2.3. Use designated walkways only. 1.2.4. Do not walk behind plant if possible. 1.2.5. Verify suitable lighting. 1.2.6. Obey mandatory signages. | U | Cr | L |
|  | 1.3. Falling Material. | 1.3.1. Use appropriate PPE (hard hat, safety boots, etc.) 1.3.2. Verify that no personnel are working over head. 1.3.3. Verify that there is no loose material on False work/ Form work. | U | Cr | L |
| 2. Access to inspection area. | 2.1. Falls from Height. | 2.1.1. Refer to AHA 03 use of scaffolding. 2.1.2. Refer to AHA 06 use of ladder. 2.1.3. If there is no safe working platform in place with edge protection as per local legal requirements, Verify the use of a full body safety harness and two lanyards with fall arrest system. | U | Cr | L |
| 3. Inspection of false work/ form work. | 3.1. Failure of false work/ form work. | 3.1.1. Verify false work/ form work has been inspected and signed off by contractor’s engineers. 3.1.2. Verify no visible deficiencies in false work, missing members, poor ground conditions. 3.1.3. Verify that placing of concrete takes place in line with the approved method statement and risk assessment. | U | M | L |
|  | 3.2. Falls on same level. | 3.2.1. Wear appropriate foot wear. 3.2.2. Do not step over tools or material. 3.2.3. Use designated walk ways/ cross over’s. 3.2.4. Obey all mandatory signs. 3.2.5. Stay out of cordoned areas. 3.2.6. Keep hands out of pockets & free of objects. 3.2.7. Do not walk on slippery surfaces. 3.2.8. Verify adequate lighting is available for work in dark areas or at night. | U | M | L |
|  | 3.3. Falls From Height. | 3.3.1. Refer to AHA 03 use of scaffolding. 3.3.2. Refer to AHA 06 use of ladder. 3.3.3. If there is no safe working platform in place with edge protection as per local legal requirements, Verify the use of a full body safety harness and two lanyards with fall arrest system. | U | Cr | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Personal Protective Equipment. | SH&E Site Induction. | PPE to be inspected monthly. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |