| Activity/Work Task: Inspection On Scaffold | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Movement Across the Site. | Refer to AHA – Movement across the site. | Refer to AHA – Movement across the site. |  |  |  |
| 2. Pre inspection before accessing scaffold. | 2.1 Falling material. | 2.1.1 Comply with Site PPE Regulation (Hard hat, Steel toe safety shoes & eye protection). 2.1.2 Verify edge protection in place. 2.1.3 Verify no unsecured materials above. | U | M | L |
|  | 2.2 Unsafe scaffold | 2.2.1 Verify scaffold is green tagged prior to use. 2.2.2 Verify safe working load of the scaffold is Displayed and adhered to. 2.2.3 Verify scaffold is placed on correct feet and base plates 2.2.4 Verify scaffold is secured/ tied off if required. 2.2.5 Verify wheels of the mobile tower scaffold are locked. 2.2.6 Verify scaffold is placed in firm and level ground. | S | M | L |
| 3. Climbing up the scaffold through stairway / ladder. | 3.1 Failure of the stairway / ladder. | 3.1.1 Verify safe position of stairs. 3.1.2 Verify staircase is installed at least 30 degrees, and no more than 50 degrees, from the horizontal. 3.1.3 Verify ladder is safely positioned at 75 degree angle (1:4 ratio). 3.1.4 Verify ladder / staircase is secured. 3.1.5 Verify ladder/stairs is on firm base and footing. 3.1.6 Verify railing of stairways as per standard. 3.1.7 Only one person at a time will use the ladder. 3.1.8 Do not use job made ladder unless certified. 3.1.9 Check SWL of the ladder. | S S | M M | L L |
|  | 3.2 Falling from height | 3.2.1 Always maintain 3 point contact while using ladder. 3.2.2 Verify stairs are clear from any obstruction & non slippery. 3.2.3 Verify rungs of the ladder are clear of mud/grease & are not damaged. 3.2.4 Verify proper footwear is worn and is free from mud/grease. 3.2.5 Verify ladder extends 1 meter from the top landing platform. 3.2.6 Verify ladder is safely positioned at 75 degree angle (1:4 ratio). 3.2.7 Do not carry a load on a ladder 3.2.8 Verify that adequately illumination is provided on site. 3.2.9 Always face the ladder when climbing up or down. | U | Cr | L |
| 4. Failure of the scaffold | 4.1 Failure of scaffold. | 4.1.1 Verify scaffold is green tagged prior to use. 4.1.2 Verify safe working load of the scaffold is Displayed and adhered to. 4.1.3 Verify scaffold is placed on correct feet and base plates. 4.1.4 Verify scaffold is secured/ tied off if required. 4.1.5 Verify wheels of the mobile tower scaffold are locked. 4.1.6 Verify scaffold is placed in firm ground. | S | M | L |
|  | 4.2 Fall on the same level. | 4.2.1 Verify scaffold platform is free from any obstruction. 4.2.2 Verify scaffold platform is free from oil, grease & mud. 4.2.3 Always look and scan ahead of your route. 4.2.4 Hands should always be out of the pocket. 4.2.5 Avoid stepping over cables, tools, materials, etc. 4.2.6 Verify platform planks are secured and extends 12” beyond its support/ledger | U | M | L |
|  | 4.3 Falling materials. | 4.3.1 Comply with Site PPE Regulation (Hard hat, Steel toe safety shoes & eye protection). 4.3.2 Verify toe board in place. 4.3.3 Verify no unsecured materials above. | U | M | L |
|  | 4.4 Protruding materials. | 4.4.1 Always look and scan ahead of your route 4.4.2 Always follow mandatory warning signage 4.4.3 Verify that adequately illumination is provided on site. 4.4.4 Verify all protruding materials are provided with end caps. | U | M | L |
|  | 4.5 Falling from height. | 4.5.1 Verify edge/fall protection are in place. 4.5.2 While working on scaffold Verify no alteration to take place of any scaffold components, handrails, mid rails, toe boards, bracing, ladder, frames, platform, couplers or ties. 4.5.3 Verify no large gaps in between the platform planks or between the platform and working surface. 4.5.4 Verify platform planks are secured and extends 12” beyond its supports/ledgers 4.5.5 Do not over reach beyond the guardrails 4.5.6 Never lean on guard rails. | U | Cr | L |
| 5. Climbing down from the scaffold through stairway/ladder. | 5.1 Falling from height. | 5.1.1 Always maintain 3 point contact while using ladder. 5.1.2 Verify the rungs; stair& working platform are not slippery. 5.1.3 Personal protection equipment should be worn at all times, wear safety harness if required & anchor the lanyard to safe anchorage point. 5.1.4 Verify scaffold platform / ladder is not slippery. 5.1.5 Verify proper footwear is worn and is free from mud/grease. 5.1.6 Always face the ladder when climbing up or down 5.1.7 Do not carry a load on a ladder 5.1.8 Verify that adequate illumination is provided on site. 5.1.9 Verify firm and solid landing platform / ground. | U | Cr | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Personal Protective Equipment (PPE). | 1. SH&E Site Induction. 2. Working at height. 3. Safe use of scaffolds. | PPE to be inspected monthly. Verify daily inspection of scaffold and ladder is done. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |