| Activity/Work Task: Inspection Using Mechanical Equipment | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Inspection of Mechanical Equipment. 1. Inspection of Mechanical Equipment. (Cont…) | 1.1 Falls on level. | 1.1.1 Verify designated walkway is clear. Avoid protruding parts of the equipment. Avoid stepping over obstacles such as tools, materials and cables. 1.1.2 Correct safety footwear should be worn. 1.1.3 No oil, grease, and other slippery substance on floors and shoes. Avoid slippery walkway surface. 1.1.4 Verify adequate illumination/lighting provided. 1.1.5 Follow mandatory signs. | S | M | L |
|  | 1.2 Exposure to contact injuries (e.g. entanglement, abrasions, cuts, crashed). | 1.2.1 Attend site orientation / pre task briefing prior to start activity. 1.2.2 Do not wear loose clothing. 1.2.3 Verify guard or cover of the moving parts is in place. 1.2.4 Verify lock-out / tag-out procedure is implemented for any equipment to be inspected. 1.2.5 Maintain safe distance from the machine. Do not place/put any body part near any moving part of the equipment. 1.2.6 Do not operate the equipment unless trained and authorized. 1.2.7 Verify adequate illumination/lighting is provided. 1.2.8 Follow mandatory signs. | S | Cr | M |
|  | 1.3 Exposure to hot surface. | 1.3.1 Verify contractor has identified and highlighted the hot parts of the equipment. 1.3.2 Maintain safe distance from the hot parts of the equipment. 1.3.3 Wear appropriate hand gloves if required. 1.3.4 Follow mandatory signs. | S | M | L |
|  | 1.4 Exposure to high noise level | 1.4.1 Refer to AHA 19 Exposure to noise | S | M | L |
|  | 1.5 Exposure to fumes | 1.5.1 Verify adequate ventilation Is provided. 1.5.2 Wear appropriate masks 1.5.3 Avoid getting near the exhaust of the equipment. 1.5.3 Follow mandatory signs. | S | M | L |
|  | 1.6 Exposure to vibrations | 1.6.1 Duration to exposure should be limited. | S | M | L |
|  | 1.7 Falling materials | 1.7.1 Wear appropriate head protection (hard hat). 1.7.2 Verify no unsecured materials are placed overhead. | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Personal Protective Equipment. | SH&E Site Induction. | All PPE should be inspected prior to use. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |