| Activity/Work Task: Inspection Using Mobile Elevating Work Platform (MEWP) | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Arrival on site / Pre inspection of MEWP. | 1.1 Falls on level. | 1.1.1 Verify walkway is clear. 1.1.2 Slippery surface should be avoided. 1.1.3 Wear correct footwear. | U | N | L |
| 2. Stepping into MEWP. | 2.1 Falls on level. | 2.1.1 Verify access gate is clear from any obstruction. 2.1.2 Verify no slippery substance on the steps and landing platform surface. 2.1.3 Verify 3 points of contact while stepping into the platform. | S | M | L |
| 3. Inspection on MEWP. 3. Inspection on MEWP. (Cont…) 3. Inspection on MEWP. (Cont…) | 3.1 Incompetent operator. | 3.1.1 Verify operator is third party certified to use the type of MEWP for which he is operating. | S | M | L |
|  | 3.2 Failure of MEWP. | 3.2.1 Verify MEWP is certified and in good condition. 3.2.2 Verify daily inspection being carried out by the operator prior to use (see the inspection report). | S | Cr | M |
|  | 3.3 Overturning / tipping over. | 3.3.1 Verify MEWP is suitable and sufficient for the ground condition. 3.3.2 Verify outriggers (if available) are properly deployed and extended and not placed near edge of excavation, manholes, etc. 3.3.3 Verify platform is not overloaded. Check Safe Working Load (SWL). 3.3.4 Verify MEWP maneuvering should be avoided while platform in the raised position unless designed to do so. 3.3.5 Verify tires are choked / blocked to prevent unintended movement. | S | Cr | M |
|  | 3.4 Falls from height. | 3.4.1 Appropriate safety harness must be used with lanyard attached to load bearing anchor. 3.4.2 Verify railings and toe boards are complete. 3.4.3 Verify gate is secured and closed. 3.4.4 Do not elevate your position by standing on the guardrail. | S | Cr | M |
|  | 3.5 Contact with fixed machinery. | 3.5.1 Verify operator is third party certified and operate the type of MEWP for which he is trained. 3.5.2 Verify operator is site oriented and familiar with the working area. 3.5.3 Verify banksman or spotter is designated to check the clearance between the equipment and the fixed machinery. 3.5.4 Verify operator safely and smoothly operates the equipment. | S | M | L |
|  | 3.6 Contact with moving vehicles. | 3.6.1 Verify work area is cordoned off around MEWP. 3.6.2 Verify competent banksman is designated. 3.6.3 Verify traffic management plan is observed and implemented. | S | Cr | M |
|  | 3.7 Adverse weather / strong wind. | 3.7.1 Verify MEWP should not operate beyond wind speed limit specified by manufacturer. 3.7.2 Work should be immediately stopped and platform to be brought down on the ground. | S | M | L |
|  | 3.8 Overhead power lines. | 3.8.1 Avoid working near overhead electrical cables. 3.8.2 Verify safe working distance is maintained from overhead live electrical cables. | S | Cr | M |
|  | 3.9 Heat Stress. | 3.9.1 Exposure under hot environment should be regulated. 3.9.2 Avoid dehydration. Drink water frequently in small amounts. 3.9.3 Wear light and comfortable clothing during hot weather. | S | M | L |
| 4. Getting off from MEWP. | 4.1 Falls on level. | 4.1.1 Use 3 points of contact. 4.1.2 No oil and grease on steps or shoes. 4.1.3 Use step/ladder provided. | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Basic PPE + Safety Harness | 1. SH&E Site Induction. | All PPE should be inspected prior to use. 3rd party certification of MEWP All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |