| Activity/Work Task: Inspection in Tunnels | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Entering the tunnel. | 1.1 Refer to AHA 25 Confined Space Entry. | 1.1.1 Refer to AHA 25 Confined Space Entry. | S | Cr | M |
| 2. Inspection inside the tunnel (under normal conditions). 2. Inspection inside the tunnel (under normal conditions). (Cont…) | 2.1 Refer to AHA 25 Confined Space Entry. | 2.1.1 Refer to AHA 25 Confined Space Entry. | S | Cr | M |
|  | 2.2 Refer to AHA 16 Exposure to Dust. | 2.2.1 Refer to AHA 16 Exposure to Dust. | S | M | L |
|  | 2.3 Refer to AHA 15 Exposure to Noise. | 2.3.1 Refer to AHA 15 Exposure to Noise. | S | M | L |
|  | 2.4 Exposure to Moving Plant and Vehicles. | 2.4.1 Wear high-visibility vest. 2.4.2 Use designated walkways. 2.4.3 Do not stay behind plant/vehicles. 2.4.4 Look out for moving plants and vehicles. 2.4.5 Verify banksmen are provided in the area. | S | Cr | M |
|  | 2.5 Ground conditions: obstructions (tools, materials, cables, etc…) | 2.5.1 Keep hands free (not in pocket) while walking on site. 2.5.2 Be aware/look ahead. 2.5.3 Verify suitable lighting is provided on site. 2.5.4 Keep shoe laces tied. 2.5.5 Don’t step over obstructions walk around. | S | M | L |
|  | 2.6 Slippery ground conditions. | 2.6.1 Verify that walking surface does not have oil, grease or water. 2.6.2 Avoid slippery ground surfaces where possible. 2.6.3 Use alternate route. 2.6.4 Use suitable footwear. | S | M | L |
|  | 2.7 Uneven/unlevel ground. | 2.7.1 Keep hands free (not in pocket) while walking onsite. 2.7.2 Be aware/look ahead. 2.7.3 Verify suitable lighting is provided on site. 2.7.4 Avoid unlevel/uneven ground and use alternate route. | S | M | L |
|  | 2.8 Falls on same level. | 2.8.1 Use designated route/walkway. 2.8.2 Look ahead/be aware. 2.8.3 Keep hands free (not in pocket) while walking on site. 2.8.4 Follow mandatory signs on site. 2.8.5 Avoid grease mud water on walking surfaces where possible. 2.8.6 Avoid stepping over obstacles such as tools, materials and cables. | O | N | L |
|  | 2.9 Excavations/ground openings. | 2.9.1 Follow mandatory signs on site. 2.9.2 Verify edge protections are in place. 2.9.3 Use designated walkways. | S | Cr | M |
|  | 2.10 Walking in extreme temperatures. | 2.10.1 Check for humidity/temperature outside before moving across site. 2.10.2 Wear correct clothing and PPE. 2.10.3 Reduce exposure. 2.10.4 Implement work rotations. 2.10.5 Stay hydrated during hot weather condition. 2.10.6 Use skin protection like sun block, etc. during hot weather conditions. | S | Cr | M |
|  | 2.11 Poor illumination. | 2.11.1 Verify contractor has completed illumination level monitoring of the area. 2.11.2 Verify provision of lighting system in the absence of natural light. | S | N | L |
| 3. Inspection inside the tunnel during blasting operations. | 3.1 Refer to AHA 16 Exposure to Dust. | 3.1.1 Refer to AHA 16 Exposure to Dust. | S | M | L |
|  | 3.2 Refer to AHA 15 Exposure to Noise. | 3.2.1 Refer to AHA 15 Exposure to Noise. | S | M | L |
|  | 3.3 Flying rocks and debris. | 3.3.1 Do not enter the tunnel prior to and during blasting operation. 3.3.2 Verify contractor had provided warning signs and barricades to cordon off people from the danger area. 3.3.3 Verify work is being carried out by authorized blasting specialist. 3.3.4 Verify contractor had provided adequate shielding to minimize rock and debris throw during blasting. 3.3.5 Verify contractor has suitable warning alarm to Verify evacuation of blasting zone. | S | Cr | M |
|  | 3.4 Exposure to air blast, fire and ground shock. | 3.4.1 Do not enter the tunnel prior to and during blasting operation. 3.4.2 Verify contractor had provided warning signs and barricades to cordon off people from the danger area. 3.4.3 Verify work is being carried out by authorized blasting specialist. | S | Cr | M |
| 4. Inspection inside the tunnel during scaling. | 4.1 Rock fall or collapse as result from instability of exposed rock surface. | 4.1.1 Do not enter the danger zone before scaling is completed. 4.1.2 Verify contractor had provided warning signs and barricades to cordon off people from the danger area. | S | Cr | M |
|  | 4.2 Refer to AHA 16 Exposure to Dust. | 4.2.1 Refer to AHA 16 Exposure to Dust. |  |  |  |
|  | 4.3 Refer to AHA 15 Exposure to Noise. | 4.3.1 Refer to AHA 15 Exposure to Noise. |  |  |  |
|  | 4.4 Exposure to Moving Plant and Vehicles. | 4.4.1 Refer to 2.4.1 to 2.4.4 |  |  |  |
| 5. Inspection inside the tunnel during removal of blast spoil. | 5.1 Exposure to Moving Plant and Vehicles. | 4.4.1 Refer to 2.4.1 to 2.4.4 | S | Cr | M |
|  | 5.2 Refer to AHA 16 Exposure to Dust. | 5.2.1 Refer to AHA 16 Exposure to Dust. |  |  |  |
|  | 5.3 Refer to AHA 15 Exposure to Noise. | 5.3.1 Refer to AHA 15 Exposure to Noise. |  |  |  |
|  | 5.4 Falling materials during loading operations. | 5.4.1 Do not enter into the loading area. |  |  |  |
|  |  | 5.4.2 Verify dumpers are not overloaded. |  |  |  |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |  |
| - Personal Protective Equipment. | - SH&E Site Induction. - Confined space training | Pre-operation plant inspection. Confined Space Entry Pre-entry Inspection All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |  |