| Activity/Work Task: Inspection of Concrete Batch Plants | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| Inspection of Concrete Batching Plants Inspection of Concrete Batching Plants (Cont) Inspection of Concrete Batching Plants (Cont) | 1.1 Struck by passing plant or vehicle. | 1.1.1 Observe all site rules on traffic and pedestrian areas. 1.1.2 Use designated walkways and pedestrian routes where possible. 1.1.3 Do not use headphones or talk on a mobile phone while walking around site. 1.1.4 Do not walk behind any vehicle; where possible keep eye contact with the Operator/Driver. | S | M | L |
|  | 1.2 Exposure to Dust. | 1.2.1 Wear a dust mask in line with the recommendations in the Material safety Data Sheet; ensure the correct filter is fitted for respirator type masks. 1.2.2 Make sure all clothes are thoroughly cleaned off before entering vehicles or buildings. 1.2.3 Follow any mandatory signage that has been displayed regarding the wearing of PPE. | S | M | L |
|  | 1.3 Exposure to Noise. | 1.3.1 Follow any mandatory signage that is displayed. 1.3.2 Carry a set of ear plugs in your PPE just case there is sudden exposure. 1.3.3 Where possible limit exposure times in noisy environments. | S | M | L |
|  | 1.4 Entrapment/entanglement in machinery. | 1.4.1 Tuck in loose clothing, don’t wear head scarves, or ID cards on a neck chain/lanyard. 1.4.2 Follow all mandatory guidance signage, e.g. out of bounds, do not touch. 1.4.3 Be aware that machinery may start unexpectedly. 1.4.4 Do not touch any mechanical machinery. | S | M | L |
|  | 1.5 Exposure to Electrical Equipment. | 1.5.1 Follow all mandatory signage and pay attention to yellow hazard warning signs. 1.5.2 Do not touch any electrical equipment or cables during the inspection. | S | M | L |
|  | 1.6 Fire or Explosion. | 1.6.1 On arrival at the site ensure that you are given a full SH&E induction that should include actions to take in the event of fire or explosion. | S | M | L |
|  | 1.7 Slips, Trips and Falls | 1.7.1 Wear correct safety shoes with good grip. 1.7.2 Observe the ground in front of you when walking, do not run. 1.7.3 Avoid where possible walking in water, mud or wet sand. 1.7.4 Use designated pathways, walk around obstacles as opposed to stepping over them. | S | M | L |
|  | 1.8 Falls from Height. | 1.8.1 Do not enter or climb onto walkways or platforms that are not protected on all sides by a hand rail, a mid rail and a toe board and that are of substantial and safe construction. 1.8.2 Do not lean against barriers guard rails or barriers. 1.8.3 Do not use a working at height harness to gain access unless you have received full training in the site operator’s safe system of work and have been certified as physically fit and competent. | S | Cr | M |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Hard Hat, Safety Glasses, Class 2 Reflective Vest & Safety footwear, gloves. | Site Induction | PPE before use All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |