| Activity/Work Task: Inspection of Painting and Spraying Works | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Arrival on site. | 1.1 Unfamiliar with site layout and emergency plans. | 1.1.1 SH&E induction prior to any activity on site. | S | M | L |
|  | 1.2 Unfamiliar with painting and spraying works. | 1.2.1 Painting and spraying operation orientation/Pre task briefing by contractor. |  |  |  |
|  | 1.3 Falls on level. | 1.3.1 Use designated route/walkway. 1.3.2 Look ahead/be aware. 1.3.3 Keep hands free (not in pocket) while walking on site. 1.3.4 Follow mandatory signs on site 1.3.5 Avoid grease mud water on walking surfaces where possible. 1.3.6 Avoid stepping over obstacles such as tools, materials and cables. 1.3.7 Keep shoe lace tied. |  |  |  |
| 2. Conducting Inspection on site using scaffolding, ladder or Mobile Elevated Working Platform (MEWP). | 2.1 Refer to AHA 03 - Use of Scaffold, AHA 06 - Use of ladder and AHA 13 - Use of Mobile Elevated Working Platform (MEWP). | 2.1.1 Refer to AHA 03 Use of Scaffold, AHA 06 Use of ladder and AHA 13 Use of Mobile Elevated Working Platform (MEWP). |  |  |  |
| 3. Observing painting and spraying works. | 3.1 Exposure to paint fumes. | 3.1.1 Verify proper distance on painting operation and use proper respiratory mask intended for the task. | S | Cr | M |
|  | 3.2 Contact with paint. | 3.2.1 Avoid direct contact with paint material. Wear safety glass and hand gloves. Verify eyewash and emergency shower is available for first aid use. |  |  |  |
|  | 3.3 Uncontrolled sources of heat, sparks or flame. | 3.3.1 Verify no sources of heat, sparks or flame near painting works that use solvent base paints. 3.3.2 Verify proper controls and preparations are set-up on site during painting works near sources of heat, sparks or flame. Check for the availability of fire protection, fire extinguishers. |  |  |  |
|  | 3.4 Exposure to noise. | 3.4.1 Refer to AHA 20 – Exposure to Noise |  |  |  |
|  | 3.5 Uncontained paint materials/spillage. | 3.5.1 Verify paint containers or paint materials are provided with necessary drip trays or containments for possible spillage. |  |  |  |
|  | 3.6 Exposure to pressurized hose line. | 3.6.1 Verify pressurized hose lines are in good condition and regular maintenance is being conducted by contractor. 3.6.2 Verify hose connections have whip checks. 3.6.3 Maintain safe distance from spray painting operation. Wear appropriate PPE such as safety glass and respiratory mask intended for paint fumes. |  |  |  |
|  | 3.7 Failure of pressurized hose line. | 3.7.1 Verify that contractor inspected the pressurized hose line prior to use. 3.7.2 Maintain safe distance and look out for possible leaks or hose damage. 3.7.3 Wear appropriate PPE such as safety glass and respiratory mask intended for paint fumes. 3.7.4 Verify proper hose clips/crimps are fitted. 3.7.5 Verify hose pressure capacity is not exceeded. |  |  |  |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Personal Protective Equipment. | SH&E Site Induction. | All PPE should be inspected prior to use. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |