| Activity/Work Task: Inspection of Piling Operations | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): Pa | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Inspection near the rotary drill rig machine. 1. Inspection near the rotary drill rig machine. (Cont…) | 1.1 High level of noise coming from the drill rig machine. | 1.1.1 Ensure contractor has identified the source of the noise and proper safety signage and hearing protection are provided. 1.1.2 Follow mandatory signage provided as safety measures on the noisy machine. 1.1.3 Ensure PPE provided is suitable for the specific noise level and exposure identified. 1.1.4 Ensure that contractor is carrying out regular noise measurement to determine the level of noise exposure and provide the necessary hearing protection. 1.1.5 Where possible, limit exposure to the drill rig machine. | S | M | L |
|  | 1.2 Exposure to dust coming from the rotary drill. | 1.2.1 Ensure PPE provided is suitable and sufficient for the specific dust identified. 1.2.2 Ensure that contractor is carrying out dust suppression. 1.2.3 Ensure that contractor provided proper containment for the dust producing activity. | S | M | L |
|  | 1.3 Slippery / muddy ground condition. | 1.3.1 Maintain walking surface by making sure it is free from mud, oil, dirt, or water. 1.3.2 Avoid slippery ground surfaces where possible. 1.3.3 Use alternate route. 1.3.4 Wear approved footwear that is free of mud/oil/grease. | S | M | L |
|  | 1.4 Rotating parts of the drill rig. | 1.4.1 Verify guards are fitted on all exposed rotating parts of the machine. Any defective guards must be reported. 1.4.2 Maintain a safe distance away from the drill rig machine. 1.4.3 Avoid wearing loose clothing. 1.4.4 Stay away from the slewing radius of the machine and ensure contractor had demarcated the area. | U | Cr | L |
|  | 1.5 Drill rig hitting underground utilities. | 1.5.1 Verify contractor had secured a permit to dig prior to boring. 1.5.2 Verify trial pits and utility clearance program had been conducted prior to the boring activity. | S | Cr | M |
|  | 1.6 Unexpected loss of stability of the drill rig machine. | 1.6.1 Verify machine is situated on a level and stable ground. 1.6.2 Verify operator is trained and certified to operate the drill rig machine. 1.6.3 Drill rig machine shall not be allowed to travel with its mast raised. | U | Cr | L |
|  | 1.7 Uncontrolled rotation or motion of drill rig. | 1.7.1 Maintain a table of recommended drill speeds for the drill size and material is available to all drill operators. 1.7.2 Verify operator is trained and certified to operate the drill rig machine. 1.7.3 Verify means of emergency stopping of machines is fitted. | U | Cr | L |
| 2. Inspection of bored hole in the ground. | 2.1 Absence of edge protection and warning signs. | 2.1.1 Maintain a safe distance away from the edge of the bored hole. 2.1.2 install an approved cover, edge protection and warning sign immediately. | U | Cr | L |
| 3. Inspection during lifting of pile casing or steel cages. | 3.1 Refer to AHA# 09 Observing lifting operations. | 3.1.1 Refer to AHA# 09 Observing lifting operations. | S | Cr | M |
|  | 3.2 Absence or poor lifting points on the steel casing or steel cages. | 3.2.1 Verify steel casings or steel cages are lifted on its designated lifting points. 3.2.2 Verify steel casings or steel cages are lifted vertically. 3.2.3 install additional restraining clamps to avoid slippage during lifting. | S | Cr | M |
| 4. Inspection during driving / pull-out of steel casing or sheet piles into / from the ground. | 4.1 High level of noise coming from the vibro-hammer and compressor source. | 4.1.1 Follow mandatory signage provided as safety measures on the noisy equipment. 4.1.2 Verify PPE provided is suitable for the specific noise level and exposure identified. 4.1.3 Verify that contractor is carrying out regular noise measurement to determine the level of noise exposure and provide the necessary hearing protection. 4.1.4 Where possible, limit exposure to the case driving activity. | S | M | L |
|  | 4.2 Toppling of steel casing. | 4.2.1 Verify that a firm grip is established between the vibro-hammer and steel casing. 4.2.2 Verify that vibro-hammer operator is trained and certified to operate the machine. 4.2.3 Stay a distance away (more than the length of the casing) from the driving activity. 4.2.4 Verify that contractor did a pre-operational check of the vibro-hammer machine including all its hydraulic hoses. | S | Cr | M |
|  | 4.3 Falling or shooting debris. | 4.3.1 Verify casing / sheet piles are free from mud and debris that may fall during the driving activity. | S | M | L |
|  | 4.4 Hose Failure. | 4.4.1 Install whip-line protection at all hose connections. 4.4.2 Install pressure relief valves are provided at compressor source. | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Personal Protective Equipment. | 1. SH&E Site Induction. | - Pre-operation plant inspection. - Third party test certificate of crane, drill rig and lifting tackles. - Excavation Permit 4. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. 5. Effort should be made to communicate findings to contractor. |  |  |  |