| Activity/Work Task: Inspection on Open Floors / Floor Voids | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Inspection. | 1.1 Falls from height. | 1.1.1 Verify edge protection is in place before proceeding with the inspection. 1.1.2 Follow mandatory warning signage and out of bounds areas. 1.1.3 Do not approach unprotected leading edge. 1.1.4 Verify sufficient lighting provided in the area. 1.1.5 Use only designated routes/walkways. 1.1.6 Do not step on/over covered voids where possible. | U | Cr | L |
|  | 1.2 Falls on same level. | 1.2.1 Use only designated routes/walkways in the site. 1.2.2 Always look and scan ahead of your way. 1.2.3 Hands should always be out of the pocket. 1.2.4 Always follow mandatory warning signage. 1.2.5 Verify sufficient lighting provided in the area. 1.2.6 Avoid muddy/greasy/wet floors where possible. 1.2.7 Avoid stepping over cables, tools, materials, etc. | U | M | L |
|  | 1.3 Falling materials. | 1.3.1 Do not walk under unprotected work/materials. 1.3.2 Wear the mandatory PPE. 1.3.3 Always follow mandatory warning signage. | S | M | L |
|  | 1.4 Protruding materials. | 1.4.1 Use only the designated routes/walkways in the site. 1.4.2 Always look and scan ahead of your way. 1.4.3 Always follow mandatory warning signage. 1.4.4 Verify sufficient lighting provided in the area. 1.4.5 Maintain distance away from embedded rebars / stakes/marking pins. 1.4.6 Avoid stepping over scrap wood, formworks / shuttering materials. 1.4.7 Verify all protruding rebars are provided with caps. | U | M | L |

| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |
| --- | --- | --- |
| Personal Protective Equipment. | SH&E Site Induction. | PPE should be inspected prior to use. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |