| Activity/Work Task: Invasive And Injurious Plants | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Working Near Plants | - Contact With Skin | - Gloves shall be worn by Staff - Skin shall be covered at all times | S | M | L |
|  | - Preventive Spread On Invasive Weeds | - Material contaminated with invasive weeds shall not be moved from one site to another. - Treatment of invasive weeds shall be undertaken in such a way as to avoid the spread of these plants. | S | M | L |
|  | - Waste Disposal | - All invasive weeds shall be disposed of through a licensed waste contractor or in consultation with the Main Contractor. - All bags and skips containing materials contaminated with invasive weeds shall be sealed or covered when being moved. | S | M | L |
|  | - On Site Waste | - A Site Waste Management Plan (SWMP) shall be in place for the Etihad Rail project - All waste shall be kept securely and transferred to a licensed contractor. | S | M | L |
|  | - Recovery and Return of Scrap Metal | - Mixing scrap metal with other wastes may result in refusal of uplift so staff shall be briefed on waste segregation | S | M | L |
|  | - Waste Electrical and Electronic Equipment | - Waste Electrical and Electronic Equipment shall be stored separately to other waste and disposed of using a licensed waste contractor | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Hand gloves - Respiratory protection (As Needed) - Safety Glasses | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly - When Required |  |  |  |