| Activity/Work Task: Laboratory Inspection | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Walking around the laboratory. 1. Walking around the laboratory. (Cont…) | 1.1 Ground conditions: obstructions (tools, materials, cables, etc…) | 1.1.1 Keep hands free (not in pocket) while walking on site. 1.1.2 Be aware/look ahead. 1.1.3 Verify suitable lighting is provided on site. 1.1.4 Keep shoe laces tied. 1.1.5 Don’t step over obstructions walk around. | O | M | M |
|  | 1.2 Slippery ground conditions. | 1.2.1 Verify that walking surface does not have oil, grease or water. 1.2.2 Avoid slippery ground surfaces where possible. 1.2.3 Use alternate route. 1.2.4 Use suitable footwear. | O | M | M |
|  | 1.3 Poor Illumination. | 1.3.1 Verify contractor has completed illumination level monitoring of the area. 1.3.2 Verify provision of lighting system in the absence of natural light. | S | M | L |
|  | 1.4 Exposure to chemicals due to poor storage. | 1.4.1 Verify all the chemicals in the laboratory are properly stored and segregated. 1.4.2 Verify all chemicals are identified and labeled. 1.4.3 Verify copies of MSDS are kept in the laboratory and personnel are trained on MSDS. 1.4.4 Wear the appropriate PPE when entering the laboratory. | S | Cr | M |
|  | 1.5 Presence of sharp and protruding materials. | 1.5.1 Always look and scan ahead of your way. 1.5.2 Follow all warning signs in the laboratory. 1.5.3 Verify that adequately illumination is provided on the laboratory. 1.5.4 Verify laboratory equipment are provided with guards. 1.5.5 Verify materials in the laboratory are properly stored. | S | M | L |
|  | 1.6 Exposure to hot surfaces. | 1.6.1 Verify contractor has identified and highlighted the hot surfaces in the laboratory. 1.6.2 Maintain safe distance from the identified hot surfaces in the laboratory. 1.6.3 Wear appropriate hand gloves if required. 1.6.4 Follow mandatory signs. | S | M | L |
| 2. Witnessing of the calibration and testing of materials. | 2.1 Exposure to projectiles during testing. | 2.1.1 Verify sample materials are tested in an isolated area/chamber. 2.1.2 Wear the appropriate eye protection when entering the laboratory. | S | M | L |
|  | 2.2 Exposure to silica dust during sieve analysis and concrete testing. | 2.2.1 Wear the appropriate eye & respiratory protection when witnessing tests. 2.2.2 Verify sample materials are tested in an isolated area/chamber or local exhaust ventilation is in place. | S | M | L |
|  | 2.3 Exposure to heavy metal (mercury in sealed glass manometers) during testing of asphalts. | 2.3.1 Verify manometers are placed in non-porous pan during use. 2.3.2 Verify laboratory personnel follow the correct test protocols. 2.3.3 Verify laboratory personnel do not overpressure test apparatus. | S | M | L |
|  | 2.4 Exposure to high pressure lines. | 2.4.1 Verify contractor had conducted a walk-thru inspection prior to testing. 2.4.2 Verify all pressure relief valves are identified and protected. 2.4.3 Stay away from known locations of pressure relief valves. | S | M | L |
|  | 2.5 Exposure to contact injuries (e.g. entanglement, abrasions, cuts, crashed) from mechanical testing equipment. | 2.5.1 Do not wear loose clothing. 2.5.2 Verify guard or cover of the moving parts is in place. 2.5.3 Verify lock-out / tag-out procedure is implemented for any equipment to be inspected. 2.5.4 Maintain safe distance from the machine. 2.5.5 Do not operate the equipment unless trained and authorized. 2.5.6 Verify adequate illumination/lighting is provided. 2.5.7 Follow mandatory signs. 2.5.8 Do not place/put any body part near any moving part of the equipment. | S | Cr | M |
|  | 2.6 Exposure to high noise level. | 2.6.1 Refer to AHA 15 Exposure to noise. | S | M | L |
|  | 2.7 Use of chemicals during testing. | 2.7.1 Verify all chemicals are identified and labeled. 2.7.2 Verify copies of MSDS are kept in the laboratory and personnel are trained on MSDS. 2.7.3 Wear the appropriate PPE when entering the laboratory. 2.7.4 Verify sample materials are tested in an isolated area/chamber or local exhaust ventilation is in place. | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |  |
| - Personal Protective Equipment. | 1. SH&E Site Induction. | All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |  |