| Activity/Work Task: Lifting And Thimbling Operations | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Lifting and Thimbling Operations | - Lifting Failure | - A pre-site survey shall be performed before any crane operations are started, including a check of ground conditions for suitability for cranes and loads to be lifted - A site specific risk assessment, method statement and job lifting plan shall be produced for each lift by a competent person - Check correct selection of the crane for the task and that it will be able to lift the required loads, within a given radius or reach by consulting the duty charts provided - Safe Working Limit, Radius and Load and Rated Capacity Indicators shall be operable at all times and not turned off - All staff involved in the use of cranes, including operators, controllers, banksman, signallers, slingers shall be trained and competent in those duties - Suitable exclusion zones dependant on lift being undertaken shall be created and maintained - All cranes and associated equipment (including lifting tackle) shall be subjected to a statutory inspection and a program of planned preventative maintenance - Operators and slingers shall perform pre-user checks prior to all lifts to check that all lifting equipment (including loose lifting accessories) is free from defects - Effective communication (signals or back to back radios) shall be in place between operators, controllers, signaler’s and slingers - The route of travel for travelling cranes shall be kept free from obstructions and the operator shall have a clear view of the route – movements shall be controlled by a controller or banksman - Suitable lighting shall be provided on all sites - Prior to work the operator shall check the condition of the ground so that it is suitable for the lift - Spreader plates and outriggers shall be deployed and level prior to making lifts - Contact any airfield if work is performed within 6km and operations will exceed 10 meters or above surrounding structures and trees | S | M | L |
|  | - Overturning of RRV | - All lifting shall be performed in accordance with the lifting plan - Outriggers shall be used where provided and practicable - Crane operations shall only be performed on level and firm ground whenever reasonably practicable | S | M | L |
|  | - Loss Of Load | - All lifts shall be performed within the safe working limit of the crane and its lifting tackle - Correct lifting techniques shall be used i.e. loads not snatched - Loads shall be carried slowly, movements controlled, as low as reasonably practicable. No-one shall be permitted to be underneath the load when being lifted - The centre of gravity and lifting points shall be indicated on the load and the load slung accordingly - Slinging shall only be performed by competent persons and using correct equipment that has been the subject of statutory inspections | S | M | L |
|  | - Struck By Load/ Falling Objects/ Crane Movements | - Check and secure / remove all loose objects or materials from loads prior to lifting - Use handlines where necessary - Loads shall not pass over people | S | M | L |
|  | - Electrocution | - See AHA Working adjacent to DC electrified rails - See Risk AHA Working near electrified overhead line equipment | S | M | L |
|  | - Personal Injury | - The operator shall have a clear view of the route – movements shall be controlled - Suitable lighting shall be provided on all sites - Use “Pick & lift” or “Pick & Carry” technique only – check snag lines or controlled handling device are used to restrain the rail. - Exclusion zones of 3m lateral and 5m from the ends of the rail shall be created and maintained | S | M | L |
|  | - Tandem Lifting | - Tandem lifting shall only be performed by crane operators who have been trained and competent in these duties - Effective means of communication shall be available, tested and working between all parties. Where radios are used, these shall use dedicated frequencies to relay instructions. - On or off tracking shall not take place when undertaking a tandem lift - Agree the lifting plans with the operators - Load shall remain “plumb” – load to be vertically below the hook at all times on both machines | S | M | L |
|  | - Tandem Lifting Derailment | - The normal safe working load of each machine (for the required duty and radius) shall be not less than the calculated share of the load plus 50% - Where one machine is to take more than half the weight of the load, both machines shall have a safe working load of not less than the heaviest load plus 50% (for the required duty and radius) - Rated Capacity Indicators shall be fully functioning on all machines used for tandem lifting - Where one machine is to operate in rail mode and the other in road mode, both machines RCI shall be operated in the lower rated mode. (which is usually rail mode) | S | M | L |
|  | - Thimbling | - Check the Rail Road Vehicle has “green” or “orange” data panel sticker and is used only where authorized. - Rail length shall be greater than 100m - Exclusion zones of 3m lateral and 5m from the ends of the rail shall be created and maintained - Correct machine shall be selected with a minimum SWL of 2 tone for the radius of operation - Check thimble is correctly attached to the machine - Land rail on suitable blocks before moving thimble position | S | M | L |
|  | - Overturning Of Rail Road Vehicle Whilst Thimbling | - All Thimbling shall be performed in accordance with the lifting plan. - Thimbling operations shall be carried out over the fixed end of the machine. - Rail shall be free to move | S | M | L |
|  | - Personal Injury | - Secure / remove all loose objects or materials from site prior to Thimbling | S | M | L |
|  | - Loss of Load While Thimbling | - Check the thimble is compatible with the rail section to be lifted - Check thimble is correctly attached to the rail - Correct lifting techniques shall be used i.e. loads not snatched - Slinging shall only be performed by competent persons and using correct equipment that has been the subject of statutory inspections | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Safety Glasses - Hand Gloves | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly - When Required |  |  |  |