| Activity/Work Task: Loan Working | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Loan Working | - Trains | - Work shall be planned using Sectional Appendix and Hazard Directory - A place of safety shall be established prior to any work commencing - Staff shall look up every 5 seconds to check for trains - See AHA for – Work on or Near the Line (Including Cab Riding) | S | M | L |
|  | - Confined Space, Hill Top and Radio sites | - Means of communication shall be available at all times, including the ability to communicate with emergency services, to summon assistance - The means of communication shall have sufficient battery life to allow for its use at all times when at site - Before attempting to access site staff shall assess the local risks e.g. underfoot conditions, weather conditions to identify additional risks that may arise - Lone workers shall arrange to call their home depot / Infrastructure Fault Control / Signal box on a regular basis when working alone in remote locations and whilst working trackside - Lone working shall not be performed in areas of known violence - Lone working shall not be undertaken in confined spaces – for working in Confined Spaces see AHA – Entry into Confined Spaces | S | M | L |
|  | - Adverse Weather | - Lone working shall not be undertaken when visibility is poor - See AHA – Work on or Near the Line (Including Cab Riding | S | M | L |
|  | - Slips, Trips and Falls | - See AHA for Underfoot Conditions | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Safety Glasses | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly - When Required |  |  |  |