| Activity/Work Task: Long Welded Rail Delivery And Recovery By Train | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Long Welded Rail Delivery And Recovery By Train | - Struck By Train | - See AHA Work On or Near the Line - Possessions shall be taken as per Rule Book - When the Rail Puller train is in operation a minimum of 425 meters shall be maintained at all times at the rear of the Rail Putler Train and any other train or OTP movements on the same Line in the engineering worksite - Separation requirement shall be briefed to all drivers, operators and other staff involved in the worksite - Communications will be by dedicated radios | S | M | L |
|  | - Derailment Of Train | - All movements will be controlled by the Traffic Officer | S | M | L |
|  | - Delivery Of Rails | - When delivering / recovering rail from train, clear communication shall be established and maintained between the train crew - Hard hats, HV Clothing (All Orange), Safety Footwear and gloves shall be worn at all times when trackside and Ear, Eye Protection & Respirators where necessary | S | M | L |
|  | - Manual Handling | - See AHA Manual Handling | S | M | L |
|  | - Electrocution | - Isolation of DC Electrified lines will be required on the line the delivery is taking place and any adjacent lines less than 4 metres distance away from the line on which the delivery is taking place - See Working Adjacent to DC Electrified Rails, - Where the Personal Safety Clearance of 600mm can not be guaranteed then an Isolation of the AC Equipment shall be taken when working on AC Electrified Lines - See AHA Working Near Electrical Overhead Line Equipment | S | M | L |
|  | - Slips, Trips and Falls | - See AHA Underfoot Conditions - Train shall be completely stopped before accessing or alighting | S | M | L |
|  | - Fuel Oil Storage | - Spillage / Environmental Kits shall be made available | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Safety Glasses - Hand Gloves - Dust Mask - Hearing Protection | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly - When Required |  |  |  |