| Activity/Work Task: Movement On Site | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title) | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Movement Across Site. | 1.1 Unfamiliar with site layout and emergency plans. | 1.1.1 SH&E induction prior to any activity on site | S | M | L |
| 2. Passing near Moving Plant and Vehicles. | 2.1 Passing near Moving Plant and Vehicles. | 2.1.1 Wear high-visibility vest 2.1.2 Use designated walkways 2.1.3 Do not stay behind plant/vehicles 2.1.4 Look out for moving plants and vehicles | S | Cr | M |
| 3. Passing in a Dusty area, dust from activities. | 3.1 Passing in a Dusty area, dust from activities. | 3.1.1 Wear appropriate dust mask. 3.1.2 Maintain good visibility before any movement on site. | S | M | L |
| 4. Walking in extreme temperatures/weather conditions. | 4.1 Walking in extreme temperatures/weather conditions | 4.1.1 Check for humidity/temperature outside before moving across site. 4.1.2 Wear correct clothing and PPE. 4.1.3 Reduce exposure. 4.1.4 Implement work rotations. 4.1.5 Stay hydrated during hot weather condition. 4.1.6 Use skin protection like sun block, etc.. during hot weather conditions. | S | M | L |
| 5. Passing to a Noisy area. | 5.1 Passing to a Noisy area | 5.1.1 Check if the contractor has conducted noise survey. 5.1.2 Follow mandatory use of PPE. 5.1.3 Reduce exposure time. | S | M | L |
| 6. Passing by excavation. | 6.1 Excavations/ground openings | 6.1.1 Follow mandatory signs on site. 6.1.2 Maintain edge protection. 6.1.3 Use designated walkways. | S | Cr | M |
| 7. Passing by lifting operations. | 7.1 Lifting Operations | 7.1.1 Stay out of cordon off lifting area. 7.1.2 Be aware of slewing crane/swing radius. 7.1.3 Where possible inform rigger/banksman of your presence. 7.1.4 Follow any instruction by lifting supervisor. 7.1.5 Never pass under a slung load. | S | Cr | M |
| 8. Passing Concrete pumping activity. | 8.1 Concrete Pumping | 8.1.1 Be aware of pump movement 8.1.2 Follow any instruction by supervisor 8.1.3 Follow mandatory on sites 8.1.4 Maintain safe distance from tremie pipe 8.1.5 Wear basic PPE’s. Hardhat, safety glasses, and safety shoes | S | Cr | M |
| 9. Passing by live utilities. | 9.1 Live utilities | 9.1.1 Ask contractor if there is any live utilities and what are the control measures provided. 9.1.2 Maintain safe distance from live utilities 9.1.3 Follow mandatory signs and instructions by the supervisor in charge | S | Cr | M |
| 10. Walking on sites. | 10.1 Ground conditions: obstructions (tools, materials, cables, etc…) | 10.1.1 Keep hands free (not in pocket) while walking on site 10.1.2 Be aware/look ahead 10.1.3 Ensure suitable lighting is provided on site 10.1.4 Keep shoe laces tied 10.1.5 Don’t step over obstructions walk around | S | M | L |
|  | 10.2 Slippery ground conditions. | 10.2.1 Maintain clean walking surfaces by preventing walking surfaces from having oil, grease or water. 10.2.2 Avoid slippery ground surfaces where possible 10.2.3 Use alternate route 10.2.4 Use suitable footwear | S S | M M | L L |
|  | 10.3 Uneven/unlevel ground. | 10.3.1 Keep hands free (not in pocket) while walking onsite 10.3.2 Be aware/look ahead. 10.3.3 maintain suitable lighting on site 10.3.4 Avoid unlevel/uneven ground and use alternate route | S | M | L |
|  | 10.4 Falls on same level. | 10.4.1 Use designated route/walkway 10.4..2 Look ahead/be aware 10.4.3 Keep hands free (not in pocket) while walking on site 10.4.4 Follow mandatory signs on site 10.4.5 Avoid grease mud water on walking surfaces where possible. 10.4.6 Avoid stepping over obstacles such as tools, materials and cables. | S | M | L |
| 12. Walking on elevated areas. | 11.1 Falls from height. | 11.1.1 Maintain approved edge protection 11.1.2 Follow mandatory warning signs on site 11.1.3 Do not approach unprotected edge 11.1.4 Use designated routes and walkways 11.1.5 Do not stop on/over covered voids where possible 11.1.6 Where required use safety harness, ensuring suitable anchor point | S | Cr | M |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Personal Protective Equipment (PPE). | - SH&E Site Induction. | - PPE should be inspected prior to use. - All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. - Effort should be made to communicate findings to contractor. |  |  |  |