| Activity/Work Task: Noise – Refueling | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Refueling | - Transporting Fuel | - All fuel shall only be carried in suitable containers and secured within the vehicle - Spill kits shall be available in all vehicles transporting fuels and oils - The spill kit capacity shall be proportional to the volume of fuel or oil carried - Spill kit shall be replaced if used | S | M | L |
|  | - Decanting Fuel From One Container To Another/ Filling Small Plant With Fuel | - Fill plant on surfaced ground so spills can be mopped up - Decant/ refuel machine using a flexible spout or funnel. - Avoid spilling any fuel on the machine. container but if spilling occurs then wipe of the excess fuel (dispose of any rags as hazardous waste and check they are kept apart from sources of ignition) and leave the machine for approximately two minutes before starting it up - Check that the fuel cap has been replaced and tightened correctly before staring the plant including the seal is in place - Check that the fuel container cap has been replaced and tightened - Where possible fill all plant at depots rather than on site. | S | M | L |
|  | - Refueling On Protected Site | - Designate a refueling area on the site and lay a spill mat over the refueling area, so in the event of a spill it is captured. | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Hand gloves - Respiratory protection (As Needed) - Safety Glasses | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures - Training in the use of spill kit | - Daily - Weekly - When Required |  |  |  |