| Activity/Work Task: Observing Arc Welding | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Observing Arc Welding. | 1.1 Exposure to UV rays / Infrared rays. | 1.1.1 Wear appropriate safety glasses with side shields and a correct grade of filter. 1.1.2 Do not directly look on the welding flash. | S | M | L |
|  | 1.2 Exposure to sparks/molten slugs. | 1.2.1 Protect your body from welding spatter with clothing made from durable, flame-resistant material. 1.2.2 Avoid clothing made of synthetic materials, which can melt when exposed to extreme heat or sparks, or cotton unless it is specially treated for fire protection. 1.2.3 Keep your clothes free of grease and oil, which may ignite. 1.2.4 Verify you are protected from spatter with non-flammable protective screens or curtains. | S | M | L |
|  | 1.3 Exposure to welding fumes and gases. | 1.3.1 Avoid getting exposed to welding fumes. 1.3.2 Wear an appropriate filter mask if exposure cannot be avoided. 1.3.3 Verify area is adequately ventilated or provided with exhaust. 1.3.4 Read and understand the Material Safety Data Sheets (MSDS) of the welding rod. | S | M | L |
|  | 1.4 Exposure to electric shock. | 1.4.1 Verify equipment is properly grounded, in good repair, and installed according to prevailing codes. 1.4.2 Verify equipment is turned off when not in use. 1.4.3 Use plywood, rubber mats or other dry insulation to stand on. 1.4.4 Don't lean on the welding machine and the work piece. 1.4.5 Verify work is not directly next to the welding power source. 1.4.6 Do not step on the electrodes or work cables. | S | Cr | M |
|  | 1.5 Exposure to fire or explosion. | 1.5.1 Verify fire hazards are removed from the welding area or covered with a fire-resistant shield if necessary. 1.5.2 Verify welding is away from unshielded fuel, hydraulic lines or flammable cylinders. 1.5.3 Verify welding is away from fumes of other processes, such as cleaning, degreasing or painting. 1.5.4 Verify firefighting equipment is available and accessible. 1.5.5 Verify a fire watcher is present in the area during the entire operation and half an hour after. 1.5.6 Keep your clothes free of grease and oil, which may ignite. 1.5.7 Check that there are no combustible materials hidden behind walls or partitions which could be ignited. 1.5.8 Use guards or covers to prevent hot particles passing through openings in floors and walls (doorways, windows, etc). | S | Cr | M |
|  | 1.6 Absence of Permit To Work (Hot Work Permit) | 1.6.1 Stop the welding activity and advise the contractor to secure a hot work permit. | S | Cr | M |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |  |  |  |
| - Personal Protective Equipment. | - SH&E Site Induction. - SH$E Hot work Training | Hot work permit inspection guidelines All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |  |  |  |