| Activity/Work Task: Observing Lifting Operations | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Arrival on Site. | 1.1 Unfamiliar with site layout and emergency plans. | 1.1.1 SH&E induction prior to any activity on site. | S | M | L |
|  | 1.2 Unfamiliar with lifting activity. | 1.2.1 Lifting operation orientation/Pre task briefing by contractor. |  |  |  |
|  | 1.3 Falls on level. | 1.3.1 Use designated route/walkway. 1.3.2 Look ahead/be aware. 1.3.3 Keep hands free (not in pocket) while walking on site. 1.3.4 Follow mandatory signs on site. 1.3.5 Avoid grease mud water on walking surfaces where possible. 1.3.6 Avoid stepping over obstacles such as tools, materials and cables. 1.3.7 Keep shoe lace tied. |  |  |  |
| 2. Passing/Approaching near lifting operations. 2. Passing/Approaching near lifting operations. (Cont…) | 2.1 Lifting operation. | 2.1.1 Stay out of cordon of lifting area. 2.1.2 Be aware of slewing crane/swing radius. 2.1.3 Where possible inform rigger of your presence. 2.1.4 Follow any instruction by lifting supervisor. 2.1.5 Never pass under a slung load. | S S | Cr Cr | M M |
|  | 2.2 Falls on level. | 2.2.1 Use designated route/walkway. 2.2.2 Look ahead/be aware. 2.2.3 Keep hands free (not in pocket) while walking on site. 2.2.4 Follow mandatory signs on site. 2.2.5 Avoid grease mud water on walking surfaces where possible. 2.2.6 Avoid stepping over obstacles such as tools, materials and cables. 2.2.7 Keep shoe lace tied. |  |  |  |
| 3. Observing lifting Operation. | 3.1 Falls on level. | 3.1.1 Use designated route/walkway. 3.1.2 Look ahead/be aware. 3.1.3 Keep hands free (not in pocket) while walking on site. 3.1.4 Follow mandatory signs on site. 3.1.5 Avoid grease mud water on walking surfaces where possible. 3.1.6 Avoid stepping over obstacles such as tools, materials and cables. 3.1.7 Keep shoe lace tied. | S | Cr | M |
|  | 3.2 Lifting Operation. | 3.2.1 Stay out of cordon of lifting area. 3.2.2 Be aware of slewing crane/swing radius. 3.2.3 Where possible inform rigger of your presence. 3.2.4 Follow any instruction by lifting supervisor. 3.2.5 Never pass under a slung load. |  |  |  |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Personal Protective Equipment. | SH&E Site Induction. | PPE should be checked prior to use. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |