| Activity/Work Task: Overseeing Non destructive Testing (NDT) Activity | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Visual Inspection | 1.1 Sharp edges/pointed objects | 1.1.1 Verify sharp/pointed portion of the object to be tested is protected or provided with insulation. 1.1.2 Wear proper hand gloves for the inspection. 1.1.3 Follow mandatory safety signs. 1.1.4 Wear appropriate safety glass. | S | M | L |
|  | 1.2 Hot surfaces | 1.2.1 Verify material to be tested is cooled down before performing the inspection. - Verify contractor has identified and highlighted the hot parts of the material to be tested. 1.2.3 Maintain safe distance from the hot part of the material. 1.2.4 Wear appropriate hand gloves if required. 1.2.5 Follow mandatory signs. | S | M | L |
|  | 1.3 Falls on level | 1.3.1 Verify designated walkway is clear. Avoid stepping over obstacles such as tools, materials and cables. 1.3.2 Correct safety footwear should be worn. 1.3.3 No oil, grease, and other slippery substance on floors and shoes. Avoid slippery walkway surface. 1.3.4 Verify adequate illumination/lighting provided. 1.3.5 Follow mandatory signs. | S | Cr | M |
|  | 1.4 Working in confined space | 1.4.1 Refer to AHA 25 Confined Space Entry |  |  |  |
|  | 1.5 Extreme weather conditions | 1.5.1 Check for humidity/temperature outside before moving across site. 1.5.2 Wear correct clothing and PPE. 1.5.3 Reduce exposure. 1.5.4 Implement work rotations. 1.5.5 Stay hydrated during hot weather condition. 1.5.6 Use skin protection like sun block, etc. during hot weather conditions. | S | M | L |
|  | 1.6 Poor ventilation | 1.6.1 Verify that contractor provides suitable ventilation on the working area. | S | Cr | M |
| 2. Dye Penetrant Inspection | 2.1 Uncontrolled sources of heat, sparks or flame. | 2.1.1 Verify no sources of heat, sparks or flame near the testing area. 2.1.2 Verify proper controls and preparations are set-up on site during testing work near sources of heat, sparks or flame. Verify that contractor provides necessary fire protection. |  |  |  |
|  |  |  | S | Cr | M |
|  | 2.2 Eye exposure to Ultra Violet (UV) light. | 2.2.1 Use approved filtered/safety glass for the activity. | S | M | L |
|  | 2.3 Exposure to Dye Penetrant Testing materials | 2.3.1 Use approved respiratory protection when needed 2.3.2 Wear safety glass and appropriate working clothes. 2.3.3 Verify that contractor maintains necessary Material Safety Data Sheet for the chemicals being used and safety precaution is being followed and informed to all employees that will be exposed to those chemicals. | S | M | L |
| 3. Eddy Current Testing | 3.1 Electrical hazards | 3.1.1 Verify that the condition of equipment being used is in good condition. 3.1.2 Verify electrical plugs and sockets are in safe condition. 3.1.3 Electrical equipment shall not be operated in areas where combustible gases or vapors may be present unless the equipment is explosion proof. 3.1.4 Verify that working area is dry. 3.1.5 Implement good cable management. | S | Cr | M |
| 4. Ultrasonic Inspection | 4.1 Electrical hazards | 4.1.1 Verify that the condition of equipment being used is in good condition. 4.1.2 Verify electrical plugs and sockets are in safe condition. 4.1.3 Electrical equipment shall not be operated in areas where combustible gases or vapors may be present unless the equipment is explosion proof. 4.1.4 Verify that working area is dry. 4.1.5 Implement good cable management. |  |  |  |
|  |  |  | S | Cr | M |
| 5. Radiographic Testing | 5.1 Electrical hazards | 5.1.1 Verify that the condition of equipment being used is in good condition. 5.1.2 Verify electrical plugs and sockets are in safe condition. 5.1.3 Electrical equipment shall not be operated in areas where combustible gases or vapors may be present unless the equipment is explosion proof. 5.1.4 Verify that working area is dry. 5.1.5 Implement good cable management. |  |  |  |
|  |  |  | S | Cr | M |
|  | 5.2 Exposure to ionizing radiation | 5.2.1 Verify that contractor provides barricades and safety signs to isolate the radiographic testing activity. 5.2.2 Maintain safe distance from the testing area. 5.2.3 Follow mandatory safety signs. 5.2.4 Regular health screening and use of dosimeter should be followed for employee who regularly exposed to radiation. | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |  |
| Personal Protective Equipment. | SH&E Site Induction. | All PPE should be inspected prior to use. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member |  |  |  |  |