| Activity/Work Task: Oxygen Fuel Gas Cutting And Heating | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Gas Cutting And Heating | - Struck By Train | - See AHA for – Work On or Near the Line | S | M | L |
|  | - Manual Handling | - Manual handling training shall be provided - See AHA for Manual Handling - Provision of mechanical lifting / carrying devices e.g. rail trolleys - Use manual handling techniques | S | M | L |
|  | - Fire Explosion | - Use trained and competent operators - Schedule planned maintenance of equipment - Conduct site specific assessment and briefing - Operator shall conduct pre-use checks by - Cylinders shall be turned off when not in use - Propane and acetylene cylinders shall be kept upright - Use Flashback arrestors - Hoses shall be purged. They shall not be coiled - Fire precautions, including provision of fire fighting equipment on site - Welding area shall be dry prior to commencement of work i.e. pre- heated - Components of greater than 70% copper shall not be used - Use leak detection spray - Cylinders and petroleum spirit shall be excluded from proximity of ignition sources - Create and maintain exclusion zone - Propane Gas shall not be used in tunnels | S | M | L |
|  | - Noise | - Ear protection e.g. ear defenders shall be worn - Site specific briefing shall be given - Noise exclusion zone shall be created and maintained - Maintain equipment to reduce noise sources - Information / signage shall be displayed on machinery depicting noise hazards and the need to wear ear protection |  |  |  |
|  | - Toxic Gases | - Health surveillance where work performed with lead (in line with the Control of Lead at Work Regulations) shall be undertaken - Provision of information on hazardous substances and control measures, including hazard of fumes when hot working shall be available - Personal protective equipment, including respiratory protective equipment shall be provided - Only trained and competent operators shall be used - Site specific assessment and briefing shall be completed - Exclusion zone shall be created and maintained - Positive air-flow system for galvanized and lead coated material shall be provided | S | M | L |
|  | - Failure of The Platform In Raised Position. | - At least one driver or operator shall remain on the ground when the platform is being used | S | M | L |
|  | - Burn Spatter | - Personal protective equipment, including eye / face and respiratory protective equipment, flame retardant clothing, boots and gauntlets shall be provided - Site specific assessment and briefing shall be undertaken. - Exclusion zone shall be created and maintained - Use trained and competent operators | S | M | L |
|  | - Working In Confined Space | - See AHA for Confined Space | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Safety Glasses - Hand Gloves - Full Body Safety Harness - Flame Retardant Clothing | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures - Fire Watch | - Daily - Weekly - When Required |  |  |  |