| Activity/Work Task: Paving and Asphalt Works | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1 Paving and Asphalt Works | 1.1 heat injury or illness | 1.1.1 Reduce exposure time, conduct survey early morning or late afternoon. 1.1.2 Avoid dehydration. Drink water frequently in small amount. 1.1.3 Wear light clothing. 1.1.4 Avoid contact with hot asphalt/hot equipment | S | M | L |
|  | 1.2 Heavy equipment including compactors | 1.2.1 Stay out of unauthorized/cordoned off areas 1.2.2 Maintain contact with banksman when in areas 1.2.3 Yield to equipment operators 1.2.4 Stay alert 1.2.5 never rest under heavy equipment 1.2.6 wear high visibility vest | S | M | L |
|  | 1.3 Heat Stress | 1.3.1 avoid working in direct sunlight when possible 1.3.2 work in cooler part of the day if possible 1.3.3 stay hydrated 1.3.4 take frequent breaks 1.3.5 self pace | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Hard Hat, Safety Glasses, Class 2 Reflective Vest & Safety footwear, gloves. | - Task AHA Development and Training Requirement - Safety Orientation | Daily check list 3rd party inspection certificate All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |