| Activity/Work Task: Rock Trenching and Tunneling | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Rock Trenching and Tunneling | - Exposure to moving equipment | - Training to work in areas where moving equipment is working - No loose clothing will be allowed in the area - Walking and working areas will be separated | S | M | L |
|  | - Fumes/ gas / dust | - Artificial ventilation will be provided to populate fresh air in the areas - Exhaust fans will be placed to suck out all the contaminated air - Respiratory protection will be provided for all the employees - No naked flames will be allowed in the area - Gas monitoring will be done and recorded frequently as required - Only trained and certified employee will be allowed to monitor the gas | S | M | L |
|  | - Lighting | - Adequate lighting will be provided according to standard requirements - Work will not commence unless adequate lighting is provided | S | M | L |
|  | - Flooding | - A pump will be available to pump out all the water in case of flooding - No employee will be allowed to enter the until the flood is cleared - All employees will have to evacuate the area if flooding begins | S | M | L |
|  | - Injury/ Death | - Adequate supervision will be available on site - All employees will have safety induction - First aid station and emergency phone numbers will be posted on site - Emergency procedure will be prepared and communicated to all employees | S | M | L |
|  | - Collapse | - Adequate support system that has been engineered by a qualified person will be used - Daily inspection of the area will be done and documented - An emergency evacuation plan must be established and communicated to all employees - A rescue plan will be in place | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Hard Hat, Safety Glasses, Class 2 Reflective Vest & Safety footwear, gloves, safety Harness. Respiratory Protection | - AHA training - Safety Orientation - Emergency procedures | - Daily - weekly |  |  |  |