| PROJECT HEALTH AND SAFETY PLAN |  |
| --- | --- |

TENDER NUMBER:

"DESCRIPTION"

| Revision nr. | Date | Nature of the revision |
| --- | --- | --- |

|  | Name | Date | Signature |
| --- | --- | --- | --- |
| Prepared by |  |  |  |
| Controlled by |  |  |  |
| Approved by |  |  |  |

| Issued to | Position | Date | Signature |
| --- | --- | --- | --- |

###### FRAMEWORK

The present document sets out to meet the requirements of the legislation in force relating to Safety, Hygiene and Health at Work.

The preventive and protection measures defined take into consideration the project definitions, the simultaneous or incompatible activities, the equipment, materials and products, the special risks, the building site project as well as the training and notification of the employees.

Our Health and Safety Management System (HSMS) is organized by way a set of procedures which we call Safety Management Procedures (SMP) which are associated with standard formats and Working Instructions (WI) which are to be implemented on the works so as to meet the legislation in force.

TABLE OF CONTENTS

###### STRUCTURE OF THE DEVELOPMENT OF THE HEALTH AND SAFETY PLAN

1.	SCOPE, OBJECTIVE AND DISTRIBUTION OF THE DEVELOPMENT OF THE HSP	6

1.1.	Scope	6

1.2.	Objective	6

1.3.	Distribution of the Development of the HSP	6

2.	ORGANISATION OF THE HEALTH AND SAFETY MANAGEMENT SYSTEM - HSMS	7

2.1.	Health and Safety Management System - HSMS	7

2.2.	Organization Chart of the Works contract and the Definition of Functions	7

2.3.	Health and Safety Policy	12

3.	COMMUNICATION MANAGEMENT	12

3.1.	Technical Compilation Management	12

3.2.	Management of Communication between all those involved in the Works contract	13

3.2.1.	IMPLEMENTING ENTITY I SUPERVISORS I CLIENT	13

3.2.2.	MEANS OF SUPPORT FOR THE CIRCULATION OF INFORMATION	15

4.	CONTROL OF SUBCONTRACTORS AND SUCCESSIVE SUBCONTRACTING CHAIN	15

4.1.	Restrictions on the selection of Subcontractors/free-lance workers, Suppliers of Materials and Work Equipment and their control	15

4.2.	Working Hours	18

4.3.	Civil Liability Insurance  of Accident at Work	18

5.	IMPLEMENTATION OF THE WORKS CONTRACT	19

5.1.	Works Schedule	19

5.2.	Men Power	19

5.3.	Equipment List	19

6.	FACILITIES OFFICES PROJECT	19

6.1.	Road Impact Plan	19

7.	CONTROL OF EQUIPMENT  PLAN	19

8.	PREVENTIVE RISCK AND MEASURES AVALIATION	21

8.1.1.	Definitions	21

8.1.2.	Identify the Hazard	21

8.1.3.	Identify the  Risks	22

8.1.4.	Risks Prevention Files	22

8.2.	Materials and Products with Special Risks or Material Safety Data and Sheet	22

8.3.	Monitoring of the Implementation of the Preventive Measures on the Site	23

9.	MEDICAL SUPPORT ( HIV CONTROL)	23

10.	PERSONAL PROTECTION EQUIPMENT	23

10.1.	Respiratory Protection	24

11.	COLLECTIVE PROTECTION PLAN	25

12.2.	Rigger’s	32

12.3.	Manual Loads Movements	33

13.	HOUSEKEEPING AND FIRE PREVENTION	34

14.	SCAFFOLD and SHORING ERECTION	36

15.	Mensory	37

16.	Demolition	37

17.	Painting	38

18.	ELECTRICAL INSPECTION AND ASSURE GROUNDING	39

21.	TRAINING	41

22.	ACCINDENT PLAN AND INCCIDENT RATE	45

23.	VISITORS’ PLAN	46

24.	EMERGENCY PLAN	47

26.	ALCOHOL  CONTROL	49

List of Appendices

Appendix nº

- Distribution of the Development of the Health and Safety Plan..................................................

- Organization Chart.......................................................................................................................

- Health and Safety Policy..............................................................................................................

- Working Hours.............................................................................................................................

- Civil Liability Insurance...............................................................................................................

- Work Schedule.............................................................................................................................

- Men Power...................................................................................................................................

- List of Equipment.........................................................................................................................

- Access Control on the facilities....................................................................................................

- Equipment Check List..................................................................................................................

- Hazards Risk Analyses per Activities Equipment and Control Factors Assessments...............

- MSDS.........................................................................................................................................

- Monitoring Measures..................................................................................................................

- Medical Support.........................................................................................................................

- PEE Plan.....................................................................................................................................

- Specific Lifting Procedure Cranes and Lifting Operation..........................................................

- Specific  Procedure for Scaffolds..............................................................................................

- Specific Procedure for Demolitions...........................................................................................

- Specific Procedure for Excavations...........................................................................................

- Traffic Control...........................................................................................................................

- Training......................................................................................................................................

- Accident and incident Rate........................................................................................................

- Visitors Plan...............................................................................................................................

- Emergency Plan.........................................................................................................................

- Monthly Report.........................................................................................................................

- Alcohol Control.........................................................................................................................

- SCOPE, OBJECTIVE AND DISTRIBUTION OF THE DEVELOPMENT OF THE HSP

As the HSP constitutes one of the basic tools of planning and organisation of safety at work, it has become necessary to carry out its development and its constant updating, these being parameters that Contractor a shall assume during the implementation of the present works contract.

## Scope

The present document applies to the activities to be undertaken by Contractor  l for the implementation of the works contract “DESCRIPTION”.

## Objective

This Plan seeks to ensure compliance with the stipulations of prevailing legislation and with the Safety Management System of Contractor.

The present document shall henceforth be referred to as the HSP (Health and Safety Plan). Upon approval by the Client, the HSP shall be available for consultation at the Works site, being permanently complemented and updated with the elements and records produced during the implementation of the works contract.

## Distribution of the Development of the HSP

As foreseen in the Safety Policy for the present works contract, the HSP shall be distributed to all the subcontractors and free-lance workers who sign a record of said distribution, declaring a commitment to its dissemination and compliance, as well as compliance with all the legislation in force as regards health, safety and hygiene at work.

For the purposes of disseminating the HSP to the main parties responsible for its implementation, this document shall be distributed, accompanied by the standard format "Documentary Evidence of Distribution of the Content of the HSP for the Implementation of the Works", a copy whereof Shell be found in Appendix 1

- ORGANISATION OF THE HEALTH AND SAFETY MANAGEMENT SYSTEM - HSMS

## Health and Safety Management System - HSMS

The HSMS submitted herein relates to the works in question, their specific aspects and characteristics. It intends to demonstrate the way in which safety management is to be carried out at the works site, not only including, the internal structure of Contractor, but also the relations to be established between the Client, the Supervisors, subcontractors, visitors and other external entities.

## Organization Chart of the Works contract and the Definition of Functions

The Organization Chart of the Works contract indicates the functional posts, the names of the employees associated with them and establishing the hierarchical and/or functional dependencies.

This Organization chart is approved by the Technical Manager of the Works contract and is updated whenever alterations are made to the organisational structure of the Implementing entity.

The Safety Manager of the Implementing entity functionally relates to the Technical Manager of the Works contract and the Representative at the works site of the Implementing entity.

The Organization chart of the Works contract, as well as the Definitions of Functions, shall be filed on appendix 2, with the chart being displayed in a clearly visible place, to wit on the notice board intended for information relating to safety at the Works Site.

Project Manager

It is the task of the Project Manager (PM) to carry out the following:

- Coordinate and supervise the execution of works and to represent the Subcontractor

- To coordinate and to lead the internal management and coordination team for the works and to over see the progress of the work carried out;

- To ensure the fulfillment of approved working programs;

- To promote the updating of programs, whenever it is necessary to ensure a due response to all requests made by the Works Owner and Monitoring Body;

- To promote the upkeep of an archive, which is continuously organized and updated withal l document s received and sent ;

To ensure technical control, and economic and financial balance of the contract, in terms of:

- Verifying that monthly situation report s are duly fulfilled in accordance with the terms of the Contract ;

- To promote the product ion of Hygiene, Health, Safety and Environment Plan for the works and to ensure its fulfilment;

- To promote the product ion of a Quality Plan and to ensure its fulfilment.

Schedule and Control Manager

It will be the task of the party in this post to carry out the following:

- To produce, in accordance with the guidelines established for these purposes by the PM and DP, the general programme of works , as well as remaining auxiliary Programmes which are duly detailed and integrated in order to give an insight into requirement s in terms of human resources , equipment , material s and finances ;

- Based on the previous paragraph, to produce the global financial schedule for the works and detailed programmes of procurement of materials, equipment and subcontracts, as well as of submittals, which are duly supported by the officials in charge, where and when these are necessary;

- To support the DP in studying and controlling work quantities, materials and labour , and producing charge sheets for each task;

- To carry out a weekly work programme development target , and based on these target s to warn the DP and PM in due time of any relevant delays that have been recorded, including the reasons for these delays , and the corrective measures which have been proposed and the corresponding consequences of these measures ;

- Based on these targets, to update each month the general works programme, the procurement programme, the financial schedule and any other detailed programmes required by the PM;

- Assist and present a toolbox talk to the workers;

To support the contract management department by means of preparing and supplying any details required for each given moment in order to support claims related to deadlines in an adequate manner.

Administrative Manager

- Ensure all administrative tasks of a development arising from the work of work in order to provide data to various areas of the company

- Differentiate the file of the work awarded, leaving it to the seat, giving knowledge to the Director of Work and address the correlation of the work which plays the role 
• In the client, has the mission to provide you access to the documents for this application, and run the monthly billing of the work in question, based on measurements taken by analyzing the time.

- In conjunction with the Director of Work, prepares an annual statement of costs in the sector, and this is the Director reported that the budgets of each contractor on an award;

- The control and distributions field shall go all times with documentation of the employee when is transferred.

Health and Safety Oficcer

- Assistant to the Project Management in the HHSW area (Hygiene, Health, Safety in the Works) ;

- Check the implementation level of the HES (using checklist s of activities , equipment and processes , whenever they exist ) and if justified propose its revision;

- Organise Enrolment Actions to make al l workers aware, before starting the job, of the specific risks of the Under taking and the preventive measures adopted, as well as the organisation of the safety measures implemented at the Construction Site;

- Implement “Enrolment Leaf let” from the moment the constructions site opened and keep it controlled and updated throughout the under taking;

- Divulge information about the HHSW to the worker it is aimed at ;

- Request and assess suggestions from worker s with regard to hygiene and safety issues and present them at the Safety Commission meetings ;

- Check the objective safety conditions of the Project in line with a pre-established list ;

- Draw up safety report s at a frequency appropriate for the Under taking;

- Control , Check and/or Approve the checklists with regard to Safety;

- Point out any non-conformity with the SHP and implement al l the necessary corrective actions ;

- Carry out Technical Audi t s to assess SHP implementation level.

- Analyse the Works Programme and collaborate in the programming and implementation of the Safety activities.

- Control the safety conditions of material s and equipment arriving at the Under taking;

- Check the supply and management of stocks of Collective Protective Equipment (CPE) and Personal Protective Equipment (PPE) ;

- Check the tidiness of the Company Premises ;

- Check the state and good-working conditions of the fire- fighting equipment ;

- Take corrective measures or organise their implementation in order to eliminate risks ;

- Co-ordinate the procedures to be taken in the event of a serious accident ;

- Monitor and support intervention in the Project, as par t of the HHSW, by the Developer.

QA / QC Manager

- Also reporting to the PM, itis the task of the official in this post to produce the Contractor ´ s, Quality Control Program, in accordance with the terms of the Contract , and to implement it (including all procedures parallel to its fulfilment ) as well as monitoring, setting up laboratories on site or of f site and supervising the corresponding work under the terms of the plan and as established in the Contract . Specifically:

- To adapt the Contractor ´ s Quality Control Program (CQCP) to the situation on site and to update it wherever necessary accompanying its implementation and development ,integrating the Quality Plans of subcontracted parties for this purpose;

- To be intermediary in contact s with the Works Owner in the field of quality matter s ;

- To coordinate remaining Quality Technicians in the Consortium, where applicable, and of the subcontracted parties ;

- To define with the DP a plan of inspect ions and tests (PI E) for activities considered to be critical , taking into account the demands of the general specifications ;

- To organise the files and al l technical documentation related to mate ial s and subcontracted works which confirm their adequacy, to be submitted to the contractor and to archive at a later date;

- To compile and archive the test records of the PI E;

- To analyse complaints , audit reports , carry out the recording / study of Non-Conformities and to propose corresponding corrective measures ;

- To promote the improvement of quality in the works.

Construction Manager

- Reporting to the Product ion Manager, its the task of the Engineering Department Manager (EDM) to accompany and coordinate al l areas of (a) works preparations (shop drawings),(b) planning and (c) preparation / control of “submittal s”. This party is expected to maintain a relationship of intense inter facing with the Monitoring Body and must manage and produce al l technical correspondence with the Monitoring Body /Works Owner for approval and signing by the PM. The EDM i s al so charged with;

- Assist and present a toolbox talk to the workers;

- Preparing any reports directly related to hi s/her areas of responsibility and which must periodically be discussed with the Monitoring Body;

- Preparing the monthly report for the works , in which, among other issues , the main questions to be discussed with the Monitoring Body must be listed, including pending technical problems , etc. and as produced together with other member s of the team, al l information relevant for the report must be included (planning targets, photographic report , submittals report , etc. ) .

Site Supervisors

- Be familiar with all Safety an Health procedures relation to the work being carried out under their supervisor;

- Assist and present a toolbox talk to the workers;

- Ensures that the workplace is maintained in a safe and tidy condition;

- Take prompt action to rectify unsafe acts and unsafe condition;

- Give full co-operation to the Site Safety Officer and Comply with Safety systems of Contractor;

- Ensure that all necessary safety devices and Personal Protective Equipment is provided and use. Ensure that all site personnel under their supervisor, is provided with and wear the required Personnel Protective Equipment and that all safety procedures pertaining to the works are complied with;

- Complete regular inspections of their workplace area;

- Be familiar with all the emergency and accident / incident reporting procedures, ensure that they are made know to their subordinates and implement those procedures as required;

- Promote safety awareness through leading by example.

Site Safety Officer

A site safety officer shall be implement the safety System of Contractor and with the cooperation of the project manager and superintendent to assurance the safety   of the construction and accomplish with the Devilment of the Safety Plan;

The site safety Officer is empowered to stop immediately any work, activity on the project and instruct and assist management or any sub-contractor in taking urgent action to make safe the Site and/or works to prevent unsafe working practices or other infringements of the “Health and Safety Management Plan” or statutory regulations.

The Safety officer shall compliance the flowing:

- Assistant to the Project Management in the HHSW area (Hygiene, Health, Safety in the Works) ;

- Check the implementation level of the SHP (using checklist s of activities , equipment and processes , whenever they exist ) and if justified propose its revision;

- Assist and present a toolbox talk to the workers;

- Organise Enrolment Actions to make al l workers aware, before starting the job, of the specific risks of the Under taking and the preventive measures adopted, as well as the organisation of the safety measures implemented at the Construction Site;

- Divulge information about the HHSW to the worker it is aimed at ;

- them at the Safety Commission meetings ;

- Check the objective safety conditions of the Project in line with a pre-established list ;

- Draw up safety report s at a frequency appropriate for the Under taking;

- Control , Check and/or Approve the checklists with regard to Safety;

- Point out any non-conformity with the SHP and implement al l the necessary corrective actions ;

- Analyse the Works Programme and collaborate in the programming and implementation of the Safety activities.

- Control the safety conditions of material s and equipment arriving at the Under taking;

- Guaranties all employees as the  Personal Protective Equipment (PPE) ;

- Check the tidiness of the Company Premises ;

- Check the state and good-working conditions of the fire- fighting equipment ;

## Health and Safety Policy

The Health and Safety at Work Policy for this Works contract shall be displayed on the building site notice board and is shown in Appendix 3.

- COMMUNICATION MANAGEMENT

## Technical Compilation Management

For drawing up the Technical Compilation of the Works, Contractor assumes the guarantee to submit the following documentation:

- Data relating to the implementing entity as well as subcontractors or free-lance workers whose interventions are relevant to the building characteristics which are the object of this works contract;

- Technical information relating to the general project and to the projects of the various specialist areas, including the design memoranda, the final design and record drawings which refer to the structural aspects, the technical networks and the systems and materials used which are relevant to the prevention of professional risks;

- Technical information relating to the equipment installed which are relevant to the prevention of risks deriving from its use, upkeep and maintenance;

- Information which is useful to the planning of health and safety in the carrying out of works at constructed work sites whose access and circulation entail risks.

## Management of Communication between all those involved in the Works contract

The methodology to be implemented for the management of the communication channels between the various parties involved in the implementation of the works has been summarised in the paragraphs below. With the implementation of this subchapter it is intended to achieve the following objectives:

- To identify the resources to ensure cooperation between the various parties involved in the implementation of the works, bearing in mind the health and safety requirements established;

- To define the information and communication management between all those involved at the building site as regards the prevention of professional risks.

All the protagonists are vital for a correct management and circulation of information, though we are highlighting the parties involved who, by dint of their functions and responsibilities, assume a vital role in the coordination and feasibility of the HSP and consequently of the HSMS of the works contract:

- Client;

- Prevention and Safety Division;

- Technical Manager of the Works contract;

- Representative of the Implementing entity;

- Prevention and Safety Technician;

- Foremen;

- Administrative Technician.

## IMPLEMENTING ENTITY I SUPERVISORS I CLIENT

The table below identifies the communication channels between the parties involved in the implementation of the works.

| RECEIVER ISSUER |  | COMMUNICATION CHANNELS |  |  |
| --- | --- | --- | --- | --- |
|  |  | Employer | Engineer | Contractor |
| COMMUNICATION CHANNELS | Employer |  | - Works Contract Coordination Meetings (Minutes) - Safety Coordination Meetings (Minutes) - Safety Committee (Minutes) - Letter / Fax | - Works Contract Coordination Meetings (Minutes) - Safety Coordination Meetings (Minutes) - Safety Committee (Minutes) - Letter / Fax |
|  | Engineer | - Works Contract Coordination Meetings (Minutes) - Safety Coordination Meetings (Minutes) - Safety Committee (Minutes) - Letter / Fax |  | - Works Contract Coordination Meetings (Minutes) - Safety Coordination Meetings (Minutes) - Safety Committee (Minutes) - Letter / Fax |
|  | Contractor | - Coordination Meetings - of Works contract (Minutes) - Safety Coordination Meetings (Minutes) - Safety Committee (Minutes) - Letter / Fax | - Works Contract Coordination Meetings (Minutes) - Safety Coordination Meetings (Minutes) - Safety Committee - (Minutes) - Letter / Fax - Works start information - Works Plan - Training and Information plans - Monthly Safety Plans with: Monitoring of Health and Safety at Work and other annexes: |  |

The Works Contract Coordination Meetings and the Safety Coordination Meetings shall be held on the dates and at the place to be arranged with the Client / Supervisors.

In addition to the elements indicated in the previous table above, the Safety Office of the Implementing entity shall always have available for the consultation of the Supervisors/ Client all the records produced during the implementation of the HSP, to wit:

- Records and summaries Identification of Hazards, risk assessment and controls definition

- Hazards/Risk analyses per activities/equipment and Control Factors Assessments’);

- of training and information actions;

- Companies’ procedures;

- Employees’ procedures;

- Equipment procedures;

- Summary files for product safety data.

The copies of these documents shall be supplied to the Supervisors whenever requested.

## MEANS OF SUPPORT FOR THE CIRCULATION OF INFORMATION

Information shall circulate via the following supports/means:

- Written - formal and informal by way of conventional means of communication, seeing to their registration and filing at the works site, bearing in mind that the electronic message is only regarded as an informal means of communication.

- Written/ Pictographic – by displaying posters, building site ground plans, pamphlets, convening notices, warnings, miscellaneous information.

- Verbal, gestual and sound – direct instructions, in the main on the works fronts

- CONTROL OF SUBCONTRACTORS AND SUCCESSIVE SUBCONTRACTING CHAIN

## Restrictions on the selection of Subcontractors/free-lance workers, Suppliers of Materials and Work Equipment and their control

The present chapter sets out to provide a brief description of the constraints on the selection of subcontractors, free-lance workers, suppliers of materials and work equipment in accordance with that stipulated in the Quality System implemented at Contractor and on the control of documentation deriving from the latter.

## SELECTION OF SUBCONTRACTORS AND FREE-LANCE WORKERS

When selecting subcontractors and free-lance workers for this works contract, the same principles were followed as are explained in the point regarding the selection of suppliers of materials and work equipment plus the following aspects, deemed as potentially restricting their performance in the works as regards prevention and safety:

- Technical Staff  on the Company Works site;

- Experience of the implementation of works of the same nature;

- Accident rates;

- Updating of the Equipment possessed.

Further to verification of these principles, a consultation is also carried out of awards already registered with Contractor, with a view to identifying any "bottlenecks" in the response capacity of the subcontractor which may bring about a possible reduction in the general performance levels and particularly as relates to prevention and safety.

## SELECTION OF SUPPLIERS OF MATERIALS AND WORK EQUIPMENT

In the consultation and selection of suppliers, priority is given to suppliers contained in the Book of Suppliers/ Providers of Services of the Quality System of Contractor, in other words, those suppliers that attain the quality levels required by the procedure relating to the qualification and assessment of suppliers and subcontractors.

This procedure of the Quality System of Contractor evaluates quality in terms of the speed of supply, an economic assessment, the response to the implementation requirements, term for completion and performance in the context of works safety and prevention.

## CONTROL OF THE DOCUMENTATION OF THE SUBCONTRACTING CHAIN

Contractor assumes its responsibility for selection and management of subcontractors, to this end implementing the procedures defined to this end which have been summarised in the table on the following pages:

| SUBJECT | PROVISION/METHODOLOGY |  | RESPONSIBILITY |  |
| --- | --- | --- | --- | --- |
| Contract | Existence of contract • At the very least the works shall possess a copy of the documentary evidence of the relationship established between the parties |  | Contractor |  |
| Accident at Work Insurance | Contractual clause |  | Contractor |  |
|  | Proof of up-to-date insurance - • Whosever fails to provide up-to-date proof of accident at work insurance is not allowed to work. Copy of the policy and the last receipt paid. |  | Administrative Services of the works • In the event of any difficulty in the provision by the Subcontractor of the respective documentary evidence, the timekeeper shall inform the works manager. |  |
| License | Contractual clause |  | Contractor |  |
|  | Evidence of the license - • Civil construction or temporary work license, as the case may be. |  | Administrative Services of the works • In the event of any difficulty in the provision by the Subcontractor of the respective documentary evidence, the timekeeper shall inform the works manager. |  |
| Medicine at Work | Contractual clause |  | Contractor |  |
|  | Evidence of the license - • Civil construction or temporary work license, as the case may be. |  | Administrative Services of the works • In the event of any difficulty in the provision by the Subcontractor of the respective documentary evidence, the timekeeper shall inform the works manager. |  |
|  | Medical examinations • When Contractor has accordance with the existence health delegation |  | - Administrative Services of the works |  |
| Normative control of employees | Contractual clause |  | Contractor |  |
|  | Identification documents • ID Card • Passport and contract of employment |  | Administrative Services of the works - Copy of the personal and professional identification elements of the employee. |  |
|  | Social security - • The Subcontractor shall provide monthly proof of the settlement of the Social Security of the employees on the works |  | Administrative Services of the works • In the event of any difficulty in the provision by the Subcontractor of the respective evidence, the timekeeper shall inform the works manager. |  |
|  | Staff’s daily work schedule • Up-to-date nominal list per employer, including all the staff on the works |  | Administrative Services of the works |  |
| Individual Protection Equipment | Contractual clause |  | Contractor |  |
|  | Distribution PPE • Should Subcontractors supply PPE to their employees, the requirement for the PPE Registration File remains valid. |  | Prevention and Safety Technician.. |  |
| Notification of accidents at work |  | Contractual clause |  | Contractor |
|  |  | Evidence of notification to the insurer • The employer shall notify the Occurrence of accidents to its insurer. |  | Administrative Services of the works • A copy of the notification by the employer of the employee suffering an accident to its insurer shall be obtained |
|  |  | Evidence of notification to the IGT (General Labour Inspectorate) where applicable • Notification to the IGTA is the responsibility of the employer of the employee suffering an accident |  | Contractor • In accordance with the methodology established in the HSP. |
|  |  | Notification to the Client, its representative or Safety Coordinator. |  | Contractor • In accordance with the methodology set out in the HSP. |
|  |  | Accident analysis report • Procedure in accordance with that defined in the HSMS. |  | Prevention and Safety Technician / Prevention and Safety Division |
| Blood Alcohol Level | Contractual clause |  | Works manager |  |
|  | Regulation • when approved with the IGT |  | Prevention and Safety Technician Prevention and Safety Division and Health Department |  |
| Training | Contractual clause |  | Contractor |  |
|  | Evidence of the actions undertaken - • Recording of attendance of training, information and awareness actions (identification and signature of the employee, the company and the action undertaken) |  | Contractor Prevention and Safety Technician |  |

In accordance with the procedure submitted, failure to submit all the aforementioned documentation, be it of the company or of its employees, shall be a constraint upon the carrying out of the works in the works contract.

Having confirmed the suitability of all this documentation, each employee shall be given an ID card, and any employee who fails to submit this ID card shall obviously be assumed to be in an illegal situation.

All the documentation relating to the companies and its employees shall be filed  shall be filled on the aquiver of administrated  of the construction.

## Working Hours

The working hours for the present works contract shall be filed on Appendix 4.

## Civil Liability Insurance  of Accident at Work

The policies and respective documentary proof of payment of insurance shall be filed for all those companies

Involve in the construction of this venture and Shell be filled  on Appendix 5.

- IMPLEMENTATION OF THE WORKS CONTRACT

## Works Schedule

The Works Plan shall be filed on Appendix 6 .

## Men Power

The Labour Schedule shall be filed on Appendix 7.

## Equipment List

The Equipment Schedule shall be filed on Appendix 8.

- FACILITIES OFFICES PROJECT

The building site project shall be drafted based on a surveying of the local conditions and constraints carried out in advance.

## Road Impact Plan

The site plan shall identify the accesses to the building site as well as the circulation routes on it.

The whole building site shall be signposted in accordance with that foreseen in the legislation in force. Shall be posted the speed limit for all areas and this speed limit will be placed according to the surrounding characteristics.

All the Traffic, road impact plans shall  filed on Appendix 9.

- CONTROL OF EQUIPMENT  PLAN

The control of equipment shall be considered in three situations:

- A documental verification prior to the entrance of the equipment on the works carried out by the Plant Division  and by the Prevention and Safety Technician;

- Periodic verification (fortnightly for the implementation of the control of support equipment),

- Extraordinary verification (whenever the equipment on the works is subject to transformations, structural repairs, if state of conservation is suspect, or also after an accident).

The equipment to be used on the constructions shall always be accompanied by the following documents (in addition to the aforementioned legal requirements):

- Insurance policy for the equipment in question;

- Service book/ proof of last service (in accordance with the maintenance manual)

- Instructions manual;

- Credentials of the operator.

Motor-driven equipment to be used on the  site and which may circulate on the public Roads shall always be accompanied by the following documents (in addition to the aforementioned legal requirements):

- Registration document and/or document containing similar information, legally accepted for this purpose;

- Property registration and/or document containing similar information, legally accepted for this purpose;

- Insurance policy for the equipment in question;

- Service book / proof of last service (in accordance with the maintenance manual);

- Instructions manual;

- Credentials of the operator.

Also complying with the following:

- They shall be endowed with lighting and regulatory signposting systems;

- They shall be endowed with other lighting systems for any night-time work;

- They shall be endowed with sound warning and signposting systems (reverse lights and warning sound, yellow rotating bulb and a red triangular sign, with reflecting material placed at the back);

- They Shall meet which is regulated in the highway-cod.

Drivers / operators of mobile equipment:

The drivers / operators of mobile equipment must be endowed with experience and qualifications which are appropriate for discharging their respective duties and the employer shall issue a statement attesting to said qualifications. The equipment shall be operated by specialised operators who are duly informed about:

- The equipment usage conditions;

- Anomalous situations;

- Foreseeable situations;

- Conclusion to be drawn from prior experience of using said equipment;

- Minimum safety conditions;

- The responsibility of each operator to strive for the smooth operation of the equipment which he operates and notify as regards any anomaly he detects.

For all equipment an identical procedure shall be followed as regards the entrance onto the works and its use. Hence, all equipment must meet the requirements of the procedure involving the "Verification for authorisation of entrance of the equipment onto the works" of Contractor whose registration model shell be in Appendix 10 .

## PREVENTIVE RISCK AND MEASURES AVALIATION

## Definitions

- Hazard: Anything (e.g. condition, situation, practice, behaviour) that has the potential to cause harm, including injury, disease, death, environmental or property and equipment damage.

- Hazard Identification: This is the process of examining each work area and work task for the purpose of identifying all the hazards which are “inherent in the job”. Work areas include but are not limited to machine everything that involves the implementation of an activity.

- Risk: The likelihood or probability that a hazardous event (with a given outcome or consequence) will occur.

- Risk Assessment: Is defined as the process of assessing the risks associated with each of the hazards identified so that appropriate control measures can be implemented based on the probability, likelihood that harm, injury or ill health may occur and how severe the consequences of exposure might be.

- Risk Control: This is the process of identifying and implementing the most cost effective risk control measures having regard to the Hierarchy of Control Principle, legislative provisions and other relevant information.

- Monitoring and Review: This involves ongoing monitoring of the hazards identified, risk assessment and risk control processes and reviewing them to make sure they are working effectively.

## Identify the Hazard

Contractor have a internal Health and safety control and requires the identify all potentially hazardous situations which could result in any worker in the workplace being harmed. The hazard identification process requires that:

- Past incidents/accidents be examined to see what happened and whether the incident/accident could happen again.

- Work areas or work sites be examined to find out what is happening now.

- Information about equipment and Material Safety Data Sheets be reviewed to see what is said about safety precautions.

## Identify the  Risks

This should include:

- Identify factors that may be contributing to the risk,

- Review health and safety information that is reasonably available from an authoritative source and is relevant to the particular hazard,

- Evaluate the likelihood of an injury occurring and the likely severity of an injury or illness that may occur,

- Identify the actions necessary to eliminate or control the risk; and

- Identify records that it is necessary to keep to ensure that the risks are eliminate or controlled.

## Risks Prevention Files

In accordance with the Safety Management System of Contractor , after the drafting of the files for the Hazards/Risck Analyses per Activities/equipment and Controls Factors Assessments  should be filled on Appendix 11.

## Materials and Products with Special Risks or Material Safety Data and Sheet

Worthy of particular attention are materials / products with special risks which, by dint of their chemical and/or physical characteristics, such as toxicity, irritation, corrosion etc, including its form of handling and storage.

For each of the hazardous chemicals, the works shall have a "Safety Material Data Sheet" supplied by the respective supplier and this files are the same of MSD’S. Contractor will apply to all suppliers’ technical files and Safety of chemical material before starting work with the objective to eliminate all risks to which workers are exposing. The information  of the Materials Safety Data Sheet should be available in Portuguese and English for all hazards substances in storage and in use and should be provided to the workers.

When storing materials and products, the hygiene, safety and prevention rules shall be complied with, to wit:

- Storage of parts bearing in mind its volume, weight and dimensions in piles of no higher than 2 meters which shall be stable and also bearing in mind the time of its application and/or use on the works;

- Definition of and compliance with the accesses of mechanical means of moving loads;

- Definition of and compliance with incompatibility owing to chemical characteristics and risk level;

- Aeration and thermal conditions;

- Protection of humidifies and atmospheric conditions;

- Arrangement, cleaning and removal of waste.

The Safety Data Summary should be filled on Appendix 12.

## Monitoring of the Implementation of the Preventive Measures on the Site

In Appendix 13 to this document we will provide the standard format for Non-Compliance for the monitoring of the application of the safety measures on the work fronts.

- MEDICAL SUPPORT

The methodology which shall be implemented to Contractor for all the employees regarding the following situations:

- To prevent the professional diseases;

- Ensure a good health on all the employees;

- Ensure that the category is adequate for the condition of the employee;

- Sensitize all employees and community for the HIV ;

- To define the process relating to the employment and identification of the employees of the Implementing entity and of its subcontracting chain, to wit the employees of the Subcontractors, the Temporary Employees and free-lance workers;

- To define the process relating to the control of the employees on the works contract,

- To define the process relating to the checking of the medical fitness of employees, bearing in mind the tasks which they are carrying out;

- To define all the sanitary measures to be implemented on the works.

- The identification and control data as regards medical capacity shall be entered for all employees at the Works site which we shall file on Appendix 14.

- PERSONAL PROTECTION EQUIPMENT

The methodology implemented in the identification of the Individual Protection Equipment by professional category and in its use has the following objectives:

- To identify all the risks, by professional category, which restrict the choice of the PPE to be used by the employees of the present works contract;

- To distinguish the PPE of compulsory use and those of temporary use;

- To make employees responsible for the correct use and maintenance of the state of conservation of its PPE;

- To establish a methodology which allows the control of the distribution of IPE to employees in line with the usage  conditions of these equipment, to wit as regards its duration, bearing in mind the seriousness and frequency of exposure to risk, the characteristics of the work posts of each employee, and the performance of the safety equipment;

- The control and distributions field shall go all times with documentation of the employee when is transferred.

To establish the control methodology for the effective use of PPE by employees on the various works fronts.

Upon admission, the equipment required for their activity/task is submitted to each employee, and/or said submission is verified, shall fill the standard document in Appendix 15. This filled will have the specifics PPE for several category but the minimum standards are:

For the works contract in question the following PPE must be used by all participants and visitors:

- Helmet;

- Safety Boots;

- Reflective clothing;

- Work Clothing.

As the activity and the situation should use the following PPE:

- Safety glasses;

- Safety Gloves.

- Ear plugs;

- Welding mask/goggles;

- Physical filter masks;

- Raincoat;

- Safety harnesses;

- Safety glasses;

- Autonomous breathing Equipment.

## Respiratory Protection

Contractor will only use the following masks

Quarter Mask — With or Without Filtering Face Piece

• Removes particles only;

• Also known as a disposable respirator, dust mask and single-use respirator;

• Should be discarded after one use;

• There are many different types and sizes.

Half Mask – Painters

• Removes dust and vapor;

• There are many different sizes and types.

Storage

• Respirators must be stored in a clean dry place in a tight container or a sealed plastic bag when not in use

• Respirators should be kept in a convenient location close to where they are to be used

• Respirators must be protected from dust, sunlight, extreme heat and cold, moisture, chemicals and physical damage

• Do not store respirators unprotected in your work area.

- COLLECTIVE PROTECTION PLAN

The methodology implemented in the identification of the collective protection measures which are to be adopted and in the use and control of the Collective Protection equipment, sets out to identify all the needs in this regard which have been adopted in line with the construction methods and processes used, their associated special risks and local constraints.

## COLLECTIVE PROTECTION TYPES AND MATERIALS

1. Orange Polyethylene Nets, placed around the whole perimeter of the excavation zones with a vertical face lower than 1.20m; and on the delimitation of the roads and pathways, outside and within the building site.

2. Hard Barricades at 3 levels placed on ground dips/rises throughout the perimeter of excavations of depths greater than 1.20 m and throughout the perimeter of the slabs during the implementation of the structures. Shall be use also on floor openings and use a safety sing opened floor.

3. Barricades Types – This barricades are use to sign a danger zone or sing a caution. Shall be use a white and red type for danger with a danger safety sign.

3. Lifelines associated with safety straps on all works entailing the risk of a fall from a height, either because it is impossible to assemble more effective collective protection, or at the time of assembly of the collective protection (body rail), it shall always be deemed necessary to protect the employees carrying out this task.

4. Rebar Caps and Mushroom Caps placed on all the starter reinforcements which may cause risks of perforation or cutting.

5. Inclination of the excavation slopes in accordance with that defined in the final design, or with the type of ground forming slopes with the natural inclination, or deploying containments / propping.

6. Sufficient lighting depending on the workplace and type.

7. Formwork equipment endowed with accessories which include collective protection from falls from a height

- LIFTING AND RIGGING OPERATION

## General Requirements

## Planning Lifting Activities

# All lifting activities shall be planned in accordance with the specific requirements of their category:

- Medium lifts( ≥  6; 25 Tons ≤ ) do not required a formal lifting plan or calculations. However, the competent person shall be available to review light lifting activities and work shall be performed by qualified crane operator and qualified rigger using safe work practices;

- Heavy haul operations, heavy and critical lifts ( ≥ 25 Ton) require a detailed lifting specific safety procedure PES for the Lifting , with drawing and calculations. The operations shall be carried under supervision, using qualified crane operator rigger.

## Movement of loads by mechanical means

The present procedure relates to the prevention of the risk of works involving the mechanical movement of loads using specific equipment, and the safety procedures to be borne in mind when carrying out the respective slings.

This procedure sets out to implement the implementation of measures to prevent the risk of materials falling during mechanical movement. In case of doubt, the employees who carry out the slings shall request their superiors for such clarifications as they see fit.

The main points to be observed with a view to reducing the risk of materials falling, trapping and/or crushing at the time of moving loads by mechanical means of raising are:

- Correct positioning of the loads (centre of mass) with regard to the points whereby the slings are carried out so that they maintain balance during the movement;

- Correct implementation of the sling itself which shall always have at least two fixing points to the load.

On days when the wind is more intense, the securing of the loads shall always be distributed in such a way that the latter is transported in stable fashion with the greatest possible distribution of weight at the chains, in order that:

- It doesn’t sway too much during the raising, moving and lowering of loads, thereby reducing the risk of the loss of control and balance of said load which shall cause a necessary increase in the danger of clashing with materials whose consequences may impact not only in terms of work accidents (and hence with personal injuries), but also possible damages to the structure and/or work equipment;

- In the event of a chain falling, the others can support the load, at least for such time as is necessary for said load to be lowered immediately.

This will only be possible if, every time a load is moved by mechanical means, the largest number of suspension points possible are placed, evenly shared amongst the material to be transported.

For this reason, only slings positioned at 2 points, at least, shall be allowed on these works, and whenever the load so requires, regardless of the atmospheric conditions slings at 4 points.

Generally speaking, the equipment slings on site shall, whenever possible, be carried out at the points intended for this purpose and, in the event of materials, specific devices shall be used for their arrangement and transport.

A list has been drawn up below – which is not exhaustive – of the most appropriate forms for carrying out the slings of materials and equipment in the works, some of which have been illustrated, with correct and incorrect examples for easier understanding.

| Equipment |  |  |  |
| --- | --- | --- | --- |
| - Generators/Compressors This equipment has its own rings for transport by crane. In cases where the equipment only has one ring, 2 hooks shall always be secured to the ring so as to increase the safety level. |  |  |  |
| - Concrete Buckets The concrete buckets are endowed, in the wing structure, with a triangular ring intended for their suitable securing for transport with the aid of mechanical means of moving heavy loads, to wit the cranes. |  |  |  |
| - Scaffolding, Gangways and Ladders The scaffolding, metallic gangways and ladders are moved by way of slings at 4 points surrounding the base elements and/or secured to the suspension rings to be found at this equipment. |  |  |  |
| - Container Portable Toilet The Container Portable Toilets are transported to the centre of the site or exterior track for cleaning by way of a sling at 4 points on rings placed on top of its structure for this purpose |  |  |  |
| - Scaffolding The scaffolding to be found on the works is made up of a wholly metallic structure and has net protection with a sufficient height to prevent the fall of any materials transported. They are endowed with 4 rings intended for transport by crane but may also operate coupled to equipment of the multipurpose type. |  |  |  |
| - Transport of waste These buckets shall be adapted to the transport conditions by crane by way of welded straps and handles, for example steel bar in such a way that it does not give to the breaking of the side walls. Shoring posts / Tubular Guard Rails / Posts for Safety / Scaffolding Material, Gangways and Ladders Since this material slips when wrapped in the chains, it shall always be transported using appropriate pallets. . |  |  |  |  |
| - Woods The wood shall be placed on simple or dual piles with 90° rotation between courses and duly strapped. It shall then be wrapped in the crane chains and transported to the place for which they are intended. |  |  |  |  |
| - Steel Bars The steel bars shall be transported being duly tied, though never with the tying being directly to them wherever possible, a specific beam to this and shall be used or, failing that, surround, at least at 3 points, all of the chain in such a way that the load is balanced. In cases of bars of a different length, their arrangement shall be borne in mind in such away that the center of mass of the whole is situated more or less in the middle. |  |  |  |  |
| - Reinforcements The reinforcements shall be transported horizontally using the sling at 2 points at least. In the case of elements with bars of a small diameter, the chains shall be wrapped around the load. In other cases the hooks may be directly secured to the reinforcements. |  |  |  |  |
| - Clamps The clamps are secured by means of bars with a small diameter and they shall never be moved with the hooks placed at these ties. The most appropriate way of transporting this material is the placement of the chains, 2 at least, accompanying the tie, thereby surrounding the whole bunch of clamps. |  |  |  |  |
| - Formwork panels The formwork panels shall be transported with the aid of at least 2 raising hooks. In the case of panels with large dimensions, 4 raising hooks shall be placed where the hooks of the chains shall then be placed. |  |  |  |  |
| - Accessories for formwork / Hand tool These accessories shall be transported with the deployment of metallic baskets or a similar device which shall have 4 suspension points. |  |  |  |  |

## Maintenance, examination and Testing Equipment

# During The course of the project, continuous inspections and maintenance are essential to the safe and reliable operation and lifting operation.

## Rigger’s

# The responsibilities of a rigger not only include using various rigging components, but also ensuring that the load is safe to lift before it leaves the ground.

# If, the rigger for any reason, thinks the  lift is unsafe, he must refuse to make the lift until safe conditions have been assured.

# A qualified rigger, are responsible for all lift types including:

- Preparing a load for safe movement

- Assisting in the movement or directing the movement of a load

- Using various hoisting and load movement equipment safely

- Select components used to assist crane, hoist, or other equipment

- Ensure information, procedures, and equipment necessary to move loads without damage or injury are available.

- Make sure substandard or unsafe equipment or methods are immediately reported to supervision & removed from service.

- As a qualified Rigger, has to work closely with the Crane Operator. Both parties have overlapping areas of responsibility and, therefore, must communicate to ensure that each lift is performed safely.

- Lifts should all have one thing in common—PLANNING. It is extremely important that all lifts are preplanned by rigger and the Crane Operator.

# To qualify the riggers Contractor gives specific internal training.

The specific procedure cranes and Lifting Operations if required shall be filled on Appendix 16.

## Manual Loads Movements

Manual handling

- Back injury potential.

- Musculoskeletal injuries;

Controls

- For questions about any work or operation instructions to the task request;

- Where it is not possible to avoid manual handling, should be taken measures to alleviate the hard work and avoid the risks, including the journeys made with loads should be as short as possible;

- Must observe the characteristics of the cargo before its elevation (volume, center of gravity, weight, distribution of weight, shape, etc ...)

- Should position itself with the load, placing one foot behind the other side and charge the same, in order to keep the body balanced;

- Must use the strength of the legs because these muscles are the strongest of the body. This should lower it bends the knee without sit in heels

- For placing the material on the floor the workers should have the same posture do the same way.

- Use two hands instead of one for a task --to reduce excess demand on a single muscle group.

- Use tools that are right for the job and proportioned for your body.

- Use power tools instead of manual tools when possible.

- Take frequent breaks from repetitive motion tasks.

- Avoid repeating awkward movements or holding yourself in awkward positions.

- Wear protective gloves that reduce pressure or tool vibration on your fingers. .

- Change positions, stretch often to improve blood circulation, and take breaks regularly.

- HOUSEKEEPING AND FIRE PREVENTION

## Housekeeping

Housekeeping is the routine care and cleaning that needs to be acted on daily in order for a production facility to function safety and properly. Injuries and illnesses in the construction settings are common because the housekeeping of the construction has become slack, or an excessive amount or workers has been allowed to accumulate and causes accidents or injuries because of the falls, the workers will clean the workplace during and in the final of the activities.

The areas of particular concern are:

General:

- Boards with nails must be kept clear of

- Work areas

- Passage ways

- Stairs

- In and around the building.

General:

- Oily trash and rags must be:

- Put in covered containers

- Containers must be removed from site daily.

Ladders and Stairways

- Debris and material

- Must be kept away from the top and bottoms of ladders and stairs.

Scaffolds

- Excess materials and debris

- Cannot build up on scaffolds

- Only one day’s supply of material

- Daily clean-up of waste.

## FIRE PREVENTION

Fire extinguishers are designed to prevent relatively minor incidents becoming major conflagrations, the types of fire extinguishers are:

- Flammable liquid/gasoline(Class B);

- Flammable gases (Class C) ;

- Freely burning materials (Class A)

- Electrical equipment ( Type CO2)

The Contractor will use the ABC extinguishers and CO2 depending of the activity in question and the type of material to extinguish, taking into account the following features:

## Sitting of extinguishers

- Extinguishers should be located in conspicuous positions, available at all times for immediate use and fitted on brackets or stands where they will be readily seen by persons following an escape route.

- Fire extinguishers should be securely hung on wall brackets. Where this is impractical extinguisher should be located on suitable stands (not on the floor).

- If wall mounted the carrying handle of larger, heavier extinguishers should be 1 meter from the floor but smaller extinguishers should be mounted so the carrying handle is 1.5 meters from the floor.

- Extinguishers should be sited in such a way that it is not necessary to travel more than 30 meters from the site of a fire to reach an extinguisher.

- To avoid confusion, all extinguishers installed in any one building or single occupancy should have the same method of operation and if intended for the same function should be similar in shape, appearance and color.

- Wherever possible, portable extinguishers should be grouped to form a fire point.

## Maintenance

- Extinguishers should be routinely inspected to make sure that appliances are in their proper position and have not been discharged or lost pressure;

- The user should replace extinguishers not available for use by serviceable extinguishers;

- Monthly the fire extinguisher shell be inspect and the responsible person shell be sign on the tag;

- Annual inspection, service and test discharging should be carried out by a competent person and company

- SCAFFOLD and SHORING ERECTION

## The scaffolding has the following features:

- Supported scaffolds consist of one or more platforms supported by outrigger beams, brackets, poles, legs, uprights, post, frames, or similar rigid support.

- Guardrails or personal fall arrest systems for fall prevention/protection are required for Works on platforms.

- Working platforms/decks must be planked close to the guardrails.

- Planks are to be overlapped on a support the weight that is need it  ;

- Legs, post, frames, poles, and uprights must be on base plants and mud sills, or a firm foundation, and be plumb and braced;

- When the scaffold erection are complete a competent person will circumscribe a statement committing itself that is well mounted and/o risk well mounted as the Project.

## Scaffold user training must include:

- Maximum intended load and capacity;

- Recognizing and reporting defects;

- Fall hazards;

- Electrical hazards including overhead lines;

- Falling object hazards;

- Other hazards that may be encountered.

Inspect scaffolds and scaffold parts daily, before each work shift and after any event that may have caused damage:

- Verify  that the scaffolds is the correct  type for the loads, materials, employees, and weather conditions;

- Check footings to see if they are level, sound, rigid, and capable of supporting the loaded scaffold;

- Check legs, post, frames and uprights to see if they are on base plates and mudsills;

- Check metal components for bends, cracks, holes, rust, welding splatter, pits, broken welds, and non-compatible parts;

- Check for save access. It’s not allowed use cross braces as a ladder for access or exit.

Contractor has the specific safety procedure for assembly, use and disassembly of scaffolds and Shoring shall be filled on the Appendix 17.

- Mensory

- Must be assured that the platforms for unloading material should be  disposal on solid ground and  stability ;

- The wood pallets has to be moved by mechanical means and  near from the places where the employees will be use;

- Do not overload pallets and do not put yourself on line of fire, overhead loads;

- Inspection the wood pallets before use and before moved them;

- The blokes has to be secured with a strap, before moved the load;

- To prohibit any work near from exterior walls built recently if necessary should be provide work planned in order to analyze the necessary means to do the work safely;

- Working platforms shall be provided with guardrails and toeboards ;

- The debris should be deposited in a specific location;

- The block can‘t be use to support platforms should always be based on beams or joists in good condition
- Prohibit the improvisation work platforms;

- Is required a daily housekeeping to avoid the accumulation of debris;

- The placement of the pallets with the blocks  we must take in consideration where to place because de area must support beyond the weight of the pallets with the blocks;

- Do not overload the wood pallets;

- The forks of the metallic pellet fork can’t be in the middle of woof pallet otherwise will be unbalanced;

- When placing the straps around blocks should take measures for the sharp edges. Must be protective ;

- Inspected the wood pallets every time you need load on the metallic pallet fork;

- If necessary erect a platform parallel of the scaffold to put the blocks.

- Demolition

Before the start of any demolition project, Contractor shall have in consideration the following guidelines for this activity:

- Planning for a demolition project is as important as actually doing the work;

- A competent person experienced in all phases of the demolition should conduct the demolition planning.;

- Planning should involve the entire demolition operation including methods to be used to bring the structure down, necessary equipment to do the job, and measures to be taken to perform the job safely.

- Prior to the start of demolition, an engineering survey must be completed to assess the condition of the framing, floors, and walls to prevent a possible premature collapse of the structure.

- The demolition contractor is responsible for planning the wreckage of the structure, the equipment to do the work, informing worker of hazards and safety requirements, and public safety.

- Planning should include necessary safety equipment such as specific respirators, hearing protection, safety nets, lifelines, fall protection, warning signs, eye and face protection, and any other hazard protection device needed for the job.

- The engineering survey should determine if there are any chemicals, gases, explosives or flammable materials previously used or stored at the work site, which may still present a hazard. Examples include asbestos containing insulation or lead-based paint used in the initial construction.  Then before demolition begins, electric, gas, sewer, water, steam, and overhead lines etc. must be located and shut off, capped or controlled. If it’s necessary to maintain some of the services, temporary relocation should be performed and all workers notified of the new locations to avoid accidents.

- The demolition area should be clearly marked as such to ensure that only authorized personnel are allowed within restricted areas of the site. All site workers or authorized personnel should be dressed in appropriate personal protective wear and be informed of safety practices and emergency procedures.

Contractor has the specific safety procedure for demolitions shall be filled on the Appendix 18.

- Painting

Before the start of any demolition project, Contractor shall have in consideration the following guidelines for this activity:

- place all empty paint cans in waste bin only after they've dried;

- replace and secure lid on any can that still has wet paint inside it

- clearly identify/label cans that have left-over paints

- do not leave or store paint solvents in paint cans;

- ensure cans of unused paint have lids tightly closed before storing;

- wipe excess paint from can lid and can rim after opening;

- cover cans of paint with a rag when closing a lid to prevent paint splattering;

- never carry solvent soaked rags in pockets or leave bunched up or in an unventilated area;

- read safety MSD on manufacturer paint cans;

- use Stepladders only to do small or temporary works, can’t use like a platforms work;

- clean all tools using appropriate solvent;

- regroup tools, paint and equipment at end of each day;

- solvent used to loosen frigid brushes overnight should be kept isolated in areas away from flammable products;

- large container to hold excess or spent material should be used when spinning brushes and rollers;

- Only use paints that are recommended for indoor use.

- Do not use exterior paints inside.

- Hazards vary from product to product, so don't assume you know how to use the paint.

- Open windows, doors, and vents. Make sure there is adequate cross-ventilation and use a fan blowing out from a window to help draw fumes away from the inside.

- Once the job is finished, keeps the space open, and well ventilated until the paint is thoroughly dry.

- Clean up according to the directions on the paint.

- Never use gasoline to clean brushes. If you are using a flammable product designed to clean alkyd paints, be sure there is nothing that can ignite around the clean up area.

- Analyse the MSDS for proper storage, how to proceed in an emergency situation and specific PPE should be used.

- Use flammable sings if applicable.

- ELECTRICAL INSPECTION AND ASSURE GROUNDING

To prepare electrical installation Contractor  will have a competent person and will be delivered a statement of responsibility to ensure it is properly installed.

## The Contractor  will focus on the following character:

- Metallic masses of energized apparatus and masses likely to became charged should be grounded and be equipotential to each other. Distribution meters placed upstream from users shall be protected by 30A differential circuit breaker and power circuit breakers appropriate for the expected consumption of each terminal.

- An exception to this is the connection to heavy equipment equipped with its own protection device that guarantees the triggering off of the leakage current.

- All electricity meters including removable site meters should be closed, signposted equipped with 30mA  circuit breakers and grounded.

- Management should immediately be informed of all irregularities detected in any installation and/or electrical apparatus so that the necessary corrections may be made.

- All maintenance and repairs should only be carried out by specialist;

- The electrical specialist should use protective gloves and boots;

- The CO2 extinguisher will be placed near electrical switchboards.

- EXCAVATION

Contractor  will not start any excavation without  the excavations Permit, the excavation shall have more them 1,20m. The excavation Permit it’s a documents with all the information about the excavations completed by the construction manager with the support of the safety manager or officer

Spoil material shall be kept at least 1m away from the excavation edge and where trenches has to have access. All the excavation shall be barricade with a white and red type for danger and for caution yellow type. If necessary Contractor will use hard barricade and put warning signs.

Before beginning the execution of excavation should take into consideration the following situations:

- Underground installations;

- Access and egress;

- Exposure to vehicular traffic;

- Exposure to falling loads;

- Warning system for mobile equipment;

- Hazards atmospheres;

- Protection from hazards associated with water accumulation;

- Stability of adjacent structures;

- Protection of employees from loose rock or soil ;

- Inspections ;

- Walkways shall be provide;

- Requirements for protective systems.

All the documentations of  excavation shall  filled on Appendix 19.

- TRAFICC CONTROL

The Safety Department shall do specific traffic control wherever need on the contractions having on considerations the item on the contract and following requisites:

- Prior to the beginning of work operations, evaluate all aspects of the work area, including sight distance, traffic speed, volume, road approaches, work duration, and the type of work activity, before deciding on a traffic control plan

- Whenever the temporary traffic control zone is regulate on the D.L Nº51/2009 de 29 de September, Mozambique low..

- Traffic control devices are used to visually guide drivers through work zones. Signing, channelizing devices, arrow panels, and warning beacons all provide a message to the driver. Work zone credibility is established through the proper use of these devices to send correct messages to drivers. Poor work zone credibility has a direct, negative impact on work zone safety by causing driver confusion, frustration, and disrespect.

Contractor has the specific safety procedure for traffic Control shall be filled on the Appendix 20

- TRAINING

This works contract shall ensure the training of and information to employees, bearing they duties they discharge, the workpost they hold and the risks to which they are exposed.

The Training and Information plan of the employees envisages:

- Before enter to site the shall be given to the all employees all the  information actions about the general health, hygiene and safety rules of the building site ( safety orientations training );

- To provide specific safety training to employees who carry out tasks involving specific risks;

- To promote raising awareness actions of a general nature for all the employees on the works;

- To promote periodic meetings by groups of employees;

- To promote raising awareness actions when there is observed to a breach of the prevention and safety standards and procedures.

These training and information actions shall also be carried out on the occasion:

- Of a transfer or change in functions;

- Of a change in working equipment; or,

- Of the introduction of a new technology.

The training shall be in accordance with the Works Plan and obviously the emergence of new activities adapted to the evolution of the risks or the appearance of new risks, being periodically renewed. It shall take place within working hours.

To provide all the employees entering the works with information actions about the general safety, hygiene and health rules of the building site:

Whenever a new worker or employee is employed, they shall be given the necessary instructions about the health, hygiene and safety rules of the site (Full orientation). To complement these initial instructions, the worker shall be provided with a brochure containing the main prevention rules, as well as a pamphlet which shall always accompany the employee. This procedure shall be ensured by the Prevention and Safety Technician.

The cleaning of the building site and working areas is the direct responsibility of the employees who act therein, under the supervision of the foremen and Prevention and Safety Technician. In Welcoming Actions reference is also made to this principle, stressing the importance of cleaning, not only for maintenance of the conditions hygiene and cleanliness of the building site, as well as for those cases of falls at the same level and from a different level, since when the building site is clean both the likelihood of occurrence of this type of incident/ accident and the consequence in the event of its occurrence is drastically reduced.

To provide specific safety training to the employees who carry out tasks with specific risks:

Prior to the start of activities with special risks, training/raising awareness actions shall be promoted

with the presence of all the workers and superiors involved therein, thereby ensuring information about the risks inherent in said tasks, as well as the measures to be implemented to mitigate/annul them.

Toolboxes meeting as to be given to all the employees before start the worked once a week..

The Hazards/Risk Analyses per Activities/Equipment and Control Factors Assessments’ the safety Department with the cooperation for the supervisor shall give to the employees before starting the activity and must be review once a month by the supervisor.

To promote raising awareness actions of a general nature for all the employees on the works: Training/raising awareness/information actions shall be periodically promoted with the presence of all the workers on the works and Works Management, thereby ensuring the raising of awareness for practising prevention and safety, hygiene and health at workplaces.

To promote periodic meetings by groups of employees:

Alongside the training/raising awareness/information actions of a general nature, actions by professional groups shall also be promoted, stressing the risks inherent in the carrying out of the tasks which form part of their professional category.

To promote raising awareness actions when there is a breach of prevention and safety standards and procedures:

In the event of the occurrence on the works of any breach of the standards, rules of good practice and safety and prevention procedures, there shall be an immediate stoppage of the works and the Prevention and Safety Technician shall administer a training/raising awareness/information action as regards this breach and the implementation of the measures stipulated and required to rectify this breach, as well as in terms of the other risks inherent in the task. Only after undertaking this action can the necessary measures be implemented and the work continue.

All planning of raising awareness and training actions which are to be undertaken are based on the methodology developed in the context of HSMS which goes from the survey of specific needs of the works to the drafting of the training plans and consequent promotion of the respective actions.

As a rule, these actions take place over a short space of time, using a simple, objective language, as well as deploying audiovisuals and other appropriate teaching instruments. Due consideration shall be given to the possibility of the existence of emigrant employees and respective languages, ensuring that the latter encompass and assimilate the contents of the action.

The records of the actions undertaken on the works shall be drawn up in accordance with the internal procedures of Contractor -  as regards the themes, attendance records and summaries.

The trainers shall be the:

- Prevention and Safety Technician on the Works;

- Prevention and Safety Division for information actions, campaigns and others of a more technical nature.

Safety information shall be displayed on the safety noticeboards and in appropriate sites, such as, for example, the notices of convening for the training actions, the pamphlet of the alcohol campaign, work instructions, safety procedures, emergency contacts, the safety signposting ground plan, accident rates, amongst others deemed necessary and foreseen in the present document.

Sets out the training files, the attendance records and a summary of the actions and the alcohol campaign leaflet will be filled on Appendix 21  in the specific Safety and Health Plan.

The table below identifies the type of training / raising awareness / information action which it is intended to carry out at the Works site.

| Type of training | Categories | Objective | where | Schedule |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Prevision date | At time | Duration Time |
| Full Orientation | All employees | To inform all employees about their duties and rights; Manual Handling; Emergency response; Detection of unknown objects for example Mines; Wild Live HIV | Offices of the Implementing Entity/ Work Fronts | Prior to entry on the works of the employees | Mornings | 2 Hours at list |
| General training | Managers Topographers/ Lab. technicians Planners Surveys Subcont. managers Professional Groups All employees | To raise the awareness of those responsible for the implementation of the works and the employees about the risks associated with the carrying out thereof as well as the measures stipulated to eliminate/ minimise them. | Offices of the Implementing Entity | To be agreed with the DPS of the Implementing Entity | Mornings | 1 hour |
| Specific training | All employees | To inform employees of the risks associated with their activity and what preventive measures have been implemented to eliminate/ minimise them. | Work Fronts | Prior to the start of the activities | Mornings or afternoons (before commencing the works) | 1 hour . |

- ACCINDENT PLAN AND INCCIDENT RATE

The Contractor shall notify verbally or in writing the Engineer and Employer within 24 hours per MCC requirements and also notify the Department of Labor within 48 hours after the occurrence of any accident which has resulted in damage or loss of property, lost work time and disability or loss of human life; or, which has or which could reasonably be foreseen to have a material impact on the environment. The Contractor shall also submit to the Engineer and Employer a follow on written preliminary incident report, outlining a probable cause of the incident and corrective actions taken, within 1 week of the incident per MCC requirement and a final report meeting the requirements of the Mozambique Labor Code for such cases no later than 28 days after the occurrence of such an event.

The mythology implemented in the registration of accidents and in the calculation of the Accident Rates has the following objectives:

- To define responsibilities in the preparation, verification and approval of the "Accidents at Work Reports";

- To define the methodology to be followed in the collection of information for the calculation of the Accident Rates;

- To define assumptions for the calculation of the Accident rates;

- To define the circuit for disseminating the Accident rates;

- To define the mythology to be followed for the analysis of the Accident rates.

Depending on the seriousness of the accident, the following procedures shall be adopted:

■	Incident / near miss - no personal injury:

- Communication from the Works Management to the Prevention and Safety Division and to the Supervisors;

- Occurrence inquiry drawn up the Prevention and Safety Technician;

- Analysis of the inquiry and definition of the preventive measures to be implemented;

■	Minor accident – slight scratches, minor wounds, whose intervention on the works is sufficient:

- Communication from the Works Management to the Prevention and Safety Division and to the Supervisors;

- Survey of the accident undertaken by the Prevention and Safety Technician;

- Final survey report;

- Analysis of the report at a safety meeting at the Works site and the definition of the preventive measures to be implemented.

Serious accident – which entails hospital assistance:

- Communication from the Works Management to the Client, Supervisors and Prevention and Safety Division in writing;

- Notification of the Insurance Company of the accident victim by the Employer;

- Survey of the accident by the Prevention and Safety Division (without prejudice to any others that the Client deems necessary, or as stipulated by law);

- Final survey report;

- Analysis of the report at a safety meeting at the Works site and the definition of the preventive measures to be implemented

Fatal accident

- Communication from the Works Management to the Client, Supervisors and Prevention and Safety Division in writing

- Notification of the local authorities by the Employer;

- Notification of the Insurance Company of the accident victim by the Employer;

- Total isolation of the accident area;

- Survey of the accident by the Prevention and Safety Division (without prejudice to any others that the Client deems necessary, or as stipulated by law);

- Final survey report;

- Analysis of the report at a safety meeting at the Works site and the definition of the preventive measures to be implemented.

Will be attached Appendix 22 to the next Specific Safety and Health Plan and all the incidents shall be report in writing within 24 hours.

- VISITORS’ PLAN

Access to the industrial/social building site is controlled by way of the total delimitation of its perimeter with an indication of the entrance.

A surveillance unit shall be built on the building site which shall be the permanent Guardhouse. The building site shall be fenced off with entrances controlled 24 hours a day. All mandatory safety signposting shall be displayed at the entrance. Signaletics forbidding access by people not involved in the works shall be placed at the building site and work fronts.

When visits are made to the building site, the Client shall be informed previously who they are, the number of visitors, the purpose of their visit, amongst other information deemed relevant for the approval of the respective visit.

Authorisation of the entry of visitors onto the building site shall include the following preventive measures:

- Information about the works characteristics;

- Hazards and risks associated with the works implementation sites;

- Safety standards and rules to be observed;

- Personal Protection Equipment to be use;

- Visitors shall be accompanied by someone who is familiar with the building site. All visitors shall have a protection helmet bearing the inscription "Visitor".

The visitors plan is set out in Annex 23 to the present document

- EMERGENCY PLAN

The Emergency Plan for the Works sets out to inform in advance all employees, in compliance with their duties, visitors, as well as public entities as regards the following aspects:

- Characteristics and localization of the works;

- Existing potential hazards;

- Existing prevention systems;

- Definition of possible emergency situations;

- Definition of scenarios and intervention in emergency situations;

- Definition of principles, standards and general rules in view of the scenarios identified;

- Establishment of coordination with l ;

- Forecast and early organisation of evacuation and intervention by way of drills.

Contractor- will give the emergency contact procedures to all the field work superintended and training for this procedures.

## The first procedures have these items:

- If there is accident the superintended evaluation the status of the victim.

- If the victim is very badly the superintended call emergency on the internal radio of Contractor or cellophane available

- Them he contacts with the safety manager, to front field;

- If  the safety manager arrives on the front field and there’s no medical in the local ;

## The second procedures have these items:

- If there is accident the superintended evaluation the status of the victim.

- If the victim is conscious so superintended  contacts with the safety manager, so he can go to front field;

- Safety manager  calls emergency  to the internal radio from Contractor or cellophane available.

## In case of an emergency never do:

- Touching the victim because the victim may have serious fractures and if we moved  we can deteriorate the situation;

- Don’t give water to the victim because he is lying down  and we can’t not move so the victim  can suffocate;

- Don’t give water because the victim may have internal bleeding and the result is the victim temperature  lower and enter into thermal shock

## In case of an emergency must do:

- talk to the victim if conscious until the paramedics arrived;

- Ask what day of the week  to analyze the type of consciousness

- if the rain cover the victim with a coat;

Will be attached Appendix 24 to the Specific Emergency Safety and Health Plan.

- MONTHLY REPORT

On the 5th working day of each month Contractor  will submit to the Supervisors the Monthly Safety Report which will contain the following points:

- Monthly Summary of Accidents at Work;

- Recording of Accidents;

- Indication of Number of Hours Worked;

- Recording of production Human Resources.

Will be attached Appendix 24 to the Specific Safety and Health Plan.

- ALCOHOL  CONTROL

As a way of controlling accident rates, we believe it is vital – amongst other procedures – that Contractor has effective control of the degree of Blood Alcohol Level in the blood of the employees on the works. To this end, Contractor shall implement the alcohol control and the Mozambican IGT shall approved and the workers syndicate of Contractor. Two entities approved the Contractor shall give training to all workers and explaining the internal alcohol procedure control. The Contractor shall started doing this alcohol test complying with the regulation approval

The  Specific Alcohol Control shalll be attached Appendix 26.

aPPENDIXS

aPPENDIX 1

Distribution of the Development of the Health and Safety Plan

aPPENDIX 2

Organization Chart

aPPENDIX 3

Health and Safety Policy

aPPENDIX 4

Working Hours

aPPENDIX 5

Civil Liability Insurance

aPPENDIX 6

Work Schedule

appendix 7

Men Power

appendix 8

List of Equipment

appendix 9

Access Control on the facilities

Facilities project

appendix 10

Equipment Check List

appendix 11

Hazards/Risk Analyses per Activities/Equipment  and Control Factors Assessments

appendix 12

MSDS

aPPENDIX 13

Monitoring Measures

appendix 14

Medical Support

appendix 15

PEE Plan

appendix 16

Specific Lifting Procedure Cranes and Lifting Operation

aPPENDIX 17

Specific  Procedure for Scaffolds

aPPENDIX 18

Specific Procedure for Demolitions

appendix19

_________________________________

Excavations

appendix20

________________________________

Traffic Control

appendix21

_________________________________

Training

appendix22

_________________________________

Acident and incident Rate

appendix 23

________________________________

Visitors Plan

appendix24

Emergency Plan

# Emergency Scenarios

The Construction Site's Emergency Plan aims to respond to a set of situations that demand different and specific ways of acting. Based on the risk assessment done in this Work Unit (WU), the emergency scenarios identified are:

- fire and/or explosion

- NATURAL DISASTERS

- WORK ACCIDENTS

- INCIDENTS

In order to objectively identify the necessary type of intervention, 3 levels of intervention were defined, characterized by:

Emergency can be resolved using existing means – Emergency can be resolved by any Collaborator / Worker, immediately after its detection, with the means at his disposal.

The colour code for this kind of emergency is yellow.

Emergency can be resolved with 1st intervention - Immediate intervention as soon as the emergency is detected, in its early stages. Intervention is performed by the First Intervention & Evacuation Team (FIET), which ensures rescue, evacuation and first-aid operations for victims, in accordance with the instructions received from the Prevention & Safety Technician.

The colour code for this kind of emergency is orange.

Emergency can be resolved with 2nd intervention – In case the 1st intervention is not enough, more powerful means should be used, by contacting external entities, such as the Fire Brigade or the Emergency Medical Services.  These entities work together with the FIET, respecting the chain of command.

The colour code for this kind of emergency is red.

sudden ILLNESS OR accident

YELLOW

ORANGE

RED

In case of minor accidents, such as small cuts to the finger, any collaborator / worker should inform the Foreman or Prevention & Safety Technician immediately, so they can direct the victim to adequate assistance.

If the case is visibly severe:

The Prevention & Safety Technician shall call the medical emergency, or the local Fire Brigade, or  another entity indicated by the Developer, with the proper competence to act (given the specificity of the construction's location).

The phone call will be answered by an Emergency Centre operator or the existence medical means  on the constrution (in the case of the national emergency service), who will send the appropriate assistance. Give all the information requested, in order to allow quick and effective assistance for the victims.

Above all, remain calm and report the situation in a simple and clear manner:

Construction Site Identification

E.g.: “I’m calling from the XPTO

Type of situation (illness, accident, etc.)

E.g.: “Worker has fallen from a height”, and explain the victim's position.

Number, gender and estimated age of the persons in need of assistance;

The situation's apparent degree of severity;

E.g.: “The worker is unresponsive, with eyes shut, and does not react”

The main complaints and the changes observed;

The existence of any other situation requiring that other forms of assistance are called to the site, such as gas leaks, danger of fire, electrocution, etc.;

Telephone number from where the call is being made;

After the situation has been categorised. The national emergency service centre's operators indicate or the responsabel persoon of the medical means that existe on the constrution  the best procedure, sending - if necessary - the appropriate means of assistance. If it is not necessary to send an ambulance, all the information about the best way to transport the injured person(s) to the appropriate health centre is given.

Only end the call after obtaining the operator's approval;

After carrying out the aforementioned operations, immediately inform the following, in this order:

- Executing Entity's Representative

- HSMS / PHD Manager

- Coordinator of Safety at the Construction Site

Fires

A fire, depending on its dimension, fuel /combustion agent involved, may be considered minor or major. Thus, in the case of the former, one may consider a yellow or orange level intervention to be necessary, with internal emergency and fire fighting methods. If it is considered major, the intervention level shall clearly be red, and it is essential that the appropriate external resources intervene for an effective response to the type of occurrence.

Basic rules that must be respected :

It is forbidden to start fires in the intervention zone of this contract job;

It is expressly forbidden to burn waste in the intervention zone of this contract job;

Handle matches and cigarettes with special care: respect the existing signalling of hazardous sites;

Take special care when using devices which produce sparks. Respect the safety instructions and always have fire-fighting equipment to hand.

Rapid initial action is essential to prevent a fire from spreading and reaching uncontrollable proportions. Every effort should be made to extinguish or control any fire which occurs and, at the same time, the fire brigade must be alerted (red level situation). The alert should be raised using the fire brigade national number. The caller should be as clear and precise as possible, regarding the information to give.

YELLOW

ORANGE

Whoever detects a fire must try to safeguard the area where it begins so that, later, its causes can be investigated. At workplaces, there shall be 6 kg ABC chemical powder extinguishers, as well as in all existing mobile equipment. A fire hose reel will be placed next to social facilities.

In the event that a fire starts, the following actions must be taken:

Take out the extinguisher (they are positioned in pre-defined places, properly signalled);

Withdraw the lock pin;

Put your back to the wind (the jet should go in the direction of the wind) and point the jet at the base of the flames, advancing as the flames go out;

After using an extinguisher one should inform the persons in charge, so they may provide a replacement (Attention: even partial use of an extinguisher renders it inoperative).

In case it is necessary to prepare and use the fire hose reel, located near the social facilities:

Open the FS (Fire System). fire hose reel closet

Fully unroll the fire hose (avoid bends in the fire hose)

Open the Fire Hydrant

RED

Use the nozzle tip in an effective manner (requires proper training)

If you unexpectedly come upon a fire, call the Fire Brigade immediately, using the national fire brigade number.

The caller should transmit the following information, in a very precise and concise manner:

Approximate location of the fire;

Present estimated size of the fire (if possible);

Fastest means of access to the site

Intoxication

Phone for the medical support that have on the construtions for this particular situation . For each situation, the measures to be adopted will be advised. Explain the occurrence in detail, always trying to give information that might help the Anti-Poison Information Centre to identify the situation, namely:

- Who – age, gender, pregnancy, etc.

- What – product, animal, plant, mushroom

- How much – product quantity, exposure period

- When – how long ago

- Where – at home, in the field, in the factory, etc.

- How – fasting, with food, with alcoholic beverages, etc.

In the event of ingestion of chemical products, consult the MSDS or CSDS and identify the ingested chemical product's composition.

In the event of poisoning due to ingestion of a chemical product, you should not touch the victim's mucous membranes, but put the victim in the lateral safety position (LSP), periodically monitoring ventilation and respiratory signs.

Note: Do not force the victim to vomit: it could be extremely dangerous because it can direct the poison to the lungs, causing additional injuries.

Whenever possible, send a package of the chemical product or a sample of the ingested product to the hospital, so the medical staff can identify, immediately, the proper antidote. Your collaboration is fundamental: pay attention to the questions asked. Follow the given instructions.

Some advice to prevent accidental intoxication:

Don't take or give medications and do not exceed the prescribed dosage.

Familiarise yourself with the meaning of the symbols on the labels.

Carefully read the instructions for use and employ the products according to the safety rules, especially when using corrosive products, mineral oils, etc.

Remaining calm is very important, do not hurry but do not waste time.

Human and Material Resources

The formation of a First Intervention & Evacuation Team (FIET) led by the Prevention & Safety Technician is foreseen. This includes a first-aider, as well as other members to be named.

Extinguishers shall be used as an auxiliary method for the fire fighting first intervention. The construction site also has a fire hose reel, properly marked on the emergency plan. The operability control of the above mentioned resources will be recorded.

APPENDIX 25

Monthly Report

appendix 26

ALCOHOL CONTROL