| Activity/Work Task: Signals/ Lighting | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Signals/ Lighting | - Vehicular Accident | - All signs and signals must have warning signs for traffic that is approaching - Speed limit signs must be posted - Banks man / flag man must be available where required - Hard barriers must be provided to prevent outside vehicles from entering the work area - Daily monitoring must be done to see if improvement can be done - Only designated crossings and foot paths must be used by pedestrians - All employees must be trained in emergency plans - Traffic control plan must be established | S | M | L |
|  | - Electrocution | - Only qualified and certified electricians must be allowed to work on electrical equipment - Lock out tag out procedure must be developed and followed - Adequate supervision must be available on site - Employees must be trained in the emergency procedure | S | M | L |
|  | - Falling of material | - All material must be secured properly - Work area must be barricaded - Caution signs must be provided - All employees must wear hard hats | S | M | L |
|  | - Crane operation | - Crane shall be inspected and have a valid third party certificate - Only a certified operator will be allowed to operate the crane - Rigger must be third party certified - Only rigger will be allowed to signal the crane operator - Lifting area must be barricaded - No person will be allowed to enter under the suspended load | S | M | L |
|  | - Striking of utilities | - Obtain permits prior to start of work - Identify all existing utilities using a Cable Avoidance Tool - Manually excavate the area - Use lock out tag out procedure - Adequate supervision - Daily inspection | S | M | L |
|  | - Injury / Death | - Emergency preparedness plan must be in place - First aid station and emergency numbers must be posted on site - Adequate supervision by a competent person | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Hard hat, safety shoes, safety glasses, high visibility vest | - AHA training - Safety Orientation | - Daily - Weekly |  |  |  |