| Activity/Work Task: Stressing CWR and Stress Monitoring – Not CWR | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Stressing CWR and Stress Monitoring – Not CWR | - Manual Handling | - Sufficient staff shall be used for lifting - One person shall co-ordinate lift | S | M | L |
|  | - USE Of Jacks | - Equipment shall be inspected prior to use for good condition - Operate jack by adopting the correct stance to the jacks position - Jack shall have a firm base - Check jack has been maintained and is within service date - Body parts / clothing and team members shall be clear when lowering - Do not stand on jacked rail | S | M | L |
|  | - Welding | - Flame retardant PPE shall be worn - Welding masks shall be worn | S | M | L |
|  | - Dust, Silicate and Welding Material | - COSHH Assessments and controls shall be available | S | M | L |
|  | - Electrocution | - See AHA Working Adjacent to DC Electrified Rails - See AHA Working Near Electrical Overhead Line Equipment | S | M | L |
|  | - Cutting Rail | - Eye Protection shall be worn - Ear Protection shall be worn - Warning System shall be implemented (i.e. Touch Lookout) - See AHA Use of Rail /Non Rail Disc Cutters | S | M | L |
|  | - Drilling Rail | - Eye protection shall be worn - Hand protection shall be worn for burrs - See AHA Use of Rail/Sleeper Drill for controls and HAV information | S | M | L |
|  | - Use Of Trolley | - See AHA Use of Manual Trolleys/Rail Skate/Scooter | S | M | L |
|  | - Communication Form Failing | - Shall be done in a position of Safety | S | M | L |
|  | - Fire | - Remove flammable debris - Dry Carbon Powder Fire extinguisher shall be available at site | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Safety Glasses - Hand Gloves - Hearing Protection - Fire Retardant Overall - Welding Face Mask | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly - When Required |  |  |  |