| Activity/Work Task: Transportation to Site | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | H |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Inspection of the vehicle. | 1.1 Unsafe walking surfaces during inspection. | 1.1.1 Wear approved footwear. 1.1.2 Watch footing for obstructions and slippery surfaces. | U | M | L |
|  | 1.2 Unsafe inspection location. | 1.2.1 Move vehicle to area free from live traffic or moving equipment. | U | M | L |
|  | 1.3 Unsafe hood lift support and other dangers from touching vehicle. | 1.3.1 Ensure that hood lift support is not defective and properly secured. 1.3.2 Never inspect vehicle while it is running. | U | M | L |
| 2. Driving to site. | 2.1 Defective Vehicle. | 2.1.1 Ensure that vehicle is periodically maintained. 2.1.2 Inspection of the vehicle to be carried out prior to use. | U | M | L |
|  | 2.2 Traffic Collision. | 2.2.1 Competency to drive the vehicle and UAE valid driving license should be ensured. 2.2.2 Drive defensively, courteously, and safely. Obey all traffic regulations including speed limits and travel restrictions. 2.2.3 Avoid taking unnecessary risks on the journey, taking care for own safety and health and that of other road users and others who might be affected by the actions. 2.2.4 Always use your seat belt, ensure passengers use their seat belts including Driver (if being driven). 2.2.5 Take required training on Defensive Driving | S | Cr | M |
|  | 2.3 Lost on route / unfamiliar with road layout. | 2.3.1 Ensure that site road layout has been oriented or familiarize prior to commencing journey. 2.3.2 Route map should be carried for reference. 2.3.3 Emergency card with mobile contact details should be carried to ensure further assistance regarding road direction. | O | N | L |
|  | 2.4 Hazardous road condition. | 2.4.1 Avoid driving on poor road condition. Use alternate road, if possible. 2.4.2 Adjust driving and driving speed according to road condition. | O | Cr | H |
|  | 2.5 Adverse weather condition. | 2.5.1 Avoid driving during adverse weather conditions. Check weather conditions prior to travel. 2.5.2 Adjust driving according to adverse weather condition. Never drive beyond the limits of visibility. 2.5.3 Slow down and be alert. 2.5.4 Avoid off-road driving during any adverse weather condition (e.g. dust storm, rain, fog). | S | M | L |
|  | 2.6 Distraction while driving. | 2.6.1 Avoid using mobile phone and other devices. Do not allow Driver to use cell phone while driving. 2.6.2 Concentrate on driving and avoid frequent adjustment of vehicle accessories. 2.6.3 Avoid eating/drinking while driving. | O | Cr | H |
|  | 2.7 Fatigue. | 2.7.1 Avoid driving if feeling drowsy/sleepy/too tired. Take a break and rest. 2.7.2 Avoid heavy meals before driving. | S |  | L |
| 3. Arrival on site. | 3.1 Parking in unauthorized area. | 3.1.1 Park in designated parking areas only. 3.1.2 Avoid parking the vehicle which can obstruct others parking access/egress. 3.1.3 Use reverse parking where possible. 3.1.4 Safe distance from other vehicles to be maintained. | S | M | L |
| 4. Driving to Remote Location and Leaving Carriageway; 4 Wheel Driving | 4.1 Lost. | 4.1.1 Utilize GPS with associated Map 4.1.2 complete the Movement Control Checklist which includes Emergency Provisions/Supplies 4.1.3 Maintain a satellite phone in one vehicle 4.1.4 Ensure fuel calculation prior to trip and fullest possible fuel tank before departing carriageway. | O | M | L |
|  | 4.2 Stuck or Rollover | 4.2.1 Only operate off of roadway after taking Defensive Driver Training 4.2.2 Only operate off of roadway after taking 4WD Training (required for dunes driving only). 4.2.3 Seatbelts required by all users at all times 4.2.4 Roll bars in all off-road vehicles (required for dunes driving only) 4.2.5 Recovery equipment in all off-road vehicles 4.2.6 Leaving carriageway is not allowed without at the minimum two vehicles (required for dunes driving only) | O | Cr | M |
|  | 4.3 Encountering another vehicle. | 4.3.1 All off-road vehicles and vehicles stopping/working around live traffic to have yellow revolvers 4.3.2 Night driving not allowed 4.3.3 Avoid stopping at bottom of dunes out of visible area 4.3.4 Minimize speeds to ensure adequate reaction ~ stopping distance. | L | M | L |
|  | 4.4 Encountering vector (see Vector AHA) | See Vector AHA | O | M | L |
|  | 4.5 Unexpected Adverse Weather | 4.5.1 Discontinue movement until weather passes. 4.5.2 Park vehicle in sheltered, safe location but out of potential track of another vehicle. 4.5.3 Remain in vehicle if possible. Ensure tailpipe remains free of sand while parked. Move in small increments if necessary to clear sand from around vehicle. | L | Cr | H |
|  | 4.6 Vehicle breakdown/Out of Gas | 4.6.1 Cross check to other vehicle if repairs cannot be made. Mark location of inoperable vehicle on GPS MoB. Report when reach area phone is working (dunes driving) | S | M | L |
| 5. Driving to remote location but not off carriageway | 5.1 Lost | 5.1.1 Have detailed map of travel location 5.1.2 Complete Desert Movement Control Checklist and pre-driving checklist. 5.1.3 Proper fuel calculation prior to trip and fullest possible fuel tank. Never pass a gas station without topping off. | O | M | M |
|  | 5.2 Tired/Fatigued | 5.2.1 Driving alone is prohibited. Change drivers if primary driver is fatigued or tired (remote location and dunes driving). | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| 1. Personal Protective Equipment. 2. Vehicle. 3. Vehicle Inspection Log 4. Desert Movement Control Supplies | 1. SH&E Site Induction. 2. Valid driving license. 3. Defensive Driving Training (on-line) 4. 4WD Training | 1. PPE should be inspected prior to use. 2. Daily Inspection of the vehicle. 3. Desert Movement Control Checklist 4. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. 5. Effort should be made to communicate findings to contractor. |  |  |  |