| Activity/Work Task: Trenching and Excavation | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Trenching and Excavation | 1.1 Collapse of Excavation and trench | 1.1.1 Obtain Permit to excavate 1.1.2. inspect the excavation every day and when after any rain, heavy wind or earth quake 1.1.3. keep spoil material away from the edge of the excavation 1.1.4. Maintain safe distance for traffic movement from the edge of the excavation | S | M | L |
|  | 1.2. Damage to existing services | 1.2.1. Obtain Permit to excavate prior to excavating 1.2.2. Check drawings of the area for existing services . 1.2.3. Use a Cable Avoidance Tool and scan the area for any existing services 1.2.4. Obtain NOC from all concerned parties. | S | M | L |
|  | 1.3 Equipment/ People falling into the excavation | 1.3.1. Provide hard barricades around the Excavation 1.3.2. Provide caution signs around the excavation . 1.3.3. Maintain adequate lighting around the excavation 1.3.4. Back fill the excavation prior to end of shift if possible | S | M | L |
|  | 1.4. Falling Material | 1.4.1.Provide hard barricades around the Excavation 1.4.2. Provide caution signs around the excavation . 1.4.3. Maintain adequate lighting around the excavation 1.4.4. Back fill the excavation prior to end of shift if possible | S | M | L |
|  | 1.5. Injury / Death | 1.5.1. Only employees who are trained in excavation safety may enter in excavations 1.5.2. Only enter an excavation if there is no other alternative | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Hard Hat, Safety Glasses, Class 2 Reflective Vest & Safety footwear, gloves, eye protection. | - AHA training - Safety Orientation - Emergency Procedure | - Daily - Weekly |  |  |  |