| Activity/Work Task: Use Of Ladders | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared : |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Inspection of ladder. 1. Inspection of ladder. (Cont…) | 1.1 Sharp edges. | 1.1.1 Prevent sharp edges from being exposed. 1.1.2 Use hand protection (Hand Gloves). | S | M | L |
|  | 1.2 Hand fabricated ladder (Job made/ self constructed). | 1.2.1 Hand Fabricated or self constructed ladders are not to be used. Unless constructed to set standards and certified by a 3rd party. | S | M | L |
|  | 1.3 Oversized ladder. (Distance between each rung). | 1.3.1 Ladder rung should not exceed distance between each rung does of 12 inches/ 1 foot. | U | M | L |
|  | 1.4 Bends in ladder. | 1.4.1 Make sure that the ladder is not bent or broken. 1.4.2 Make sure that the ladder is colour coded. | S | M | L |
|  | 1.5 Wrong positioning of ladder. 1.5 Wrong positioning of ladder. (Cont…) | 1.5.1 Make sure that the ladder is placed on firm, solid & level ground. 1.5.2 Install ladder so that it is placed at 4:1 ratio. ( 75°) 1.5.3 Make sure that the ladder is not placed on any object or equipment to gain extra height/ reach. 1.5.4 install ladder not against glass or fragile surfaces. 1.5.5. Make sure that the ladder is not placed in the front of any entry/ exits. If required to be placed at any entry/ exit, ensure that the entry/ exit is properly locked. | S S | M M | L L |
|  | 1.6 Ladder placed near energized electrical lines. | 1.6.1 Maintain adequate distance when placed near energized electrical lines. If required to be placed near electrical lines make sure that all lines are de-energized and ladder should be made of non-conductive material (LOTO – Lock Out Tag Out system to be followed). | S | M | L |
| 2. Climbing up & Down the ladder. 2. Climbing up & Down the ladder. (Cont…). | 2.1 Unstable Ladder. | 2.1.1 Install ladder so that it is placed on firm, solid & level ground. 2.1.2 Install ladder so that the ladder is secured at both ends (Top & Bottom). | S | M | L |
|  | 2.2 Oily / Greasy/ Debary on ladder. | 2.2.1 Make sure the ground that the ladder is placed on is not a slippery surface. 2.2.2 make sure that foot wear of the user is cleaned from any oil, grease & slippery substances. 2.2.3 maintain good housekeeping so that the area around the ladder is free from debris. 2.2.4 Hold onto the rungs, not the sides of the ladder in case you do slip. | S | M | L |
|  | 2.3 Excess weight/ Over load on ladder. | 2.3.1 Only one person at a time is allowed to climb the ladder. 2.3.2 Install ladder so that it is placed at 4:1 ratio. ( 75°) | S | M | L |
|  | 2.4 Carrying of material while climbing the ladder. | 2.4.1 Maintain three points of contact at all times. 2.4.2 Material is not to be carried by hand while climbing the ladder. 2.4.3 Make sure that all material to be taken up to the landing platform is raised with the help of a tag line or a tool belt. | S | M | L |
|  | 2.5 Sharp edges. | 2.5.1 make sure that there are no sharp edges. 2.5.2 Use hand protection (Hand Gloves). | S | M | L |
| 3. Inspecting from a ladder. | 3.1 Over reaching from the ladder. | 3.1.1 Ensure that the ladder is secured. 3.1.2. Ensure that there is at least 1 meter of ladder above the rung that the feet are on. (Do not step on the last three rungs at the top of the ladder). 3.1.3 Ensure that you do not over reach from the ladder. 3.1.4 If necessary ladder must be relocated to gain proper reach. 3.1.5 Ensure there is a three point contact always while on a ladder. | S | M | L |
| 4. Stepping off and onto a ladder from a work platform | 4.1 Falls from height. | 4.1.1 Ensure you grip a rung with both hands when stepping off and onto a ladder. 4.1.2 Ensure that the ladder projects 1 meter above the step off point onto the work platform. 4.1.3 Climb to the rung above the platform before stepping down onto it; Don’t step up onto the platform. |  |  |  |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Personal Protective Equipment (PPE) | Site Safety Induction. | PPE should be inspected prior to use. Monthly inspection of ladder All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. |  |  |  |