| Activity/Work Task: Use Of Mobile Elevated Working Platform Boom Lift | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Working On Boom Lift | - Struck By Train | - Safe working zone shall be implemented and maintained when using powered access equipment which can come within 2 meters of the running line | S | M | L |
|  | - Trapping Of People In Working Platform | - Dangerous parts shall be adequately guarded in relation to the Provision and Use of Work Equipment regulations hierarchy - Establish and maintain an exclusion zone around any working platform mechanisms | S | M | L |
|  | - Overturning / Personal Injury | - Platform safe working load shall not be exceeded | S | M | L |
|  | - Falls Of Person, Tools, Material and Equipment | - Platform safe working load shall not be exceeded - Powered access equipment shall have guard rails and toe-boards - Staff shall be clipped to the Work Restraint points at all times when working in the confines of the basket. - If you step from the floor of the basket or handrail protection is not in place a safety harness (Fall Arrest) shall be clipped to the Fall Arrest Anchor point. - The number of staff clipped to either type anchor point shall not exceed the number it is rated for. - An exclusion zone shall be established and maintained around equipment when in operation | S | M | L |
|  | - Trapping Between Platform And Fixed Object | - An exclusion zone shall be established and maintained around equipment when in operation near to platforms or other places of limited clearance | S | M | L |
|  | - Collision with Obstructions and people, other machines, etc… | - All controls for operating and driving machine in rail mode shall be visible from the cab - Telescopic arm shall be returned to stowed position where reasonably practicable - All movements shall be made at walking pace within worksites - Safety harnesses shall be worn when the guard rail has been moved - During high winds use hand-held or fixed anemometers prior to using platform – Platform shall not be used in heavy winds - Equipment which could increase wind loading on the basket shall not be used in moderate or stronger winds - No access or work shall take place under or around the platform area - Any access ladder for the platform shall be in place prior to moving the platform - No equipment shall be used to increase the working height or reach of the platform | S | M | L |
|  | - Operation of Platform from ground / platform | - At least one driver or operator shall remain on the ground when the platform is being used |  |  |  |
|  | - Overturning / failure of Support Structure or powered operating mechanism | - Use appropriate powered access equipment for ground terrain to be used on - Check that a safe working load is indicated and observed - Do not use powered access equipment to haul or lift other plant or equipment - Use outriggers and leveling devices where provided as appropriate - Use creep speed when the platform is elevated | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Safety Glasses - Hand Gloves - Full Body Safety Harness | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly - When Required |  |  |  |