| Activity/Work Task: Use Of Pan Pullers Frozen Clip | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Use of Pan pullers / Frozen Clip | - Faulty Equipment | - The hook shall be checked against the wear gauge at periods of 6 months - The tool shall be checked for damage before use | S | M | L |
|  | - Slips | - Hook shall be placed over front arch of clip and the foot of the pan puller shall be on the edge of the housing before applying pressure | S | M | L |
|  | - Back Injury | - Adopt correct stance whilst pulling the clips from the fastenings. Both legs apart on the ballast on the same side of the rail as the clip. Both hands shall be placed on the handle so feet are not liable to slip. - To insert a clip, the handle shall be pulled towards the chest until resistance stops any further movement. - To remove a clip, the pan puller shall be pulled towards the body until clip is removed | S | M | L |
|  | - Damage to Tools and Infrastructure | - Before inserting a clip check that insulators are correctly seated between the rail foot and pandrol housing. | S | M | L |
| - Frozen Clips | - Back Injury | - If excessive force is require to remove a clip the following methods may be utilized. Use of Camber de-clipping machine - Use of Pandrol Rusty Clip Extractor - Use of Fact air Impact tool - Use of Panther De-clipping - If none of the above methods can release the clip then it is permissible to use a hammer as a last resort | S | M | L |
|  | - Struck by Flying Clips | - If hammers are being used to remove clips the following control measures shall be in place - Staff performing the operation with the hammer shall wear full face protection and eye protection to BS EN166 1.B in addition to normal PPE - Controlled blows shall be used to reduce the risk of a clip “flying” from its housing and ricocheting back to strike the user - A foot shall be placed against the pandrol clip to prevent it flying off when released from housing - An exclusion zone of 20m shall be set up | S | M | L |
|  | - Damage to Tools and Infrastructure | - Do not repeatedly hammer seized clips as this may cause the sleeper to shatter or base plate to fracture | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Safety Glasses - Hand Gloves - Hearing Protection | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly - When Required |  |  |  |