| Activity/Work Task: Use Of Rail Mounted Clipping Machines | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Use of Rail Mounted Clipping Machines | - Faulty Equipment | - Planned preventative maintenance program shall be implemented - Conduct Pre use inspection including check against inspection date prior to use - Only Trained & competent operators shall use clipping machines | S | M | L |
|  | - Refueling – Fire / Explosion | - See AHA for Decanting Fuel and Fuelling Small Plant for fuelling small plant - One 2kg Dry Powder fire extinguisher shall be available when fuelling clipping machines | S | M | L |
|  | - Electrocution | - See AHA for Working Adjacent to DC Electrified Rails - See AHA for Working Near Electrical Overhead Line Equipment | S | M | L |
|  | - Manual Handling | - Staff shall receive training in manual handling - The correct number of persons shall be used to carry the components of the machines to site, unless mechanical means is available - Start the engine only when the machine is on its trolley, the trolley on the track and the suitable tools correctly fitted and locked. - See AHA for Manual Handling | S | M | L |
|  | - Slips, Trips and Falls | - Do not exceed walking pace, noting underfoot and rail head conditions. - Do not walk on sleepers or the rail head. - Be aware of the trolley outrigger arm that runs on the opposite rail. - See AHA for Underfoot Conditions | S | M | L |
|  | - Striking By Clip | - Release the control lever as soon as the cycle is ended | S | M | L |
|  | - Eye Injury | - Correct eye protection shall be worn | S | M | L |
|  | - Noise | - Correct ear protection shall be worn | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Safety Glasses - Hand Gloves - Hearing Protection | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly - When Required |  |  |  |