| Activity/Work Task: Use of Suspended Platform | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Arrival on site / Pre-inspection of suspended platform. | 1.1 Falls on level. | 1.1.1 Verify walkway is clear. 1.1.2 Slippery surface should be avoided. 1.1.3 Wear correct footwear. | U | M | L |
|  | 1.2 Falling materials. | 1.2.1 Verify contractor coordinated with building owner / other contractors in the building about the activity and the side of the building where the suspended platform is fixed has been secured. 1.2.2 Wear hardhat. | S | Cr | M |
| 2. Stepping into the platform. | 2.1 Falls on level. | 2.1.1 Verify access gate is clear from any material obstruction. 2.1.2 No oil/grease on shoes or steps. 2.1.3 Verify to access only in designated access point. 2.1.4 Apply 3 points of contact. | U | M | L |
| 3. Inspection on the platform. 3. Inspection on the platform. (cont…) | 3.1 Failure of suspended platform. | 3.1.1 Verify platform is certified and in good condition. Verify daily inspection being carried out by the operator prior to use (see the inspection report). 3.1.2 Major lifting component like ropes/cables, platforms, etc. should be visually checked for defect. 3.1.3 Verify platform is not overloaded. Check SWL. A permanent and prominent sign shall be attached to the platform, stating the safe working load and the maximum operating wind speed. | S | Cr | M |
|  | 3.2 Incompetent operator. | 3.2.1 Verify operator is third party certified and specifically trained for the purpose. | S | M | L |
|  | 3.3 Entanglement / entrapment in moving parts. | 3.3.1 Verify guard / cover is in place for moving mechanical part. 3.3.2 Do not place body part near the movable part of the equipment. 3.3.3 Do not wear loose clothing. | S | Cr | M |
|  | 3.4 Platform struck by sudden opening of the window. | 3.4.1 Verify contractor coordinated and advised building owner/occupants regarding scheduled activities and secured the side of the building properly. | S | M | L |
|  | 3.5 Heat stress. | 3.5.1 Exposure under hot environment should be regulated. 3.5.2 Wear light clothing. 3.5.3 Avoid dehydration. Drink water frequently in small amount. | S | M | L |
|  | 3.6 Falls from height. | 3.6.1 Appropriate safety harness should be used and Verify it is hooked on independent lifeline. 3.6.2 Verify adequate handrails, mid-rails, and toe boards are in place. 3.6.3 Verify gate is locked and closed. | S | Cr | M |
|  | 3.7 During strong wind. | 3.7.1 Work should be stopped and Verify platform on the level ground. 3.7.2 A permanent and prominent sign shall be attached to the platform, stating the safe working load and the maximum operating wind speed. | S | Cr | M |
|  | 3.8 Working near live electrical cables. | 3.8.1 Verify live electrical cable is identified by contractor and properly protected. 3.8.2 Safe working distance from live electrical cable should be maintained. | S | Cr | M |
| 4. Exiting the Suspended Platform. | 4.1 Falls on level. | 4.1.1 3 points of contact at all times when exiting. 4.1.2 Verify access/egress gate is not obstructed. 4.1.3 No oil/grease on shoes or steps. 4.1.4 Verify to egress from designated access point. Do NOT climb over the rails. | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Basic PPE + SafetyHarness. | 1. SH&E Site Induction. | PPE to be inspected prior to use. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |