| Activity/Work Task: Weather Related Exposure | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | ### E |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity |  | Probability |  |  |  |  |  |  |  |  |  |  |
| Date Prepared : |  |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  |  | Unlikely (U) |  |
| Prepared by (Name/Title): | Catastrophic (C) |  | E | E |  | H | H |  |  |  |  | M |  |
|  | Critical (Cr) |  | E | H |  | H | M |  |  |  |  | L |  |
| Reviewed by (Name/Title): | Marginal (M) |  | H | M |  | M | L |  |  |  |  | L |  |
|  | Negligible (N) |  | M | L |  | L | L |  |  |  |  | L |  |
| Employer: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References: | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  |  |  | RAC Chart |  |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  |  |  | E = Extremely High Risk |  |  |  |  |  |
|  |  |  |  |  |  |  |  | H = High Risk |  |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  |  |  | M = Moderate Risk |  |  |  |  |  |
|  |  |  |  |  |  |  |  | L = Low Risk |  |  |  |  |  |
| Job Steps | Hazards | Controls |  |  |  |  |  |  |  | P | S |  | RAC |
| 1. Working in the desert during day time | 1.1 Sunburn | 1.1.1 Protect skin with loose fitting breathable long sleeve shirt and pants; cover head with hat and cover feet with shoes; wear sunscreen on exposed body parts. 1.1.2 Plan ahead and bring shade tarp if you plan to stay in one spot; always look for safe shade areas when stopping and standing 1.1.3 Follow the Heat Injury and Illness Prevention Plan in the PMC Business Office HS&E Plan. |  |  |  |  |  |  |  | O | Cr |  | H |
|  | 1.2 Dehydration | 1.2.1 Drink water regularly, even when not thirsty 1.2.2 Read urine chart and compare 1.2.3 Drink isotonic drinks like Gatorade if heavy sweating (not salt tablets or oral rehydration solution i.e. salt) 1.2.4 Follow work/rest period based on relative humidity chart (*color coding should be provided and posted on jobsite by Contractor). 1.2.5 Follow the Heat Injury and Illness Prevention Plan in the PMC Business Office HS&E Plan. |  |  |  |  |  |  |  | F | Cr |  | E |
|  | 1.3 Exhaustion | 1.3.1 Realize personal limitations; rest frequently 1.3.2 Use Buddy system to carry out tasks 1.3.3 Do not access areas if you have physical limitations such as climbing ladders; entering excavations; entering confined spaces 1.3.4 Follow the Heat Injury and Illness Prevention Plan in the PMC Business Office HS&E Plan. |  |  |  |  |  |  |  | O | M |  | M |
|  | 1.4 Medical Emergency | 1.4.1 Be prepared with personal medications 1.4.2 Know/carry Emergency numbers 1.4.3 Follow the Project Emergency Action Plan 1.4.4 Follow the Heat Injury and Illness Prevention Plan in the PMC Business Office HS&E Plan. |  |  |  |  |  |  |  | L | Cr |  | H |
| 2. Working in the field at night/dusk | 2.1 Visibility | 2.1.1 Plan to stop work with enough time to return during daylight hours 2.1.2 Adjust tasks or vehicle speed if sun is shining in eyes. Wear sunglasses. 2.1.3 Plan prior to the trip and bring torches/flash lights if dusk/night work is anticipated. 2.1.4 If you plan to pray at dusk in remote desert location, stay well off the roadway and leave vehicle headlights on; roof revolver lit, pay close attention to ground for vector/flora. |  |  |  |  |  |  |  | S | M |  | L |
|  | 2.2 Hypothermia. | 2.2.1 Plan prior trip to carry blanket and heavy layered clothing. 2.2.2 If vehicle breaks down stay with vehicle; stay inside with window cracked for ventilation; stay off road in safe area not subject to being struck by another vehicle and out of wadi’s that can flash flood |  |  |  |  |  |  |  | S | Cr |  | M |
| 3. Rain | 3.1 Visibility | 3.1.1 Prior to disembarking do vehicle inspection which includes checking wipers 3.1.2 Bring rain gear if it is rainy season/winter 3.1.3 Do not stop in wadi’s during rain events 3.1.4 If possible pull off roadway if rain is very hard/cannot see so as not to confuse oncoming motorists. |  |  |  |  |  |  |  | O | Cr |  | H |
|  | 3.2 Drowning | 3.2.1 Do not stop in wadi’s during rain event 3.2.2 Stay away from the beach during high rain/winds event. |  |  |  |  |  |  |  | U | N |  | L |
| 4. High Wind – Dust Storm | 4.1 Visibility ~ Disorientation | 4.1.1 If possible pull well off roadway if dust storm is severe so as not to confuse oncoming motorists. 4.1.2 If on foot seek shelter/vehicle 4.1.3 Pre-plan trip and bring eye goggles/ATV goggles if high winds are anticipated. |  |  |  |  |  |  |  | L | Cr |  | H |
|  | 4.2 Eye, ear, nose, throat impact | 4.2.1 Protect eyes with goggles; ears, nose and throat with cloth wrap; dust mask if available. |  |  |  |  |  |  |  | F | M |  | H |
| 5. Earthquake | 5.1 Loss Control e.g. fall down, crash vehicle | 5.1.1 Stop vehicle in safe area; stay in vehicle in the event of earthquake; If walking stop and crouch; If in buildings location proceed to area away from buildings to avoid falling debris; If in office seek cover; follow PSP for additional earthquake control measures. |  |  |  |  |  |  |  | S | Cr |  | M |
|  | 5.2 Objects falling on body or vehicle | 5.2.1 Stay away from buildings if possible if on the street; follow PMC Business Office HS&E Plan for additional earthquake control measures. |  |  |  |  |  |  |  | U | Cr |  | L |
|  | 5.3 Fall into fissure; sand engulfment | 5.3.1 If in vehicle, stay in vehicle; if on foot get to ground and protect head (if falling objects hazard); move away from obvious fissure points; after initial quake move away from fissures in preparation of aftershocks. Communicate with supervisor if possible for accountability. |  |  |  |  |  |  |  | U | M |  | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |  |  |  |  |  |  |  |  |
| - Personal Protective Equipment. - Vehicle - Sunscreen, skin covering clothing - Blanket, layered clothing | 1. HS&E Induction. 2. AHA Training 3. Pre-trip Planning 4. UAE Driver’s License | 1. PPE should be inspected prior to use. 2. Daily Inspection of the vehicle. 3. Desert Movement Control Checklist 4. Familiarity with HIIPP and Emergency Plan 5. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. 6. Effort should be made to communicate findings to contractor. |  |  |  |  |  |  |  |  |  |  |  |