| Activity/Work Task: Working AT Height | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Working On Roofs | - All Tasks | - A survey shall be performed of the roof, including inspection of building plans, to identify roof type and precautions required, such as crawling boards, etc - All separate job shall have a method statement prepared by a competent person, clearly describing the precautions and safe systems of work, which is briefed to all staff and enforced | S | M | L |
|  | - Adverse Weather | - Work shall not be performed on roofs or chimneys during heavy rain, thunderstorms, or during high winds | S | M | L |
|  | - Fall Of Material/ Tools, Etc.. | - Toe boards shall be installed in addition to safety barriers and guard rails, in addition to the provision of debris netting if practicable - Rubbish chutes shall be installed to remove waste safely, if practicable - An exclusion zone shall be established, marked and maintained around the site - Hard hats shall be worn at all times within the area and on the roof | S | M | L |
|  | - Fall Of Person from height. | - For work on flat Protective barrier shall be set back from the roof edge or roof edge protection provided, including installation of edge protection platforms. Work on flat roofs that have designated walkways and or parapets/hand rails do not require the use of safety harness. Staff shall be provided with and wear safety harnesses / fall arrest equipment, which shall be securely anchored - For work on sloping roofs Crawling boards or ladders shall be used or erection of scaffolding or working from mobile elevating work platforms considered Staff shall be provided with and wear safety harnesses / fall arrest equipment, which shall be securely anchored Roof edge protection shall be provided if roof pitch is 30% or more when dry or 10% when wet or slippery Protective barrier shall be set back from the roof edge or roof edge protection provided, including installation of edge protection platforms - Working on steep pitched roofs (greater than 50%). Staff shall be provided with and wear safety harnesses / fall arrest equipment, which shall be securely anchored. Roof edge protection shall be provided. The erection of scaffolding or working from mobile elevating work platforms shall be considered - Use of safety nets / bags, erected by a competent person shall be considered - Unauthorized access shall be restricted, especially to members of the public - Working on chimneys. Staff shall be provided with and wear safety harnesses / fall arrest equipment, which shall be securely anchored. The erection of scaffolding with a working platform, incorporating suitable fall edge protection, guard rails and toe boards shall be provided or working from mobile elevating work platforms shall be considered - Working on fragile roofs (e.g. asbestos, glass, plastic, cement or sheets). An assessment of the fragility of the roof shall be undertaken before accessing the roof – Treat all parts of the roof as fragile until confirmed otherwise. A minimum of two crawling boards or ladders shall be used at a time. Provide a platform, incorporating suitable fall edge protection, guard rails and toe boards shall be provided or working from mobile elevating work platforms shall be considered. Provide protection around valley gutters. Consider use of scaffold boarding out below the roof or erection of safety nets / bags. Protection systems shall be in place when any work is performed within 2 meters of a fragile roof | S | M | L |
| - Portable Ladders | - All Tasks. | - Portable ladders and stepladders shall only be used for short duration work (i.e. no more than 15 to 30 minutes) or light work, such as window cleaning – They shall not be used for carrying materials - A site specific assessment shall be performed to identify whether ladders / stepladders are suitable – Consider use of Mobile Elevated Work Platforms, scaffolding, etc – Ladders shall be of a suitable height - A pre-use check by the user shall be performed to check that it is free from defects e.g. stiles are not damaged, buckled or warped and that the rungs are not cracked, missing – Check that the feet are in good condition. Never use a defective ladder or stepladder - Carrying materials and equipment shall be assessed – CHECK WEIGHTS. | S | M | L |
|  | - Electrocution | - Metal ladders shall not be used in the vicinity of electrical systems, all ladders shall be made from authorized non-conductive materials - Metal reinforcements shall not be used along the stiles - Ladders shall be carried horizontally when near OLE and carried by two people where necessary | S | M | L |
|  | - Falls | - Check that ladders are rested on a firm and level surface and not on unstable surfaces or objects - The top of the ladder shall rest against a solid surface and not on fragile or insecure surfaces - If ladders are used for work on ballast the base of the ladder shall be embedded in the ballast to provide a sound and secure foundation - Ladders shall be footed and / or lashed at all times - The ladder shall be set at 1 meter out for every 4 meters up - Check that the restraint devices on stepladders are fully open and locking devices engaged - Only climb the ladder when it is secure i.e. tied to suitable points, use of a ladder stay or anti-slip device, wedging the bottom of the ladder to prevent sliding or footing the ladder as a last resort - Always have 3 points of contact on the ladder / stepladder at all times i.e. 2 feet and 1 hand – Do not over reach and keep the body within the confines of the rungs – Do not stand on the top three rungs of the ladder (or top 2 rungs of a stepladder unless a suitable handrail is fitted) - Relevant fall arrest equipment shall available and suitable training provided - Emergency rescue procedures shall be available | S | M | L |
| 3. Portable Scaffold/ Mobile Scaffold | 3.1. Contact With Electrical Equipment | - See AHA for Working adjacent to DC electrified lines - See AHA for Working Near Electrical Overhead Line Equipment - Liaise with Electrification & Plant Engineer prior to erection - Perform a site survey during planning process - Isolate all electrical equipment e.g. Over head lines - Isolate all utilities and / or move to a safe position - Carry poles / boards / sections horizontally - Erect scaffold using trained and competent persons - Check scaffold is bonded in vicinity of over head lines - Arrange isolation where possible, or screening of over head lines whilst accessing or working on scaffold | S | M | L |
|  | - Incorrect Assembly | - Erection and dismantling of scaffold shall only be undertaken be trained and competent persons | S | M | L |
|  | - Adverse Weather | - Erection and dismantling of scaffold shall not be permitted during adverse weather condition such as high winds, ice or snow - Manufacturer’s instructions shall be followed in regards to working on scaffold during adverse weather conditions - An inspection of the scaffold by a competent person shall be performed before work is allowed to start following adverse weather conditions | S | M | L |
|  | - Fall Of Tools Equipment and Materials | - During erection and dismantling an exclusion zone shall be implemented and maintained around the scaffold - Toe boards, hand rails, guard boards and boarding shall be installed on the scaffold tower prior to any work starting - Tools, equipment and materials shall be removed from the mobile tower scaffold during all movements - Tools, equipment and materials shall not be placed on the mobile tower scaffold until it is secured and braked - Hard hats shall be worn at all times | S | M | L |
|  | - People Falling From Mobile tower Scaffolding | - Mobile tower scaffold shall be installed by a competent person and inspected by a competent person prior to use - Warning notices shall be used to prevent unauthorized access and whilst scaffold is being erected - The end frame to internally climb the mobile tower scaffold or a purpose built ladder, shall be used to access the scaffold at all times - Guard rails and toe boards shall be installed before use - Harness shall be used at all times when working on mobile tower scaffold | S | M | L |
|  | - Failure Due To Components | - A pre-use check shall be undertaken prior to the start of each shift - Check that the mobile tower scaffold is within date - If the mobile tower scaffold is to be erected for more than 7-days then Form F91 shall be completed by a competent person every week in addition to a 6-month check by a competent person |  |  |  |
|  | - Over Turning of Mobile Scaffolding Tower | - Mobile tower scaffold shall be erected only on firm and level ground – Check ground conditions prior to erection to check it is suitable for the weight of the scaffold, people, tools, equipment and materials - Maximum height for the mobile tower scaffold shall not exceed the height specified in the Abu Dhabi Construction Code Of Practices - Outriggers shall be used and installed prior to use - The correct base to height ratio shall be implemented prior to use - People, tools, equipment and materials shall be removed prior to Moving - Mobile tower scaffold shall only be moved by pushing or pulling from the base - Wheels and castors shall not be less than 125mm in diameter and - shall be fitted with brakes which cannot be accidentally released –these shall be locked into the base of the standards - Establish and maintain an exclusion zone around the scaffold - Hard hats shall be worn at all times | S | M | L |
| - Mobile Elevated Work Platform Boom or Scissor Lift | - Use Of Mobile Platform | - See AHA for – Use of Mobile Elevated Work Platform – Boom | S | M | L |
|  | - Falls Of Person, Tools, Equipment and Material. | - Powered access equipment shall have guard rails and toe-boards - Staff shall be clipped to the Work Restraint points at all times when working in the confines of the basket. - If you step from the floor of the basket or handrail protection is not in place a safety harness (Fall Arrest) shall be clipped to the Fall Arrest Anchor point. - The number of staff clipped to either type anchor point shall not exceed the number it is rated for. - An exclusion zone shall be established and maintained around equipment when in operation | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Hand gloves - Safety Harness - Respiratory protection (As Needed) - Safety Glasses - Ear protection | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures - Working at Height - Working On Mobile Elevated Working Platforms | - Daily - Weekly |  |  |  |